CHAPTER 2

SPEED/TRAFFIC MEASUREMENT PROTOCOL

1 - DEFINITIONS

GENERAL CONCEPTS

Level of traffic: measurement of the traffic flow

Levels of traffic are expressed as vehicles per hour or per day (veh/h or veh/d), or for light vehicles alone: LV/h or LV/d).

The "level of traffic" will often be referred to simply as “traffic”.

SOURCE convention: Unless expressly stipulated otherwise, levels of traffic correspond to a single way of travel and solely for light vehicle traffic. (Caution: truck traffic is not taken into account. This means that it is not converted into private vehicle units –pvu-).

Oncoming traffic: Traffic travelling in the opposite way to the vehicle making the measurements.

Free-flowing traffic: All traffic conditions that enable each vehicle to travel at its own “free” speed, i.e. without being hampered or slowed down by other vehicles. By definition, traffic loses its free flow capacity when it reaches saturation flow conditions in which vehicles begin to hamper one another significantly, eventually developing into gridlock.

Free speed: The natural speed adopted by a given vehicle on a given route (with its physical characteristics, speed limits, etc.), when it is not hampered by any other vehicle (in other words, when it is alone or when the traffic remains free-flowing). In this context, the free speed is an individual concept for each vehicle.

“Net” mean speed: “Net” travel distance covered by the floating vehicle divided by the corresponding “net” travel time.

In this method, solely “net” travel times and travel distances are considered for a given section, i.e. after deducting any extraneous times and distances: namely break periods on the way, “skipped” sub-sections (where measurements have been suspended), suspect sub-sections if they are ultimately invalidated, and in the event of U-turns, invalidated duplicate measurements and return trips.

In fact, measurements on a section are not always valid over its entire “gross” length, but over a length, called the “net” length, that may be shorter for various reasons. The speeds and traffic levels to be taken into account are therefore the corresponding mean “net values”.  When processing the data, these mean speed and traffic values, which are in fact measured solely on the net length of the section, will simply be considered as attributable to the entire gross length. This is why strict limits have been fixed, by means of the “critical threshold”.

This “critical threshold” fixes re-measuring obligations where the net length is too short compared with the “gross” length, and where re-measuring is possible.

The data sheet is designed to give error-proof guidance for calculating gross and net values and for checking this “critical threshold”.

VEHICLES

## Floating vehicle (FV): Any vehicle used by the crew in charge of measurements using the “floating” method.

Light vehicle (LV): This is the only vehicle category to be taken into account for these measurements, both for adjusting the travel of the floating vehicle and for counting oncoming traffic.

The following are classified here as light vehicles (LVs): private vehicles, minibuses and small utility vehicles such as vans, light vans, panel vans and pick-ups.

It makes no difference whether these vehicles are four-wheel drive or not, nor whether they provide public transport service (bush-taxis, taxis, mini-buses) or not. As long as they are light vehicles meeting the above criteria, they are to be included

The following are not classified here as light vehicles:

- trucks, buses and coaches, farm machines, worksite machines, vehicles beyond standards (whether or not under ”dangerous/wide load”), two-wheeled and three-wheeled vehicles, and all non-motorized vehicles (vehicles drawn by animals or humans).

- by way of exception, all vehicles in motorcades or in organized convoys (whatever the type of vehicle, and all priority vehicles or those considered as such, when they are using their revolving lights or sirens. All these vehicles must thus be eliminated.

Followed vehicle (or followed LV): Vehicle followed by the FV, to which the FV adjusts its own speed. The followed vehicle is certain to be a “light vehicle” (LV). Other vehicles are not taken into account and are therefore never followed.

The floating vehicle frequently changes from one followed vehicle to another according to precise rules. Then the followed LV is considered as a "neutral LV", a "hare" or a "tortoise" as applicable.

Neutral vehicle (or neutral LV): The first LV which passes in the right way and which is then followed by the FV that has been waiting. The FV will abandon this neutral vehicle to follow the first LV it encounters travelling the right way, which will be either a hare (if this LV catches them up and overtakes them) or a tortoise (if they catch up with it).

Hare: An LV that catches up with and overtakes both the floating FV and the followed LV, this followed LV being a tortoise or a neutral LV. In this case the FV abandons the tortoise (or the neutral LV) to follow the hare.

If the LV already being followed is a hare and it is overtaken by another faster LV, this other LV will be ignored. Continue to follow the same hare without paying attention to the super-hare.

A hare is bound to come from behind the VF and catch it up. When following a tortoise, watch out for a hare in the rear vision mirror.

## Tortoise: An LV that is caught up by both the floating vehicle FV and the LV, this LV being a hare or a neutral LV. In this case, the FV will abandon the hare (or the neutral LV) to follow the tortoise.

## If the LV already being followed is a tortoise and it catches up with another slower LV, ignore this other LV. Continue to follow the same tortoise without paying any attention to the super-tortoise.

When a tortoise appears, it is bound to be far ahead and the FV is catching it up. When following a hare, watch out for a tortoise through the windscreen.

Consequently: When following a neutral vehicle, it is necessary to watch out for either a tortoise or a hare, which means looking just as much in the rear vision mirror as through the windscreen.

THE MEASURING METHOD

Floating protocol (or procedure): All the rules defining and governing the immediate behaviour of the floating vehicle and its crew during the measuring operation.

Sequence: A measuring run, performed on a section, consisting of an uninterrupted succession of sequences. A new sequence begins whenever a significant event occurs (that usually requires a change in FV response). Each new sequence corresponds to a new line on the data page, on which the relevant information is noted.

There are eight different sequence types, from A to H, which are listed hereafter:

|  | Type A - Pursuit: An ordinary sequence, of limited duration, during which the FV continuously follows the same LV. The term "pursuit" does not mean that the FV tries to catch up with the LV. On the contrary, it just follows it, i.e. it remains at the same distance and tries not to cause any change in the behaviour of the followed vehicle. |
| --- | --- |
|  | Type B - Virtual pursuit: less common sequence, of limited duration, in which the FV that has had to abandon the excessively fast LV it was following (because this LV was constantly exceeding 90 km/h), continues on its way alone and adjusts its speed to a maximum of 90 km/h. |
|  | Type C - Solo: fairly uncommon sequence, of limited duration, in which, the traffic being too light, the FV must continue alone. The driver must then adopt a natural speed, i.e. as close as possible to the free speed that an ordinary light vehicle would adopt at the same place. |
|  | Type D - LV wait: wait sequence of limited duration, without any measurements, in which, the FV, stopped at the roadside, is watching for the first LV to be followed to restart the measurements. |
|  | Type E - Standstill: Transition sequence of varying duration, without any measurements, in which the FV, stopped at the roadside, is waiting until the required measuring conditions are restored. As long as these conditions are not restored, the FV cannot open an “LV wait” sequence. It therefore ignores all passing LVs. |
|  | Type F - Skip: sequence of varying duration, in which the FV continues travelling, but with temporary suspension of all measurements (including traffic counting). This is a decision made necessary in certain cases of fairly local disturbance – it is a decision to “skip” the disturbance. The relevant sub-sections (“skipped” sub-sections) are certain to be neutralized when processing the results of this measuring run. |
|  | Type G - U-turn: sequence of varying duration, without any measurements, in which the FV, in the event of a protocol error or in some cases of disturbance, makes a U-turn and returns to restart the measurements upstream of the place where the problem began. This U-turn is not mandatory in every case. In the event of a U-turn, the corresponding kilometerage (outward and return) will duly be deducted. |
|  | Type H - Break: exceptional sequence of varying duration, without any measurements, in which the crew attends to any occupations that have no direct connection with the measuring operation (refuelling, changing a punctured tyre, etc.). The FV may move during the break but this detour will be deducted and the break will end at the same place it began. |

Suspect sub-section: You decide to open a suspect sub-section when you encounter signs of probable disturbance without being certain at the outset. This allows a limited amount of time during which measurements are continued without changing anything, in order to better assess the situation. The opening of this section is materialized by a mark and an entry on the data page, and by a mandatory sequence change.

Depending on how the disturbance develops in the moments following the opening of this section, there are several possible responses:

- Cancelling the suspect sub-section, purely and simply, if the signs of the disturbance disappear very quickly. In this case the measurements are continued.

- Closing the sub-section when the disturbance, though real, phases out.

- Closing of suspect sub-section when the disturbance is confirmed and persists (or must be considered to do so if the signs last more than 5 mn). In this case, the measurements cannot be continued and, depending on the circumstances, there will be a standstill sequence, a U-turn or a skip sequence.

Unlike the skipped sub-section, the suspect sub-section does not necessarily correspond to a single sequence, because in this case the measurements continue. It may give rise to several pursuit sequences, etc. It is therefore not a sequence in its own right.

The closing, like the opening, of the sub-section, will impose a sequence change.

Several reasons for a suspect sub-section may be compounded. In this case, cuts must be made, concatenating suspect sub-section upon suspect sub-section, but not superposing several suspect sub-sections. For example, you will move on from a first suspect sub-section, due to the onset of rain, to another, due to rain plus to the crossing of a village, then to a third, due only to the rain, etc.

Special sub-section : A special sub-section must only be created exceptionally. It is actually while making measurements on a section that some abnormalities on the road may give rise to the opening and then the closing of a special sub-section, corresponding to their extent.

There are four reasons for creating a special sub-section:

  - roads unexpectedly cut off (see parag. 7.3 further ahead)

  - all diversions (see same chapter, parag. 6.2)

  - all paved/unpaved portions within a section. This is then a mixed section for which it is helpful to know the paved percentage.

  - where it is realised in situ that an error has been made when dividing the network into sections. It is then simply necessary to identify a portion of road in the measurements, which can be subsequently reinstated as a section in its own right.

Refer here to the criteria for determining the divisions into sections (see chapter 1, parag. 2.2).

The opening and closing of a special sub-section require a sequence change.

Unlike suspect sub-sections or skipped sub-sections, special sub-sections are not determined by chance factors on the measuring run (traffic disturbances). They are usually permanent and identical for all the runs1. During a run, suspect or skipped sub-sections could lap special sub-sections.

Neutralization of a sub-section: Elimination of the sub-section for processing of the results of a measuring run. This concerns all the skipped sub-sections, on the one hand (for which neutralization is self-evident because it has not been possible to make the measurements), and some of the suspect sub-sections, on the other hand (those for which it has finally been considered that measurements were performed in really disturbed conditions and must therefore be invalidated).

For each suspect sub-section, a decision must therefore be made. The operator proposes a decision on the actual day of the measurements at the data pre-processing stage. The coordinator will review this decision at the data processing stage.

On such neutralized sub-sections, two cases may arise: either repeat measurements are likely to give processable results or it is definitively impossible to make reliable measurements. This naturally depends on the type of disturbance encountered, which is either temporary and avoidable or permanent and unavoidable.

Convoy: A group of at least two vehicles travelling in a bunch (vehicles close together travelling in a file at the same speed).

You must differentiate between:

- “Organized” convoys (when this behaviour is intentional, whether it is a military or other type of convoy). These are often convoys of trucks, which in themselves do not interest us for the measurements. But organized convoys are a disturbance to the LV traffic, which does interest us.

- Unintentional convoys, which form naturally because of the traffic itself and the road characteristics and which are also often caused by trucks (overtaking difficulties, sharp deceleration of trucks on an upward grade or in areas with many sharp bends, etc.).

2 -  GENERAL SPEED/TRAFFIC MEASURING PRINCIPLE

The technique used, termed "floating technique", consists in a measuring vehicle integrated in the traffic stream (floating vehicle) following other vehicles according to a special protocol that enables it to adjust its speed to the mean speed of the traffic stream.

This general “floating” measuring technique is applied in the same way to unpaved and paved roads, even multi-lane roads, and whatever their traffic level (except in special cases).

This practical, ultralight method yields high-quality results providing it is rigorously implemented.

2.1 – Behaviour of the floating vehicle

The "floating" vehicle must follow a light vehicle and constantly adjust its speed to that of the followed vehicle.

It will frequently change followed vehicles, choosing another vehicle during an overtaking manoeuvre between the followed vehicle and another light vehicle travelling the same way (either because the followed vehicle is overtaking another slower vehicle or because the followed vehicle and the FV are being overtaken by another faster vehicle).

But not all overtaking gives rise to a change in the followed LV. A specific rule stipulates the case in which a change occurs (see further ahead in this chapter, parag. 4.5, alternation of "hares" and "tortoises"). Another exception is that the same vehicle must not be followed for too long at a time (see further ahead in this chapter, parag. 4.8, the "15 mn rule").

From one followed LV to another, the FV "floats" in the traffic in the above manner as far as the end of the road section It thus lets itself be carried along by the traffic stream, continually adjusting its speed to the normal traffic conditions.

Only LVs are taken into consideration and followed in this method. No attention is paid to trucks or other types of road users.

Note: In the end, the mean of the mean speeds of each of the LVs travelling on the section will be equated to the resulting speed of the floating vehicle (subject to various corrections, particularly concerning sub-sections where measurements have been disturbed).

2.2 – The operator's work

For measuring the speed:

During the measuring operation, the operator records a series of indications on the elapsed time, distances covered, followed LVs and various other items of information sequence by sequence on the data sheets.

For measuring the traffic:

During speed measurements on a section, the operation consists simply in counting the oncoming LVs. This counting is also recorded on the data page.

Caution: There is no counting of oncoming traffic without measuring the speed and no measuring of the speed without counting the oncoming traffic. Traffic counts and speed measurements always go together.

2.3 – General conditions of speed/traffic measurements

The aim of the measurements is to determine the mean free speeds of LVs and levels of traffic not subject to any particular constraints. The basic principle is to make these measurements under normal traffic conditions, providing it actually remains free-flowing.

- The following requirements must be met:

- Measurements must be made on business days and in business hours, only during the daytime, from one hour after sunrise to one hour before sunset.

- Measurements must be interrupted in the event of rain or heavy showers, even if these are brief, and will begin again as soon as traffic conditions have reverted to normal.

But all chance occurrences that disturb the traffic during the measuring operation will be taken into consideration and treated in accordance with the strict rules detailed hereafter.

Providing these requirements are met, speed/traffic measurements can be made, always continuously, over an entire section.

3 – CREW AND ROAD USERS

The crew of the FV must never take any abnormal risk that would put them or third parties in danger (other users, pedestrians, etc.).

In particular, they must obey the Highway Code in use in the country and not let themselves be led to drive dangerously by any followed vehicle or by specific circumstances.

3.1 – Parking

When the FV is parked at the roadside, ready to pull out (in case of LV wait), it must never be in a dangerous position for itself or for others. Avoid exits from curves, concealed areas (behind a high point), narrow passages, etc.

3 2 – Driver's rest periods

The driver's rest periods must be appropriately scheduled. The special work required of him necessitates much closer attention than ordinary driving and is much more tiring as soon as the traffic becomes heavy.

Caution: Breaks, including technical breaks (to refuel) must not be taken during measurements on a section.

3.3 – Reactions of followed motorists

Not only must the FV never endanger other drivers, neither must it influence their speeds or routes. If the followed vehicle driver is intrigued, worried or irritated and therefore slows down or stops to let the FV overtake it, this defeats the purpose.

That is why it is essential to follow at a distance. The “pursuit” now proceeds while maintaining a certain safety distance and discretely enough so as not to worry the driver of the followed LV nor above all, to influence his behaviour. You must therefore remain quite a long way behind the followed vehicle, but close enough not to lose it. If the followed LV stops or turns off at a branch road (see further ahead), you must be able to see this.

A reasonable distance (at least 50 metres, but this will naturally depend on the road alignment) is also necessary so as not to hamper new LVs or prevent them from overtaking the FV and the followed LV successively, if the occasion arises.

Moreover, the 15 mn rule reduces the risk of following vehicles for too long and making them feel challenged or threatened (see in this chapter, parag. 4.8).

The FV is there to make its measurements discretely. It has the status of an observer and must not induce any traffic disturbance itself.

It is not permissible to attract attention when travelling or when parked, by a revolving light or any other means used by the police, that would change road users' behaviour.

3.4 – Refuelling

Refuelling must be scheduled beforehand, at refuelling points that correspond insofar as possible to section limits, so as not to interfere with the measurements. This is not generally too much of a constraint as all large built-up areas correspond to section limits.

If this is impossible, or if supply contingencies of petrol stations make it necessary to refuel during a measuring operation, the measurements will be interrupted by a "break", with or without a detour, as specified further ahead.

4 – BASIC PROTOCOL

A SOURCE tool: the input form set Characteristics This is a single Excel workbook, format 97 (around 110 KB in size) contained in the SOURCE CD-ROM that can be copied onto your hard disk or a diskette. It is named "SOURCEForms.xls" It contains side-by-side the following 3 models: Section data sheet Section data page Calibration sheet These models are not intended for computerized data entry. Each model comes as an Excel sheet designed to be printed out and then photocopied for the SOURCE measurement crews, who will fill the forms out by hand. Holding this computerized file is consequently not essential to the use of the SOURCE method. Simple hard copies of the different models are quite sufficient.

4.1 – Using the forms

Each measuring run on a section will immediately be recorded in the data pack, consisting of:

- A single "section data sheet", initially a loose sheet. To begin with, this is used to record a series of general information on the section and on the measuring conditions (Some items may be filled out in advance, or at all events just before starting out. Others must wait until measurements on the section have been completed). At the end of the measurements, it is also used to recap input data and calculate results for the section.

- As many "data pages" as necessary, depending on the section length and progress of work.

When measuring is finished on a section, after preprocessing the data later on when more time is available (and consequently after completing the data sheet), the pack can be stapled together, with the data sheet on top followed by the data pages in chronological order.

Do not staple the packs in advance as it is not practical to recap the data in a data sheet already assembled with the data pages, the number of which you never know in advance.

The data sheets and pages must be filled out legibly as the originals themselves are kept and handed over to the client after use. They must never be filled out in pencil. Should it prove necessary to transfer or copy up data because of poor quality originals, these originals must still be enclosed with the copies and also given to the client.

4.2 – Identifying section ends

The section ends must be clearly identified and described in writing on the data sheet when making the first measurements. These measurements can then be renewed on strictly identical sections, not only for the following runs, but also for campaign after campaign, even if they are made by a different employee or consultant

When a section has already been measured (during a previous run or a previous campaign), it will just be necessary to relocate previously used landmarks, the landmark at the finish being identified while travelling. It is thus essential to record in advance on the data sheet, the descriptions of these two landmarks taken from the previous documentation.

When a section has not been measured, the landmarks must be picked out for the purpose: either in advance, from maps, or on the road, by the operator. It is easier to choose the landmark at the start than the landmark at the finish because it can be done at leisure, whereas the landmark at the finish is chosen while travelling. If this choice proves unsatisfactory, you still have the possibility of making a U-turn to re-measure an end sub-section, this time stopping at a more appropriate landmark.

Both at the start and the finish, record the landmark identification accurately and concisely in the data sheet, when your vehicle is stationary.

Always choose a simple landmark that will not change, the description of which will not be confusing (give priority to bridges, junctions or conspicuous permanent buildings). Even three years later, another operator must be able to position himself accurately enough at this landmark on the road merely by using the information on the data sheet, or still more difficult, recognize it while travelling.

The following are not to be used:

- road signs indicating the entrance to or exit from built-up areas as their location may change over time,

- kilometric markers, which may disappear or be relocated during major roadworks with or without changes to the alignment,

- any landmarks that may have gone or that may not be definitely identifiable three years hence (trees or groves; light facilities such cabins, barriers, etc.; huts; first house in a built-up area, etc.).

Measuring will begin and end exactly at the landmarks, which must therefore be chosen in areas where measuring is possible without disturbances, i.e. in principle, outside built-up areas at the start or end of a section.

Where there is no reliable, well-placed landmark at the limit of the built-up area,  it is possible to choose a reliable landmark inside the built-up area (public building, monument, etc.) but with certain consequences to be detailed later (possibility of a suspect sub-section or a “skip” at starts and ends of sections).

Note: When the limit between two sections corresponds to a built-up area, the upstream end-of-section landmarks and downstream start-of-section landmarks will consequently rarely be the same. However, these section limits must be given a single name (to prevent any errors in the series of sections). Note that in this case, it is accepted that the final length of the link omits the distance between the two landmarks.. .

4.3 – Wait at the start of a section

The floating vehicle positions itself at the start of the relevant section. It is parked at the roadside, if possible exactly level with the start-of-section landmark, which has been identified from its description or which has just been newly determined (in which case, take time to choose this start-of-section landmark carefully).

Set the timer to zero, and likewise the daily totalizer of the recorder (i.e. of the two kilometric recorders on the dashboard, the one that can be reset to zero) or the topometer, where the FV has one. This information is recorded on the first line of the data page.

As soon as the crew is ready, it begins an “LV wait” sequence, ready to start off, with its engine running (except in very low-trafficked areas).

When the first LV passes in the right way, the operator triggers the timer just as the vehicle passes the FV. The FV starts off and follows this LV. The first "pursuit" sequence begins.

###### Data sheet A

No of link

No of  section

First run on the section.

Time when the first

« LV wait » sequence begins (watch).

Data page A’

Only one box to be ticked per line

The neutral LV  is neither a hare nor a tortoise. Identify the neutral LV this way.

Total number of pages for section

(to be filled out at end of section)

Begin counting of oncoming vehicles

Identify the followed vehicle as you please

(if it’s not set by the client):

make, vehicle registration, etc.

4.4 – Pursuit of the first LV

This first vehicle followed after a waiting period is always considered as a "neutral" LV.

The FV follows this neutral vehicle at a distance as far as the first LV (and only an LV) that they both encounter travelling the same way as themselves. At this time, they are confronted with one of two situations:

- The followed LV is overtaking another slower LV. In this case, the FV abandons the followed LV (does not overtake it but lets it go) and follows this new, slower LV which will be called a "tortoise",

- The FV and the first followed LV are overtaken by another LV. The FV then also overtakes the first followed LV in order to follow this new, faster LV, which will be called a "hare".

4.5 – Hare-tortoise alternation rule

From now onwards, the hare-tortoise alternation rule will be applied: The FV must follow in turn a tortoise (an LV being overtaken by the previously followed LV) then a hare (an LV overtaking the previously followed LV), but never two hares or two tortoises in succession.

A vehicle is called a "hare" or a "tortoise" just for the time of the pursuit depending solely on its overtaking behaviour that has triggered this pursuit (it has been overtaking, it is therefore a hare; it is being overtaken by the other vehicle, it is therefore a tortoise). The same vehicle, may well be followed for a while as a "hare", and then subsequently encountered and followed as a "tortoise".

When the FV is following a tortoise:

The FV is awaiting a hare and will also overtake all the still slower LVs that the tortoise can overtake ("super-tortoises"), without paying attention to them. However, it will follow the first faster LV that overtakes both of them (both the FV and the tortoise). This faster LV will be a hare and the alternation requirement will be met.

A hare is only taken into account insofar as it is effectively in the process of overtaking the LV that was being followed (tortoise or neutral vehicle). If the hare remains behind the LV for some time (or behind the FV) before being able to overtake, or if it "settles" behind the LV (or behind the FV) for a long time, the FV is always required to follow the initial vehicle (hare or tortoise). The new recordings are accordingly not made until the LV followed by the hare is being overtaken, but not before).

When the FV is following a hare:

The FV is looking for a tortoise and lets all the LVs go that are overtaking the FV and its hare. It pays no attention to "super-hares". But it will let its hare go in order to follow the first LV that is overtaken by the hare. It will thus be following a tortoise and the alternation requirement will be met.

A tortoise is taken into account as soon as it is caught up by the previously followed LV. If the LV remains behind the tortoise for some time before overtaking it, or if it "settles" behind the tortoise for a long time, the FV is always required to follow the new tortoise and the new recordings are immediately made accordingly.

#### Data page B

I’ve met 7 oncoming vehicles.

A BMW overtakes me after 1.2 km and 1 minute 23 s.

The BMW overtakes the 4L

at km 11. 6 (from starting point) and 9 mn 34 s

(from starting time). I follow the 4L and I count oncoming LVS.

###### This alternating pursuit technique is applied as far as the end of the section.

Reminder: The FV does not take into account any vehicles other than LVs. If, for instance, the followed hare overtakes a bus, the FV will continue to follow the LV, or if the followed tortoise is overtaken by a truck, the FV will let the truck go and continue to follow the LV.

4.6 – Stopping at the end of a section

Data page C

End of section: we’ve got 1 page

The section is therefore 52.2 Km long

Data sheet C’

To be filled out at end of section

Section end landmarck

Time of  end of measurement (watch)

At the end of the section, you stop the measurements. You then note the kilometerage and the time at which you pass the end-of-section landmark which terminates the last measuring run sequence).

To know where (and therefore when) to stop measuring, the end of the section must therefore be identified accurately (see above, parag. 4.2). You will be in one of the following situations:

- either while travelling, you must recognize a previously identified landmark, based on its description and distance indicated between the start- and end-of-section landmarks,

- or while travelling, you must choose an appropriate landmark.

You will finally stop the vehicle and note, inter alia, the description of the landmark. It is naturally advisable to relax for a time between two sections, where the vehicle can leave the main road or make any manoeuvres or detours, as the kilometric recorder will be reset to zero when beginning again after the stopover.

The landmark at the start of the next section will rarely correspond to this end-of-section landmark (usually one is at the entrance to a town and the other at the exit), but the same name will be used for both these section limits (see parag. 4.2).

- Any measuring incident that makes measurements unprocessable (forgetting to read the kilometric recorder, mistake in handling the timer, etc.) will make it necessary to return to the last-known place where the correct procedure was used, in order to re-start the measurements. Use your kilometric records to find the right point again.

4.7 – The 90 km/h rule

The FV speed is limited to 90 km/h. If the FV encounters an LV constantly travelling at a speed of more than 90 km/h, or if after some time, the followed LV begins to constantly exceed 90 km/h, the FV lets it go and continues on its way alone, adjusting its speed to a maximum of 90 km/h.

The time and value of the kilometric recorder corresponding to this "abandonment" will be noted as the beginning of a new sequence called a "virtual pursuit".

Note: This rule is designed to level off the range of speeds of followed LVs for the following reasons:

- For LVs that exceed 90 km/h (more frequent on sections in quite good condition), their own speed has less to do with the condition of the road (or its characteristics) because such "high" speeds depend much more on other factors such as the condition and horsepower of each individual LV in question. Therefore the fact of including the highest speed range in our samples would soon be of little significance.

- 90 km/h is a typical level of speed limit. Speed limits are naturally another factor that affects the distribution of high speeds (in different ways depending on the country, but always to a limited degree as the strict, generalized application of the Highway Code is never encountered).

Virtual pursuit

In fact, the FV behaves as if it is following a "virtual LV" travelling at a speed of 90 km/h. This "virtual LV" has the same status (neutral / hare / tortoise) as the excessively fast LV that had to be abandoned. The cases are as follows:

- If the escaping LV was a hare : the "virtual LV" is also a hare; the FV therefore looks for a tortoise.

- If the escaping LV was a tortoise (suddenly accelerates, which requires it to be abandoned): the "virtual LV" is also considered as a tortoise and the FV therefore looks for a hare. The speed of this hare overtaking the FV will be certain to exceed 90 km/h. The FV will therefore also let it go (90 km/h rule), thereby beginning a second “virtual LV" sequence, but this time with a virtual LV considered as a hare. The FV, which is still travelling at 90 km/h, will now look for a tortoise.

The timer and kilometric recorder values will naturally have been recorded when the FV was overtaken at the beginning of the 2nd sequence.

- If the escaping vehicle was a neutral vehicle: the first LV encountered by the FV will be either a hare (if it catches up with the FV) or a tortoise (if it is caught up by the FV). The procedure is comparable with that described in the previous two paragraphs, as applicable.

The FV speed during a "virtual pursuit" may fall below 90 km/h, to meet the requirements of the Highway Code or for safety reasons. But wherever reasonable and permissible, the FV will try to adjust its speed to 90 km/h during this special sequence.

A situation may occur on a given section where speeds are mostly very high (consequently a section in good or very good condition), in which the FV will have to apply the 90 km/h rule (i.e. travel alone behind a virtual LV) over a long stretch of road. In this case, the measured speed is certain to be close to the technical 90 km/h ceiling, which is the absolute maximum limit.

Data page D

Possibility of briefly exceeding 90 km/h

During an ordinary pursuit sequence, if the followed LV only briefly exceeds 90 km/h (when overtaking), the FV does likewise and continues the pursuit.

The operator only decides to let the LV go if it becomes obvious that its natural mean speed is constantly greater than 90 km/h in the conditions due to the road alignment and the state of the road.

4.8 – 15 minute rule

This is a general, easily memorized rule: Most of the sequences (pursuit, virtual pursuit, solo, LV wait) are limited to a maximum 15-minute period as follows:

- Pursuit: If the same vehicle is followed 15 minutes at a time (because it has not encountered another LV meeting the hare-tortoise criteria, the FV must stop at the end of the 15 mn (and let this LV go) and begin to wait for the next LV to be followed. The timer is stopped (but certainly not reset to zero), the values of the timer and the kilometric recorder are noted. This is an "abandonment” which begins an LV wait sequence.

- Virtual pursuit: If the FV has followed the same “virtual LV” for too long, it must also stop for an LV wait sequence. Caution, in this case, the maximum 15 mn period applies to the total time “real pursuit time (before abandonment)” + “virtual pursuit time (after abandonment)”.

- Solo (see further ahead in this chapter, parag. 5.6): When the FV is travelling alone “solo”, for lack of traffic (after an unsuccessful LV wait), the FV must also stop at the end of 15 mn (new LV wait).

- LV wait: When the stationary FV has kept waiting for the next vehicle for 15 mn ( time to be checked on the dashboard clock or a watch): it re-starts solo (see parag. 5.6), which forms a new sequence. Note also that this rule is alleviated by the 15/10/5 mn rule, if this case occurs several times in succession, as detailed in the following paragraph.

#### Data page E

4.9 – The 15/10/5 minute rule

As required by the rule, when the FV is travelling solo for lack of traffic (after waiting in vain for 15 mn the first time), it never travels in these conditions for more than 15 mn. If it has not encountered any vehicle to be followed at the end of 15 mn, it must stop and observe a new LV wait. But this time it only has to wait a maximum of 10 mn. If, during these 10 mn,  no vehicle comes that can be followed, the FV will re-start and continue again solo, again for a maximum of 15 mn. Then each time the FV has to stop for lack of a vehicle to be followed, it will only wait 5 mn before re-starting solo. This succession of LV wait and solo sequences ends as soon as an LV appears that can be followed.

The maximum time intervals for solo travel are always 15 / 15 / 15 /… The maximum time intervals for LV waits decrease to begin with according to the rule 15 / 10 / 5 / 5 / 5 /…. If the FV succeeds in following a new LV, but this situation occurs again, the corresponding LV wait times re-start at 15 / 10 / 5 /…

Thus when LV traffic is non-existent in the measuring way, the LV travels as follows: 15 mn LV wait – 15 mn solo – 10 mn LV wait – 15 mn solo – 5 mn LV wait – 15 mn solo – 5 mn LV wait…

The wait times are checked by the dashboard clock or a watch, and not by the timer, which is stopped during these sequences.

##### 4.10 – Rule of the abandoned LV

Pursuant to the 15 mn rule, any LV followed for 15 mn at a time must be abandoned. But owing to chance traffic factors, it is possible to encounter the same LV again, in a situation in which in principle, it should be followed again.

The rule is that a vehicle that has previously had to be abandoned because it was followed for 15 mn at a time, must never be followed again (during the same measuring run).

It is not permissible to follow any vehicle previously abandoned at the end of 15 mn pursuit at a time. But it is perfectly possible to follow again a vehicle abandoned before the 15 mn limit, up to the limit of the full 15 mn at a time.

The same vehicle may be followed the first time, then abandoned to follow another LV at the end of 11 mn, then encountered again by chance further ahead and followed for 9 mn, and later again for 10 mn, and so on. But if any one of these pursuits lasts 15 mn at a time, it must be abandoned and must not be followed again on this same measuring run.

4.11 – Re-starting in wait position

Several reasons for making a FV interrupt measurements on a section have already been given. Others will be given further ahead in this handbook.

In the course of a section, when the FV is in an LV wait sequence and the first LV comes up in the right way of travel, the operator again triggers the timer just as the LV passes (and not a few seconds later when the FV starts off in pursuit of this LV). The FV then re-starts and follows the LV which is considered as a neutral LV, without taking any account of the status (hare or tortoise) of the last LV followed before stopping. The fact of stopping the measurements, which has triggered the LV wait, has reset the hare-tortoise alternation to zero. The same procedure as at the beginning of a section thus starts again.

Caution: a situation may arise by chance, in which the first LV that comes up is the one that was being followed just before stopping. In this case alone exceptionally:

- on the one hand, this LV retains its former status (e.g. a tortoise),

- and on the other hand, the 15 mn rule applies from the beginning of the first pursuit (in which case the time is cumulated).

#### Data page F

Traffic measurements:

When the stationary FV is waiting to follow an LV: counting of LVs is interrupted.

When the FV is travelling with the timer running, counting of LVs is performed.

4.12 – Taking break periods into account

Break periods should be avoided during measurements on a section. However, technical or human reasons may give rise to unexpected breaks: punctures, other incidents, unplanned refuelling, police roadblocks, etc.

During the "break", the vehicle may either be stationary or be travelling freely, thereby adding kilometres to the recorder.

The break is thus counted as a sequence of a special nature (that may possible introduce "parasite" kilometerage, consisting of the “detour”).

The "break period" sequence begins at the precise place where the FV stops applying the floating protocol. It slows down, turns off, stops, etc. The time and kilometerage are immediately noted to close the current sequence. If the FV has to travel during this break, the place of the beginning of the break is accurately identified so that it can be subsequently be relocated accurately (if possible, the beginning of the "break" is only triggered when passing an easily identifiable landmark).

At the end of the break, measurements must be re-started at the exact place where they were interrupted. The new kilometerage is then noted. The guided calculation on the data sheet naturally provides for the kilometerage travelled during the break to be deducted.

5 – ROAD AND TRIP RELATED DIFFICULTIES

##### 5.1 – The followed LV parks or turns off

If the followed LV stops or leaves the section, the FV stops (where necessary a few tens of metres farther on), and the operator records the kilometric recorder and time readings (LV wait sequence).

Caution: The time is recorded at the exact time when the LV stops. The kilometric recorder must be read at the exact place where the LV stops (which may be slightly farther on, after covering the distance separating the FV from the followed LV).

The timer is stopped and the FV is waiting for the next vehicle. This vehicle will be a neutral LV, unless the first vehicle that eventually comes up is the one that stopped previously, in which case it will retain its initial status.

5.2 – Multiple-stop LV categories

Some vehicles with frequent stops (e.g. public transport minibuses) disturb measurements too much. The crew may decide to delay the measurements and start them again during a time slot when vehicles of this type are fewer and cause less disturbance.

If a commercial vehicle or a service vehicle with frequent stops, which has already been followed by the FV, has already disturbed the measurements through its incessant stops (to embark and disembark passengers or goods, whether at scheduled stops or request stops) and is encountered again in a situation in which the FV should in principle follow it, the FV will simply ignore it. The FV can likewise ignore a very similar vehicle, whose behaviour is certain to be the same (e.g. two public transport minibuses serving the same route, etc.).

However, outside peri-urban areas, bush taxis should not be ignored as they are only likely to stop every half hour or more, and on some links, regular traffic may consist solely of a few bush taxis.

5.3 – Abnormally slow LVs

As a rule, very slow LVs (because they are old or overloaded, for example) must nonetheless be considered as "normal" traffic even if, by travelling in such a state, they fail to comply with the Highway Code. They must consequently be treated like the others, which means that it may be necessary to follow them for up to 15 minutes at a time.

Exception: LVs travelling at exceptionally slow speeds as a result of immediate conditions of “distress” outside the drivers' control (LV running on the wheel rim with a punctured tyre, or again an LV with an exhaust pipe that has just become loose and is dragging along the road, or an LV with a steering or suspension part that has just failed, or even an LV that has just had an accident, etc.) must never be followed.

5.4 – Dangerously fast LVs

Depending on the road and traffic conditions, some excessively fast LVs may be difficult or dangerous to follow because they would require the driver of the FV to take abnormal risks (for his own vehicle or for third parties (pedestrians, other vehicles, etc.).

An LV taking abnormal risks (e.g. an LV free-wheeling down a long slope) must not be followed by the FV.

For reasons of safety, there is no requirement to follow such vehicles at all costs in those cases. Quite the reverse, the FV must let the dangerous LV go.

Two situations may arise:

- If the speed of the LV is greater than 90 km/h: The LV must always be abandoned pursuant to the 90 km/h rule (in which case the FV continues on a virtual pursuit);

- If the speed of the LV remains below 90 km/h (but is in fact dangerous owing to the situation): the operator must immediately opt for “abandonment”.

In this case, the FV stops the measurements, stops the vehicle and begins an LV wait sequence, as at the start of the section. The readings of the kilometric recorder and the timer are recorded.

.

5.5 – Other difficulties in following an LV

The FV may encounter other problems when pursuing an LV:

- The FV is greatly hampered by another vehicle that comes between the FV and the followed vehicle for any length of time (a truck preventing it from overtaking safely, etc.).

- The behaviour of the followed LV is strange and dangerous (zigzags, multiple driving errors, that no doubt reflect abnormal behaviour of the driver, etc.).

- The LV, which realises that it is being followed by the FV, is visibly disturbed and considerably changes its behaviour, including its speed, making the current measurements less and less significant, (the LV tries to shake off the FV or slows down to let it overtake).

Each time the FV is clearly in such a case, the procedure is to abandon the followed LV – stop the FV, stop the measurements and wait for the next LV.

When an operator suspects that such a situation is beginning but needs to see how it develops for a while to confirm his suspicion, he opens a suspect sub-section while continuing the measurements.  Depending on what happens, he can decide between the following three options:

- immediately cancel the opening of the suspect sub-section, if the signs disappear quickly and do not recur. He then continues to pursue the LV

- close the sub-section while continuing the pursuit, when the situation ends quite quickly

- close the suspect sub-section, stop the measurements while abandoning the LV, stop the FV and begin to wait for another LV. This will be the case where the problem has become perfectly clear or if it persists at the end of 5 mn.

The rule is never to keep a suspect sub-section open more than 5 mn. Before the end of the 5 mn, you must have opted for one of these three solutions. The suspect sub-section merely gives a limited amount of time to think.

Data page G

The suspect subsection ends when the 4 WD overtakes (disturbance due to 4L  has disappeared).

Traffic measurements:

On suspect sub-sections, (where measurements are potentially hampered or disturbed): continue the measurements nonetheless, including the counting of oncoming LVs. The traffic count will conform with the corresponding speed measurements: either it will finally be validated or rejected and not used. There will be no half-and-half decision.

5.6 – Lack of any LVs: FV travelling solo

Where there is very little traffic, if the LV wait time attains 15 mn (time to be checked by the dashboard clock or a watch) without any LV appearing that can be followed, the operator triggers the timer and the FV re-starts alone travelling solo. It must then adopt the most natural speed in the light of the road condition and characteristics.

Note: Such "solo" sequences are certain to affect the statistical quality of work and are to be avoided as far as possible. They can only be a stop-gap solution for sections with too little traffic. But is will always be better, where possible, to schedule speed measurements during times or even on days with more traffic, and to separate them from the traffic level measurements.

When the only possible choice is to make self-measurements travelling “solo”, it is essential to take the greatest care.

Details on the FV behaviour travelling “solo”:

The driver must try to adopt a “natural” speed, i.e. the free speed that an ordinary LV, driven in an ordinary manner, would adopt at the same place. This point is important. If one year, a driver allows himself to adopt a high "natural" speed because he likes driving fast, and three years later another driver adopts a slower "natural" speed because he is careful or is not yet wide awake, the comparison of these measurements will be biased.

Reminder: The FV travelling solo must never exceed 90 km/h (90 km/h rule). If, owing to the excellent road condition and its very good characteristics, it is probable that an ordinary LV will exceed 90 km/h, the FV travelling “solo” will nonetheless remain within the 90 km/h speed limit.

How does a “solo” sequence end?

- The FV travelling solo encounters an LV travelling the right way (slower or faster): The FV follows the encountered LV, which is considered as neutral. The protocol for the start of the section begins again (the initiated pursuit is limited to a maximum of 15 mn).

- No LV to be followed comes during the 15 mn: The FV must not travel solo for more than 15 minutes at a time. When the 15 mn are up, the FV stops and waits for a maximum of 10 mn for an LV that it can follow. Then after a new solo sequence, the LV wait time drops to 5 mn (see above, parag. 4.9, the15/10/5 rule).

- The FV arrives at a place liable to generate traffic (a junction or a village): The FV travelling solo must then anticipate its stop before the 15 minute requirement is up. It positions itself for an “LV wait” as soon as it has passed a junction or exited from a village, for a maximum of 15 mn (this intentional interruption causes it to go back to the beginning of the 15/10/5 series).

#### Data page H

Traffic measurements:

When travelling solo, it remains mandatory to continue counting LVs coming the opposite way.

##### 6 – ROAD AND TRIP RELATED DIFFICULTIES

When the traffic is free-flowing, it is considered by definition that the speed of each LV taken separately, is not notably influenced by other vehicles: this is called its “free” speed. However, the general condition of the road, its geometry, difficult points (sharp curves, narrow engineering structures, deep ruts, large potholes, etc.) may cause variations in the free speed of this LV.

These are considered as normal traffic conditions. They do not therefore distort speed measurements on the section. Quite the reverse, as regards the road condition, it is precisely to evaluate its change over time that speed measurements are made regularly  on road users.

The same measuring procedure is consequently applied uniformly whatever the variations in the road conditions and characteristics throughout the section.

However, these same variations may have an indirect effect: they may spectacularly lower the threshold of free-flowing traffic, making the traffic much more sensitive to congestion. The risk of disturbance is increased. This will firstly result in increasing effects of mutual hampering between the different types of vehicles (e.g. the route becomes mountainous, trucks which on a flat stretch do not obstruct traffic to any degree, will begin to slow LVs down on uphill slopes with few overtaking possibilities, by creating cascading traffic jams). This interferes with the measurements as the vehicles cannot keep to their free speed..

We will first examine three specific road features that almost systematically result in high risks of disturbance, thereby making appropriate precautions necessary: village crossings, diversions and multi-lane roads.

6.1 – Village crossings

In principle, only crossings through villages or very small towns are considered here, because larger towns will almost systematically correspond to section changes.

Generally speaking, as soon as the FV enters a village and traffic becomes disturbed, there will be a “skip” sequence. Measuring is suspended (timer stopped, time and kilometric recorder readings recorded) and the FV continues on its way and travels through the village. Measuring will be re-started (chronometer triggered and kilometric recorder reading recorded) as soon as it has left the village and traffic is no longer disturbed. The “skipped” sub-section, corresponding to the village crossing, will inevitably by neutralized at the data processing stage.

Traffic measurements:

During “skip” sequences in which speed measurements are suspended (while travelling): the traffic count is naturally suspended also (speed measurements and traffic measurements go together).

The criteria for deciding to open a sub-section to be skipped close to a built-up area are as follows:

- rapid densification of traffic

- the multiplication of vehicles parked right at the roadside

- the presence of many two-wheeled vehicles and non-motorized road users including pedestrians, carts, etc.…

- visible urbanization of the area

- the multiplication of significant cross traffic flows (series of junctions, crossroads)

- and naturally, the actual reduction in speed, if it has this type of cause

If the crossing is very short and the village very small, there may not be any point in suspending measurements as only marginal, or very brief speed disturbance is to be expected. In this case, do not begin a “skip” sequence but simply open a “suspect” sub-section at the entrance to the village (and record time and kilometric recorder reading), without suspending the measurements, with a view to subsequently deciding, when processing the results with more time available, whether or not to neutralize this crossing in a suspect sub-section. If the traffic is not greatly disturbed (e.g. slight reduction in speed over a short distance), continue measuring, close the sub-section and there will not ultimately be any neutralization. (If the distance is extremely short, the opening of the sub-section can even be cancelled.).

If the crossing becomes very disturbed (or the disturbance lasts more than 5 mn): you must close the suspect sub-section in order to begin a skip sequence (i.e. immediately open a sub-section to be skipped).

Re-starting measurements after a skip sequence

After a skip sequence (during which a disturbed area has been crossed without performing any measurements), as soon as the situation reverts to normal, the operator closes the skipped sub-section and re-starts the measurements. If the situation allows, he must start again in the same position as before the skip sequence. More precisely:

- If by chance, when re-starting the measurements, the FV is still behind the same LV as before the skip sequence (the LV it was then following), you will merely immediately open a pursuit sequence while continuing to follow this LV normally. This vehicle will again be given its previous status (hare/tortoise), and for the 15 mn limit, both pursuit times will be cumulated (before the skip and after the skip).

Of course, if the FV happens to precede the LV in question, you will let this LV overtake it and immediately begin the pursuit sequence (with status maintained and cumulated pursuit times).

- But if when re-starting the measurements, the FV is behind another LV, this will be considered as a neutral vehicle for this new pursuit, with full 15 mn pursuit time credit. This pursuit sequence follows on directly after the closing of the skipped sub-section.

Of course, if this other LV is behind the FV, you will let it overtake before immediately beginning the pursuit.

- Lastly, if when closing the skipped sub-section, the FV is alone, without an LV in front or behind it, it must stop to wait for the first LV to be followed. If it is finally the LV followed just before the skip sequence that comes up, it will be given its previous status and the pursuit times will be cumulated. If it is another LV, this LV will be considered as neutral, with pursuit time credit intact.

Data page I

We enter the village: measuring is suspended to « skip » the village. At the end of the village, we’re again behind same vehicle. Measuring is restarted. Followed vehicle retains its neutral LV status.

6.2 – Diversions

Roadworks, a deficient engineering structure, or severe damage affecting the road (landslides, etc.) usually only cause slowdowns, temporarily alternating or intermittent traffic, or temporary roadblocks lasting a few hours, which all remain localized. In such cases, the rules for normal disturbances will apply.

However, there may be a diversion over a significant distance, which requires appropriate handling.

Diversions may be of many types, fundamentally different from one another. The cases may:

- be set up, and in principle signposted by the competent authorities or, on the contrary, offer a solution adopted spontaneously by users;

- simply use the existing secondary road network, or on the contrary use a way specially opened for the purpose, with a fairly rudimentary alignment;

- be really temporary or, on the contrary, be liable to last several years (even if it is officially temporary);

- have no alternative (only one possible route, the most usual case) or on the contrary, offer many alternative routes with traffic dispersion (a special case).

In all cases without exception, a special sub-section must be created for the diversion.

The following procedure will be applied, as applicable

On the one hand, for "temporary" diversions (i.e. all diversions in existence for less than a year):

- Begin a skip sequence (suspended measurements) covering the entire length of the diversion (the temporary traffic conditions on this diversion are meaningless for measurements on the section). In fact, it is not the diversion that will form the special sub-section and at the same time the skipped sub-section, but the original diverted road portion (even if this portion cannot be travelled because the traffic does not travel on it).

- The kilometerage to be taken into account (kilometerage to be noted solely for the skip sequence) is consequently that of the diverted portion. It is therefore essential, on either side of the diversion, in addition to the usual recording of sequence limits at the ends of the diversion (timer and kilometric recorder reading at the beginning and end of the diversion), to accurately connect the corresponding end in distance to a fixed, permanent landmark.

It will thus be possible to make a correct estimation of the diverted length of original road (exceptionally, if there is no better solution, the closest kilometric markers can help to make a rough estimate of the diverted length. For instance, beginning of the diversion 650m after the “PK 146” marker, end of the diversion 100m before the “PK 162” marker, length estimated at (160.90 – 146.65) = 15.25 km).

Note: Of course, it makes no difference whether or not a temporary diversion has alternative routes as no measurements are made on it.

On the other hand, for "permanent" diversions (here only diversions in continuous use for more than a year, whether they are official or spontaneous).

General case: permanent diversions without alternative routes

- Permanent diversions are to be considered as an integral part of the route. It is therefore the diversion itself that forms the special sub-section (and not the original diverted road portion). The valid length is that actually travelled on the diversion.

- If  the traffic is obviously disturbed with respect to usual criteria (LVs cannot travel at their individual free speeds but hamper one another; overtaking is difficult, etc.): open a sub-section to be skipped, which will double the special sub-section.

- But if the traffic on this diversion is only exposed to a slightly characterized disturbance, the special sub-section will first be doubled simply by a suspect sub-section (which, depending on developments, will be cancelled, closed with return to normal or closed for a skip sequence).

- But if there is no traffic: travel solo in 15 mn sequences, according to the 15/10/5 rule. And so on.

Special case: permanent diversions with alternative routes

When the original carriageway is heavily degraded over a long distance (or totally impassable), LV traffic or even all the traffic, no longer uses the original road. This gives rise to a spontaneous diversion, fairly close to the edge of the road, very often with alternative routes causing some traffic dispersion.

Here, instead of a diversion along a single route, users take various paths which form alternative routes, little made up (if at all) and more or less physically manifest. Usually, these diversions are long-lasting, to be considered as permanent. They are often spontaneous (imposed by the general state of the original road and/or its engineering structures, or any other serious damage due for repairs). Sometimes they are inadequately made up official diversions, which degenerate relatively quickly into diversions with alternative routes, because the initial alignment, without a paved load-bearing area, deteriorates rapidly under heavy traffic. The general principles specific to permanent diversions, as described above, will apply but with a few additional difficulties.

When most LV users leave the original road, the FV must follow these users and make measurements as usual. It will try to take the dominant diversion.

This diversion will often have to be chosen on the basis of vehicle tracks. Where there are no indications, you should simply try to opt for the passable alternative route, like everybody else.

- Like for any diversion, the operator will have opened a special sub-section also doubled by a suspect sub-section, but exceptionally this suspect sub-section will not be limited over time.

- On the data page, you must note the fact that there are signs of traffic dispersion, because this may invalidate the oncoming traffic count (if part of it is missed out).

- Whatever the case, as the situation is particularly uncertain, the travel time is still important information for the final decisions to be taken at the data processing stage. It is therefore out of the question to suspend measurements here by a skip sequence.

- If specific difficulties require technical stops or detours (e.g. detours to identify the route, U-turns because of wrong trip routing, the FV has got lost, etc.), they must be carefully noted as usual in the appropriate type of sequence, so that they can subsequently be eliminated or deducted.

6.3 – Multi-lane roads

The localized presence of a second lane in the measuring way, over several hundreds of metres or a few kilometres will not in itself significantly disturb the measurements. This is the case for overtaking lanes, for instance, or for uphill slow-vehicle lanes.

But when a link has more than one lane per way over a significant length (more than 30 km, and therefore covering one or more sections) some precautions should be taken. This is often the case for expressways and naturally, motorways.

### Speed segregation phenomenon

On multi-lane roads, a natural speed segregation phenomenon occurs between the flee-flowing conditions (no hampering of vehicles by other vehicles) and the congestion conditions. The traffic spontaneously organizes itself into lanes according to the speed. The slowest-moving vehicles prefer the outside lane and the fastest-moving vehicles choose the inside lane. The inside lane, which was only an overtaking lane in free-flowing conditions, now becomes a traffic lane in its own right. The traffic flow has physically split up into two (or more) speed strata, but the range of speeds in each of these strata is very narrow (there is a rhythmic effect). Individual speeds are not very natural, and driving a floating vehicle that is continually moving from one lane to the other, would be difficult and dangerous. In these conditions, it is not possible for the FV to make valid speed measurements.

Moreover, there is no point in making the usual measurements of oncoming traffic, because it would be impossible to process them as the speed of the floating vehicle itself is not correctly adjusted.

How to ascertain whether there is speed segregation

A single indication is enough to characterize the phenomenon: a number of fast vehicles travelling continuously on the inside lane, without seizing the opportunity to move onto the outside lane.

When to make speed measurements

Although this phenomenon is very usual on a motorway (it occurs in traffic that is still free-flowing, long before any congestion occurs), it is essential to avoid it when making measurements. For these speed measurements, try therefore to choose off-peak hours, and check that the phenomenon has been properly avoided when measuring starts.

Note: In periods of free-flowing traffic, speeds can be quite high because motorways not only have comfortable characteristics but are most often priorities for road maintenance. It is therefore probable that you will often apply the 90 km/h rule.

For speed measurements, it will thus suffice to note, even in peak periods with segregation, that all the lanes, including the outside lane, are travelled at more than 90 km/h the whole way. The free speed to be adopted is then 90 km/h, and the travel time will follow from this speed. But this does not solve the traffic measurement problem.

Traffic measurements:

For multi-lane roads with a systematic speed segregation at usual times, traffic counts must be separated from speed measurements.

In this case, counting will be made:

- from fixed stations,

- in business hours and on business days

- from two stations only: one near the start of the section and one near the end of the section

- in a single way of travel (as usual),

- over the same fixed duration for both stations, exactly the same as the actual duration of the FV run on the section during the speed measurements (this means that you must make the traffic measurement after the speed measurement run).

The result to be taken into account is the addition of the counts from both stations, which will then enable the standard formulae for data processing to be applied.

7 – TRAFFIC DIFFICULTIES

We have just seen the special precautions to be adopted when the road itself has specific characteristics that result almost systematically in traffic difficulties: village crossings, diversions and multi-lane roads.

We will now examine several cases in which the traffic conditions themselves cause difficulties: particularly the behaviour of non-LV users (trucks and similar vehicles).

The presence of non-LV road users and any annoyance they may cause LVs (temporary overtaking difficulties, etc.) also form normal traffic conditions. Their ordinary impact on the common speed of LVs cannot be considered as abnormal distortion.

But depending on the circumstances, non-LV users can begin to cause very severe annoyance (whether isolated or cumulative phenomena) and significantly disrupt measurements. Furthermore, as already mentioned, some road characteristics, particularly narrow carriageways, can compound the risk of such obstruction to traffic.

The only cases that cause problems are those where these users form severe obstruction to traffic by creating convoy phenomena (overloaded trucks travelling uphill) or even causing traffic jams, i.e. exceptional situations.

Truck traffic must always be regarded as a potentially disturbing factor of measurements. In sectors where truck traffic is heavy, particularly where road characteristics are unfavourable, it is highly advisable to avoid times when truck traffic is heavy.

Caution: peak hours for trucks, which are to be avoided, can sometimes be very different from peak hours for LVs. For instance, approaches to port areas depending on customs opening/closing times, etc.

Convoys or traffic jam phenomena are often caused by trucks, but not only. We will successively examine the case of convoys, then that of traffic jams (and congestion in general). Then we will study various other cases of traffic difficulties.

7.1 – Convoys

LVs sometimes get caught up in convoys whether these are organized or, on the contrary, unintentional.

Unintentional convoys form naturally because of the traffic itself and the road characteristics (overtaking difficulties, sharp decelerations of trucks on upward slopes or in areas with many sharp bends, etc. An organized convoy naturally gives rise to an involuntary convoy immediately behind it, consisting of vehicles trapped in its wake.

Encountering a "convoy" is a situation which prevents correct measurements and from which it is necessary to get free as quickly as possible, either by overtaking the convoy and leaving it behind, or by letting it go in order to meet up with free-flowing traffic again (with the risk of catching up with it again quickly).

Note: These convoys of the "cascading traffic jam" type are usually caused by the first slow vehicle that is difficult to overtake, behind which other vehicles of all types bunch up and are momentarily trapped. Only the first vehicle in the convoy (which is blocking the others) is travelling at its own free speed. All the others are travelling at a constrained speed. Therefore only the first vehicle can be taken into account for measuring (if, and only if, it is an LV). No overtaking within the convoy, which simply testifies to drivers' impatience, has the slightest usable significance. It is all useless traffic for measuring purposes.

If the first vehicle in the convoy is an LV, it is certain to be a slow vehicle (because it has slowed all the others down, including the vehicle being followed by the FV). If the FV was previously looking for a hare (and consequently following a tortoise), it is forced to follow a super-tortoise (at the head of the convoy), which thus breaks the alternation rule. Furthermore, if a first hare manages to escape by overtaking the head of the convoy, it will not always be possible to follow it instantly because the FV itself will find it difficult to overtake the convoy. And when at last the FV has overtaken the convoy, it will only encounter unrepresentative traffic for some time, because this traffic will have been filtered (only the most aggressive vehicles will have been able to overtake and get clear).

How to treat a "convoy" in practice

First open a suspect sub-section as soon as you suspect that you have come upon the tail of a convoy (but if there is no room for doubt, there is no need for a suspect sub-section…).

Then as soon as you realize that you are actually trapped in a convoy, you must open a “skip” sequence, i.e. suspend measurements by stopping the timer, and you must try to overtake it. In fact, this “skip” sequence is an attempt at a skip which may not be successful. Technically, it is nonetheless a skip sequence because you will continue with all measurements suspended.

Therefore you will now try to escape from the convoy by skipping it. But be careful: never alone but while following another LV, so that as soon as you have overtaken the convoy, you can restart the measurements properly, while getting further ahead and not having to wait in the same position. You are only allowed 5 mn to achieve this (time is short because measurements are suspended for the moment). Two situations may arise:

- You are unable to overtake the convoy quickly (by pulling out behind another LV):

The rule is never to try for more than 5 mn (time to be checked by a watch or the dashboard clock, as the timer is stopped). If you are unsuccessful at the end of these 5 mn, stop and let the convoy go. Make a U-turn and go back to the place of origin of the disturbance, noted on your data page (place of origin of the skipped sub-section; or where applicable, the beginning of the suspect sub-section opened just previously). Note this U-turn carefully on your data page because it creates a detour that will have to be deducted from subsequent measurements when processing the data.

When you are back at this place of origin of the disturbance, observe a sufficiently long standstill sequence for the convoy to get well ahead so as not to become caught up in it before the end of the section.

When this standstill time seems long enough, position yourself to await the first LV to be followed (LV wait limited to no more than 15 mn, as usual).

Data page J

Timer is stopped as soon as measuring is suspended and during skip, U-turn, standstill and LV wait.

Timer is triggered again just as a new pursuit begins.

- You are able to overtake within 5 mn, while following another LV also escaping from the convoy:

A new pursuit sequence begins, in which the LV in question is considered as a neutral LV.

7.2 – Congestion and traffic jams

When preparing the measurement campaign:

Speed measurements are to be made in normal traffic conditions, but outside peak hours wherever there is a risk of congestion. In the case of heavily-trafficked portions (outskirts of big cities), speed measurements can be made on Sundays, as an exception to standard rules. The point must be carefully justified on the data sheets.

On the road:

Heavy decelerations, or traffic jams because of roadworks, roadblocks, etc. are treated simply as a suspect sub-section or directly as a skip sequence or a standstill sequence, as appropriate. Sometimes this will entail successively a suspect sub-section, then a skip sequence or a standstill sequence.

For other causes of local congestion, such as major junctions or markets, a skip sequence will be necessary.

If the cause of the disturbance is temporary and greatly affects measuring, make a U-turn and re-start measuring at the place of origin of the disturbance, once the cause of this disturbance has disappeared.

Traffic measurements

Some congestion is treated in the same way as the case of multi-lane roads with a speed segregation phenomenon. Traffic counts must be separated from speed measurements when the latter are made on Sundays or during off-peak traffic hours.

In this case counting will be made

- from fixed stations,

- in business hours and on business days

- from two stations only: one near the start of the section and one near the end of the section

- in a single way of travel (as usual),

- over the same fixed duration for both stations, exactly equal to the actual duration of the FV run on the section during the speed measurements (this means that you must make these traffic measurement after the speed measurement),

The result to be taken into account is the addition of the counts from both stations, which will subsequently enable the standard formulae for data processing to be applied.

##### 7.3 – Other traffic difficulties

A number of events can disturb traffic by causing slowdowns or even blocking the traffic to some degree.

Such cases will usually require a skip sequence (overtake the disturbance), or even sometimes a standstill sequence (wait on the spot).

Police roadblock, toll or similar episode

The FV has to slow down and it is clear that the traffic will be blocked or filtered: start a skip sequence, possibly over a few tens of metres, and at low or quasi-zero speed (it is quite possible to be blocked for a considerable length of time during a “skip” sequence). Measurements will be re-started once past the roadblock (whether or not it is official) or the toll, once the traffic has become normal again.

If the approach to a roadblock has first caused a slowdown (without the cause being necessarily known immediately), you will have opened a suspect sub-section, which will probably be neutralized in the end.

Accident

In the case of an accident blocking the traffic or slowing it down: adopt the same procedure (skip sequence or, if it is totally blocked, standstill sequence while waiting for the road to clear).

Serious damage to the road or to engineering structures

In the case of a recent event that directly affects the road (falling rocks, landslide, serious damage to an engineering structure, etc.), but where traffic can continue to pass: a simple skip sequence.

Roads unexpectedly cut off

However, if the road is cut off (bridge carried away or destroyed, landslide or any impassable obstacle without an immediate possibility of a bypass or diversion) the FV cannot behave differently from the rest of the traffic: measurements on the remainder of this section are temporarily abandoned.

When a section of a reference network is cut off, with no hope of it being reopened before the end of the measurement campaign, this section must nonetheless be measured over its entire, or virtually entire length (by subsequently gaining access to the cut-off point from the other end, providing it is possible to reach it by another route). As the traffic cannot get past the cut-off point, it will naturally be taken into account as a sub-sectioning point (opening of a special sub-section). This situation will usually require a change in the measurement campaign plan, which must be urgently validated according to the usual procedure.

Roadworks

In the event of localized roadworks causing traffic to slow down: skip sequence. If it is completely blocked, standstill sequence.

If the roadworks are intended to improve a fairly long portion of road: As soon as the FV approaches the roadworks (signs announcing the roadworks or actual traffic slowdown), the operator opens a suspect sub-section before making a final decision.

How to re-start the measurements once conditions are restored to normal

When you have opted for a skip sequence, the procedure is that detailed above in this chapter, parag. 6.1.

When you have had to wait on the spot (standstill sequence), the procedure is strictly the same:

- If the LV followed before the standstill sequence has also had to wait, you move off in pursuit of it without an intermediate LV wait sequence, with maintained LV status and pursuit time credit being used.

- If, at the time the standstill sequence is considered long enough (once the measuring conditions are restored), a different vehicle comes up, you engage directly in a pursuit (with neutral status and pursuit time credit intact).

- If you are isolated when restarting, you must position yourself in an LV wait sequence. And so on.

It is always preferable, where the situation allows, to place yourself in the same conditions as before the disturbance. But of course, this is not always possible.

8 – WEATHER DIFFICULTIES

Measurements must be made in normal traffic conditions.

If traffic is disturbed by continuous rain that reduces visibility or if a carriageway is waterlogged and slippery, measuring is not possible.

Likewise, if a section is totally or partially impassable (e.g. succession of mudholes due to bad weather) measuring is not possible either.

These cases will cause measurement on a section or even throughout a region to be postponed until conditions are more favourable (subject to the client's agreement).

8.1 – Precipitations

- If there is a shower before starting the measurements, you must wait not only for the end of the rain but also for a complete return to normal traffic conditions (for paved roads, a dry or virtually dry carriageway) before starting the measuring run.

- If a shower begins during the measurements, the operator records the chronometer and recorder readings to open a suspect sub-section).

If the first raindrops do not continue, the opening of the sub-section will simply be cancelled.

If the shower does not last more than 5 mn, and if the traffic conditions are not affected by it for long, the suspect sub-section is closed.

You must interrupt the measurements if the disturbance becomes characterized or if its effects last more than 5 mn. (The FV stops and observes a standstill sequence until the situation reverts to normal). There is no point in observing a skip sequence in the rain or on a slippery road, the solution is to wait on the spot.

As an absolute rule, in the event of sudden rain, measuring must immediately be interrupted. To re-start measuring, wait for normal conditions to be restored.

Note: The criteria for traffic disturbances due to rain are as follows:

- continuous use of windscreen wipers by most motorists

- loss of visibility

- difficult or hazardous overtaking

- slippery carriageway, puddles at the roadside, water on the carriageway surface

- effective loss of speed from one or other of these causes

To judge the end of a disturbance, you must at least wait until the carriageway is not slippery any more and passable again as usual. Regarding unpaved roads, it obviously takes a much longer time. This can last several hours, even a few days in case of heavy rain ; in such a situation, the campaign plan may be highly disrupted.

Other weather events (such as sandstorms) that affect driving are to be similarly treated.

8.2 – Passability drops

If a section includes a portion of road temporarily made not easily passable due to a recent weather event (e.g. earth track passages where mudholes are formed by a storm), you must begin a skip sequence . This is also the case for portions undergoing improvement works.

You must also suspend the measurements to begin a skip sequence at certain not easily passable places (e.g. submerged invert, isolated mudhole).

As long as most of the traffic passes, the FV will pass. But in situations where driving becomes hazardous for men or equipment, the FV must not try to pass (see above, parag. 7.3 entitled "Roads unexpectedly cut off").

9 – HOW TO HANDLE DIFFICULTIES: RECAP

The main situations are described in the previous sections, but many others may also arise. The operator will then have to react using his common sense and experience, while complying with the logical process detailed above to deal with the most usual incidents.

The following logical framework is designed to recap the treatment of disturbances and clarify ideas. It forms a commentary to the following decision tree.

Remember that the case of "convoys" is treated quite differently, as an exception (see parag. 7.1).

When you encounter signs of disturbance:

When the disturbance is characterized, i.e. clear, and the impact on speed is significant, the measurements cannot be continued.

When you consider that the disturbance is localized, you naturally stop the timer, but the FV continues on its way. You thus begin the sub-section skip sequence, which is assumed to be quite short. Measurements will be restarted as soon as you are past the disturbance and traffic conditions have reverted to normal.

However, if the disturbance is transient and theoretically widespread, you will stop the FV and position yourself at a standstill. Measurements will be restarted at this stopping place as soon as the disturbance has ended.

Any long-lasting disturbance, whatever its extent, will be treated by a sub-section skip sequence. In this case, there is no point in waiting.

When the disturbance is uncharacterized and its impact on speed remains uncertain, the operator opens a suspect sub-section to mark the beginning of a potentially disturbing situation.

In all cases, a suspect sub-section must not be kept open more than 5 mn. In the 5 mn following its opening, you must choose between the following three solutions:

Choice 1:	Simple cancellation of the suspect sub-section in the event of a false alarm (if the disturbance disappears before it has hardly begun). In this case, it will suffice to indicate this cancellation by a simple comment on the data page, on the line corresponding to the opening of the suspect sub-section. The measurements are then continued.

Choice 2:	Closure of the suspect sub-section if the disturbance has deteriorated little or not at all and disappears quite quickly. The measurements are then continued immediately.

Choice 3:	Closure of the suspect sub-section as soon as the disturbance becomes characterized. This is the same case as that described earlier, in which the measurements cannot be continued, and it is treated in the same way.

It is not permissible to continue for more than 5 mn in a suspect subsection.

If 5 mn pass without a decision becoming clear (the signs persist but the disturbance remains uncharacterised) then by default, you must make choice 3: closure of the suspect sub-section and stopping of the measurements.

For any suspect sub-section, wherever possible, the operator indicates on the data page whether he considers that the measurement has been little or not at all disturbed and thus deserves to be validated (annotation “OK” in the righthand column reserved for suspect sub-sections) or if he considers that the measurements have been really distorted and thus deserve to be invalidated (annotation “OK”). In cases of uncertainty, he will not note anything and the decision on the sub-section will be postponed until later (data preprocessing stage).

Remarks: When it has not been possible to continue the measurements, with or without a prior suspect sub-section:

- After opting to stop the FV, if a reasonable length of time in the standstill sequence has not given rise to any signs of improvement (the disturbance finally seems to be long-lasting), you will decide to restart alone, with suspended measurements, in order to skip the disturbed sub-section. The length of time will be considered unreasonable as soon as it jeopardizes the measurement program of the current half-day.

- Conversely, after opting for a disturbed sub-section skip sequence, if the disturbance finally proves to be widespread and transient, stop the FV and place yourself in a standstill sequence.

Furthermore:

When there is a risk of exceeding the critical 10% threshold (see parag. 10.7), insofar as possible and without jeopardizing the programme of the half-day, opt for an FV stop, followed by a standstill sequence, rather than a sub-section skip sequence, if uncertainties exist on the extent and duration of the disturbances.

In all cases, when there are suspect sub-sections or prolonged sub-section skip sequences, in order to comply with the critical 10% threshold, you must examine whether it is advisable to repeat the measurements immediately at the cost of a U-turn (see parag. 11.1).

In general, as soon as there is any doubt on the decision to be taken, there is never any harm in first stopping (and beginning a standstill sequence intentionally), as this enables you to examine the situation calmly, even if it means restarting after thinking for a while,  in a suspect sub-section or a skip sequence.

The following scheme, or “solution tree”, summarises the procedure for handling disturbances.

MEASUREMENT DISTURBANCE

PROCEDURAL ERROR OR TECHNICAL INCIDENT

U-turn

The Solution Tree

to re-start measurements at last known problem-free place

non characterized

characterized

Opening of a suspect sub-section to observe how things develop while continuing the measurements.

False alarm

Fairly uncharacterized disturbance that disappears before 5 mn

Disturbance that turns characterized or persists over 5 mn

Cancellation of suspect sub-section

Closure of suspect sub-section

Closure of suspect sub-section

Continuation of measurements

Measurements cannot be continued.

Extent?

Duration?

localized



skip

widespread





standstill

transient  

Interruption of measurements

long-lasting 

skip

skip

Always keep in mind the 10% critical threshold.

Where necessary, examine advisability of a U-turn tour.

10 – DATA PREPROCESSING

Data preprocessing is an important stage in the processing of data on a section.

It must be performed as soon as possible after the measuring run, so as not to completely forget the sequence of operations.

In practice, the most appropriate solution will be for the operator to perform preprocessing at the end of each measuring day, for all the runs scheduled for that day.

10.1 – Why preprocess?

Data preprocessing meets a threefold objective:

- take decisions immediately to neutralize measurements on suspect sub-sections (the recollection of the circumstances of the measurements are still intact: you have at hand the factors for the right decision),

- perform self-inspection of measurements, by checking that the data are complete and homogeneous (if not, they can immediately be corrected where possible, or it can be decided to repeat measurements as soon as possible if they are unprocessable),

- check the proportion of the “measurable” length on which you have no validated measurements, over the entire section: above 10%, the measurements must be repeated (see further ahead, parag. 10.7).

10.2 – General presentation of preprocessing

Preprocessing is performed run by run, without interference between runs on the same section.

The data preprocessing principle consists in eliminating invalid measurements from the unprocessed data (traffic count, measuring time, distance covered). For the final calculation, the only situations taken into account are those in which measurements have been made in normal traffic conditions. You must therefore deduct distances travelled during skip sequences, U-turns, breaks and on suspect sub-sections that you have decided to neutralize. For the latter, you must also deduct their travel times and eliminate the corresponding traffic counts. Any measuring sequences with obvious inconsistencies (due to data entry errors or omissions) must also be eliminated.

10.3 – The two preprocessing stages: sorting and calculations

Measurements are preprocessed in two stages:

- Firstly by filling out the lefthand part (preprocessing area) of each data page for the relevant measuring run;

- Then by transferring some data to the section data sheet, the recap page where calculations will be made to obtain the final results for the run (the only results that will be subsequently be transferred at the data processing stage).

10.4 – First sort: grouping the sequences into packets

The first preprocessing phase (ref. 2 on the data page) consists in going through the entire series of sequences in chronological order (data entry lines) and grouping them into “packets”.

A normal packet ("N" type) consists of a set of successive sequences that can all definitely be processed: succession of pursuits, virtual pursuits (with possible changes in the followed LV), solo sequences but without opening or closing of a suspect or special sub-section. The standstill or LV wait sequences can also be included in an "N" type packet as they do not affect the results (no measurements of time or traffic, no travelled distance).

An "N" packet thus groups together sequences A, B, C and D and E of the data page, without any opening or closing of a sub-section being mentioned opposite them in the corresponding column.

A separate packet must be formed, of a specific type, whenever the following occur:

- Skip sequence (F type), which alone must form an F type packet,

- U-turn sequence (G type), which alone must form a G type packet,

- Break sequence (H type), which alone must form an H type packet,

- Opening of a suspect sub-section: the complete succession of measurement sequences performed on this sub-section must form an S type packet. An “S” packet, like an “N” packet, can thus group together sequences A, B, C, D and E.

In fact, all sequence changes resulting from disturbed situations (or suspected as such) must cause a new packet to be created.

All sub-section limits also require a change of packet.

There will also be a change of packet when a special sub-section is opened or closed (e.g. for a diversion or a change in the nature of the paved/unpaved carriageway). A special sub-section will sometimes form a single N type packet but in fact it may include all types of packets. In particular, suspect sub-sections (mandatory S type packets) or skipped sub-section (mandatory F type packets) can be superimposed in whole or in part; as can G or H type packets.

For each packet thus formed, the operator calculates and records in the preprocessing area (under ref. 2), the time intervals, distance intervals and corresponding LV counts.

#### Data page K

- First identify the packets by drawing lines

- Number the packets in chronological order

- Identify the type of each packet

- Calculate lengths and time intervals and total LV counting

10.5 – Second sort: determining invalid packets

The second preprocessing phase consists in neutralizing or not neutralizing suspect sub-sections, i.e. S type packets (see ref. 3 of the preprocessing area).

The operator must decide for each case based on the actual circumstances of the measurements. He records his decisions on the data page and will then take them into account for all his calculations up to the end of the preprocessing phase.

It will subsequently be the coordinator's task to re-examine these choices, and where necessary, to correct the calculations on suspect sub-sections for which his final choice is different (see further ahead in chapter 3, parag. 1.3 “Validating preprocessing choices).

The neutralization of a suspect sub-section is required when, upon analysis, the results of the actual measurements are definitely considered to be doubtful or not significant for various reasons.

In fact, it must be determined whether the measurements have been made in "correct" traffic conditions, i.e.:

- for speed measurements, in conditions close enough to those of free-flowing traffic,

- and for traffic level measurements, excepting any occasional conditions in which the traffic is unusually heavy or unusually light (however, off-peak hours and peak hours are regular situations, quite acceptable for measuring traffic, as the level will subsequently be rectified accordingly).

All “correct” measuring conditions are described earlier in this handbook.

The following guidelines can facilitate the operator’s decision, even before making any subjective appraisal:

- If the duration of the suspect sub-section does not exceed 30 seconds, it may systematically be decided to validate it (unless for exceptional reasons). This is because it will have little impact on the final result, and a correct measurement would suffer from a relative inaccuracy that would be just as important (read-out inaccuracy).

- It may clarify matters to compare the actual speed on this sub-section (distance travelled/travel time), with speeds observed directly upstream and downstream. Clearly unjustifiable differences, assuming the measurements are not disturbed, would require neutralization, irrespective of any other considerations

It must be borne in mind that, aside from suspect sub-sections, lengths that contain errors that cannot be corrected, have data inconsistencies, or are shown to be measured in irregular conditions, must also be neutralized. They will be placed in separate packets and then neutralized. Take care to mention such cases explicitly on the data page (preprocessing area).

For each S type packet (suspect sub-section), the operator states in the preprocessing area whether his choice is to neutralize this sub-section or not.

For F, G or H type sequences, there is no validation problem: they are neutralized as a matter of course.

Data page L

- S sequences: decide either to validate or to neutralize them

- F sequences: identify them by a bullet

10.6 – Third sort: identifying non-re-measurable packets

The third preprocessing phase consists in separating packets (or sub-sections) neutralized into re-measurable packets (or sub-sections) and non re-measurable packets (or sub-sections). We will examine successively (ref. 4), the cases of S type packets (suspect measurements it has been decided to invalidate), then (ref. 5) those of F type packets (skip sequences, that have not given rise to measurements).

In each case, it must be decided:

- whether the packet is re-measurable, i.e. whether repeat measurements are likely to produce processable data on these sub-sections, either because the disturbance factors will probably have disappeared or because it is possible to arrange to avoid them (e.g. by separating speed and traffic measurements);

- or whether the packet is non re-measurable, i.e. whether, on the contrary, it is definitely impossible to obtain valid measurements on the sub-section, because the disturbances are permanent and unavoidable there (village crossing, etc.).

This distinction (between re-measurable and non-re-measurable) becomes crucial when the neutralization rate over the entire section is already high.

For each neutralized packet, the operator thus indicates in the preprocessing area (refs. 4 and 5) whether or not the corresponding sub-section is re-measurable.

Caution: Although the non re-measurable part of a section has to be estimated, it results from objective factors that cannot be avoided. It may be considered impossible to re-measure up to 60% of the total length of a section continuously passing through a succession of built-up areas, and always in a dense urban or peri-urban area. Nothing can be done about this.

But do not conclude too quickly that it cannot be re-measured; sometimes by choosing a different time of day, you will be able to make successful measurements in peri-urban areas.

# Data page M

Packet 4F: Non-remeasurable as it is a built-up area, which corresponds to a permanent and unavoidable disturbance

Packet 6S: Remeasurable, as it is a convoy, non permanent disturbance

Packet 7F:

Remeasurable, as these are consequences of the convoy. Furthermore, we have subsequently made a U-turn to remeasure the subsection: by definition, it is remeasurable

Data sheet N

First stage:

Transfer the packets by order number (so as not to forget any of them) from the data page into the corresponding charts.

10.7 – Second stage: calculations and conclusions

At this stage, the operator transfers the intermediate results from the data pages to the data sheets and makes the calculations for the entire run.

These calculations enable:

- Results to be obtained that will subsequently be transferred to the calculation workbook (net counting, net time, net measured length, measurable length, gross length (see chapter 3)

- The cases to be identified for which re-measuring is mandatory, using a simple rule that determines the threshold at which re-measuring becomes mandatory: the “critical” 10% threshold.

The critical 10% threshold

# Measurable sub-sections that have been neutralized and not re-measured must not exceed 10% of the measurable length of the section (namely, its gross length minus the non re-measurable length). In other words, the net length (on which valid measurements exist) must be attain at least 90% of this measurable length.

Although this rule may seem rather complicated, the data sheet gives simple calculation guidelines that enable this on-the-spot check to be made quickly and easily.

Data sheet O

- Make calculations for each chart and transfer results following arrows

- Make calculations for each line

- Ensure the result for « Km as counter  displayed » corresponds to the last Km result recorded on the data page

- Make the required decisions on re-measurements, depending on compliance with critical treshold (in this case: no re-measurements)

10.8 – Preprocessing: a self-inspection and control tool

The recap page (section data sheet) with its calculation guidelines is designed to check the consistency of results and is thus of great help:

- to the operator, for self-inspection,

- to the coordinator, for internal control prior to data processing

- to the external controller

11 – RE-MEASURING

11.1 – U-turns during measuring runs

Measuring incidents

During a run on a section, any measuring incident that makes measurements unprocessable (forgetting to read the recorder, error in handling the timer, improper implementation of the protocol) requires them to be stopped immediately. You must then make a U-turn to return to the last-known place where the correct procedure was used, in order to re-start the measurements.

Suspect sub-sections and skipped sub-sections

Assuming that the operator, after opening a suspect sub-section owing to a doubtful disturbance, has had to interrupt the measurements because the disturbance has become well characterized. He then closes the suspect sub-section. If the suspect sub-section is very likely to be neutralized, the operator must then return, or not return, to the place of departure of this suspect sub-section to re-measure it?

It is not absolutely mandatory to make a U-turn to repeat the measurements.

However, when the sub-section is immediately re-measurable in better conditions, it may be advisable to make an immediate U-turn if you are close to the critical threshold. In fact, at the slightest doubt, it will be highly advisable for the operator to make a U-turn if the 10% threshold of the unprocessable measurable length is liable to be reached.

The same is true at the end of a prolonged “skip” sequence, the length of which may cause problems, in cases where measurements become possible. The case of a suspect sub-section followed by a skipped sub-section must be analysed in the same way.

In general, to avoid the inconvenience and extra costs of an additional campaign to repeat measurements, it is up to the operator to anticipate and know when to perform repeat measurements immediately at the cost of making a U-turn.

11.2 – Re-measuring after the end of the measuring run

Finally, when the measuring run is complete, it is compliance with the 10% critical threshold that will determine whether re-measuring is required before the results on the section can be processed.

The unprocessable re-measurable length can group together skipped sub-sections, suspect sub-sections that it has been decided to neutralize and further deviant sub-sections owing to uncorrectable errors (erroneous data entry or protocol application), identified during preprocessing.

If re-measuring is mandatory, two cases may arise:

- The new measurements will be made on the same day (if the reasons for the disturbance were transient and the remaining time will allow): in this case, new partial measurements, limited to a small number of sub-sections, are permissible. These new measurements can be added to the same data sheet (but it is advisable to consider whether it would not be more practical to re-measure the entire section; in this case a new data sheet will naturally be opened.)

- The new measurements cannot be made on the same day: it will then be mandatory for them to extend over the entire section, and a new data sheet must be created (new data sheet with new data pages).

Whenever a problem section has to be re-measured, whether or not on the same day (whether it is on a return run scheduled from the start, a precautionary repeat measurement, or a mandatory repeat measurement to comply with the critical threshold), you must naturally ensure that you will not come across the same disturbance (by changing the time of day, etc.).

Note: Throughout any measurement campaign plan, allow a safety margin so that you can fit in a few repeat measurements where necessary, without calling the entire trip scheduling into question.

#### Special case

Where re-measuring the section would jeopardize the measurement campaign plan, if a return run is already scheduled on the section, or if an outward run has already been made in acceptable conditions, no re-measuring need be done. The measurements obtained during this other run will be the only results finally processed. Whatever the case, provided that the repeat measurements are not a constraint (if they can be made on the same day, for instance), they must be made both to obtain more complete results and as a precautionary measure (you can never be sure that the second scheduled run will take place in satisfactory conditions).

1 Except for a cut-off road or a diversion appearing in the period between two runs.