SOURCE (Standard Overall Ultralite Road Care Estimate)

Practical lessons from a SOURCE national campaign conducted in Ghana

The SOURCE methodology : Application to Ghana

#### J.B. Asafo-Boakye, ABP Consult Limited, Accra, Ghana

  - INTRODUCTION

1.1  Objectives

1.2  SOURCE Methodology

2.0  ORGANISATION OF SURVEY

2.1  Desk studies

2.2  Planning and Preparatory Phase

2.3  Measuring and Monitoring

2.4  Processing of Data

2.5  Suggestions proposed to improve experiment

3.0  COST OF EXPERIMENT

1.0 INTRODUCTION

The Road Maintenance Initiative (RMI) decided at a recent co-ordination meeting that there was the need for a simple, straight forward tool for monitoring the condition of road networks on the scale of the subcontinent.

A general practice, previously was to request assistance from a road expert by filling out simple grid, giving linear percentages for the main 'paved' and 'unpaved' network, in their conditions: good, fair or poor. This method is not scientific or reliable, and thus cannot form the basis for the analysis and assessment of road policies.

Conventional methods used on road networks such as degradation recordings and roughness measurement, give accurate results they thus enable management policies to be developed. They however need considerable resources and time frame, thus cannot be used as tools for monitoring the overall changes in road network conditions throughout the 47 countries in Sub-Saharan Africa.

1.1 Objectives

The objective of the SOURCE is to introduce a simple and straight forward method for assessing road networks. It is also not to be expensive and time consuming, and also easily repeatable.

Accordingly the project was to design, test, develop and validate an instrument for the overall monitoring of road network conditions, which will describe these conditions in terms of functional performance, i.e. the transport-service to the user.

The purpose is to set up a mechanism for cyclic observation of road conversation in the forty-seven countries of Sub-Saharan African, over a three-year measuring cycle.

The method will be based on:

- Standardized extractions of references networks for each country;

- Standardized measurements of normal traffic-flow speeds and;

- The simultaneous monitoring of traffic levels.

The latter two combined parameters will be aggregated into a single microscopic indicator to ascertain the levels of service provided by the main roads in each country. Various other derivative results will also be produced.

1.2 SOURCE Methodology

The methodology adopted was the measuring of normal traffic speeds of a well defined category of vehicles over a well-identified reference network using a standardized method. (Speed is defined as the radio of distance/time).

The experiment is thus to obtain representative travel time for a specific length of travel, in order to obtain speed.

No information exists on travel speeds of vehicles currently, thus the results of the experiment can be deem to form the control results of any future experiment.

The well defined category of vehicles chosen for the study were classified as Light Vehicles. These are:

- Private Vehicles

- Minibuses

- Small Utility Vehicles (Van, Light Van)

- Light Lorries

- Pick-Up Trucks

which could either to 2 or 4 wheel drive. These light vehicles constitute over 80% of the vehicle population in Ghana.

The well identified reference network used for the study is as shown in Fig.1. It is essentially the trunk road network of Ghana. The total length of this network is approximately 6500km, 3600km paved and the remaining 2900km unpaved.

It would be noted that majority of the roads are in the south, closer to the capital of Ghana, Accra.

2.0  ORGANISATION OF SURVEY

At the inception of the assignment the technical team to undertake the assignment was identified and assembled. This team consisted of a Project Co-ordinator and two graduate engineers. The survey was then organized in the following phases:

-Desk Studies;

-Planning Phase & Preparatory Phase;

-Measurement & Monitoring Phase.

2.1  Desk Studies

At the inception of the project the following documents were studied:

- The measurement manual from Source;

- Quality control procedure;

- The classification standard and condition of Trunk roads in Ghana (1998) by the Ghana Highway Authority

- Hotels and Rest Stops in Ghana (1996) by the Ghana Tourist Board

2.1.1  The Measurement Manual from Source

The Measure Manual explains the "floating method" adopted for the study. It also includes:

- Basic principles;

- Definition and concepts;

- The general principle of measuring speed;

- Measurement campaign resources;

- Preparation for the measurement campaign;

- General conditions for making measurements;

- Description of the measurement method of a section; .Safety of Crew and relationships with the uses; .Data input and processing;

- Counter calibration (as an Annex).

All members of the technical team studied this document. Each member came out with:

- their understanding of the assignment;

- information to be covered before and during the measuring campaign;

- issues that required clarification.

- The team discussed the above items and agreed upon what information needed to collect and note all issues that required further clarification for discussion with the ISTED Facilitator.

2.1.2  Quality Control Procedure

The experiment to be undertaken was new. It was this essential to introduce quality control mechanisms to ensure the procedure produces reliable,

usable results. This is necessary as the procedure is to be implemented by different service providers on a year to year and country-to-country basis.

These quality control were to take place primarily during the following three phases:

a) Before the campaign:

To check that the method has been properly understood by each crew, prior to the authorization to begin the series of actual measurements. This is a Preventive step intended to avoid problems at a later stage (which would result in delays, supplementary costs or penalties).

b) During the measurement campaign:

An 'in-field' quality control on the proper implementation of the method by each crew.

c) After the experimentation:

Quality control against the documents to detect after the event any errors in the conduct of the campaign or the processing of the data.

It was therefore necessary to know what was expected before the process was started.

2.1.3 The classification standard and condition of Trunk roads in Ghana (1998} by the Ghana Highway Authority

This document contains the following information on trunk roads in Ghana:

- Classification and referencing of trunk roads in Ghana;

- Length of roads, and clear subdivisions based on junction locations or surface type;

- Surface type;

- Condition maps on the trunk network are also included.

This information was supplemented by discussions with maintenance engineers at headquarters and the regions (by telephone). Additionally, historic traffic levels on road links were obtained from GHA.

2.1.4  Hotels and Rest Stop (1996} by: Ghana Tourist Board

This documents contains the following information on hotels/rest stops:

- Location;

- Star rating;

- Mode of communication;

- Costs of providing services;

2.2 Planning and Preparatory Phase

The experiment to be undertaken was new to the technical team and thus it was necessary to assimilate the method properly before the survey was undertaken. Additionally, the roads to be surveyed are scattered all over the country and thus it was important to plan the work.

The planning and the preparatory phase was thus to ensure the above were undertaken properly.

2.2.1  Preliminary: Program of Works

Based on the information collected during the desk studies, initial schedules for the execution of the works by the 2 teams were worked out.

This was based on:

- Zoning of areas of study;

- The productivity rates established for earlier studies. (180km per day for unpaved roads and 300km per day for paved roads);

- Minimization of intersection connecting hips;

- Allowance of time to cater for any interruption, using the chosen zones

- Subdividing road links into homogeneous sections (in terms traffic and road links);

- Ensuring that both teams completed their works within a similar time frame;

- Possible lodging places;

- Weather considerations;

- Identification of non representative traffic days, e.g. Sundays & Holidays;

- Day to initially undertake practice runs and choice of location;

- Meeting with ISTED expert.

The essence of the above was to:

To partition the time allocated for the study, in order to identify strategies to ensure that work can be completed within the stated time frame. (e.g. If there was the need for processing of data to be started before the end of the field work, and if so how it was to be done);

- Choice of date for training and practice of team;

- Mastering of the floating method;

- Mastering calibration techniques and ensuring the team understood its importance;

- Establishment of equipment & materials to be used in study;

- Recording on the input data sheet.

2.2.2  Training

A days training was undertaken by the team, and it highlighted on the following:

- Assimilation of the floating technique method. (Annex 1.). It is basically the L V speed measurements, systematically combined with L V traffic counts in the opposite direction.

- Calibration of vehicles. (e.g. specifically measured distance by surveyor with trial runs undertaken by vehicles);

- Identification of land marks for start and end of sections;

In order to ensure that both teams understood the assignment, the following steps were taken:

- The entire team did the initial trial runs in each vehicle to be used alternatively. This was to ensure that all parties understood the experiment and also to establish co-ordination between the field crew of each vehicle.

- Identical trials routes were chosen for each group but with a time lag between the two.

This was to check whether there was any misunderstanding between the two groups in terms of measuring. Additionally, if there was no misunderstanding, were the results likely to be consistent.

The teams met after the days training and comments worth noting include the following:

- Different traffic levels;

- Non conformity in filling of data input forms;

- The need for effective co-ordination between driver and engineer;

- Specific elements of the proposed measuring technique.

Most the issues were resolved. However, those that were not resolved were catalogued. These issues that were not resolved formed the basis for the discussion with the ISTED Expert.

2.2.3  Meeting with ISTED Expert

As specified by ISTED, an expert had to meet the team, co-ordinator and crew members. At the meeting the following were discussed:

- explain and comment on practical training method;

- undertake another day practice run to ensure the team had assimilated the process;

- the final program of work;

- quality requirements;

- control system.

A formal agreement was given by ISTED after the submission and acceptance of the following documents:

- The final detailed program of the campaign of measurements of or each of the crew; (Fig.2)

- All the input data sheets and summary reports relevant to the crew's day of training and practice after they have been analyzed;

- a questionnaire duly filled out by each crew leader. This questionnaire was basically answers to specific questions in the measurement manual.

The essence of the above is to ensure quality control, e.g. The team understood and assimilated the method of measurement.

2.2.4  Methods/Equipment to be used in study

##### Equipment

The following equipment was required:

- A standard 4 wheel drive vehicle in good, safe running conditions capable of travelling on all types of roads. It should have a standard kilometric recorder with a daiIy totaling meter (with reset button) which could display in hundred of meters.

- Stop clocks (2 per operator);

- Chipboards (for easy filling of input data sheet as the vehicle move)

- Note books (for recording of information);

- Stationary (e.g. Pens, pencils, erasers, highlighters etc.;

- Wellington boots;

- Rain coat;

- Maps;

- Supplementary facilities, e.g. Phone cards

###### Human Resources

The experiment requires the presence of two people on board each measuring vehicle:

- 1 experienced driver (with experience on all types of roads) who will assist the operator by providing him with information in the course of the experiment (kilometric recordings, etc.) and able to assimilate and accurately apply, under the operator's control, the vehicle driving rules given by the method.

- 1 operator who will be a senior technician or an engineer

-with a good knowledge of the network

-able to assess and analyze all types of situations including those requiring certain choices given in the method ( concerning when to stop measurements, driving through villages, vehicle convoys, fast LVs, etc.) and able to apply the relevant rules given in the Measurement Manual.

-displaying considerable rigor, organizational qualities, a spirit of initiative and decision-making ability .

Each crew was to remain unchanged throughout the experiment, including the preparatory phase.

The experiment also requires co-ordination tasks. The role of co-ordinator was not be entrusted to a crew member.

2.3  Measuring and Monitoring

On certification of the staff and approval of the daiIy program for each crew, by the ISTED expert, the measuring phase was undertaken.

To ensure the in-house quality was to the required standard the following were discussed thoroughly before the process was started:

- That if the work program required any modification, (e.g. Rain, road closure, etc.) the project co-ordinator should be informed accordingly. He was also to inform the ISTED office as soon as possible;

- At the end of each day, the individual crews were to contact the Project Co-ordinator;

- Measurements for counter calibrations should be undertaken in major towns, if possible.

- Counter calibration should be undertaken immediately there is a change in vehicles condition;

- Counter calibration should be checked on visits to Accra, even if the required 2 week deadline is not met;

- Forms, if needed to be re-written, should be undertaken after completion of the days activities;

- Forms would be collected by or submitted at period interval, in order to allowing processing of data to be started as early as practicable;

- Any modification that was likely to enhance the Operational Manual should be noted as soon as possible;

- The project co-ordinator would make unscheduled visits to check on their work and discuss any problems or modification required.

ISTED in-house quality control measures undertaken included:

- Unannounced series of cross-check measurements of speeds and traffic, performed according to the same method by a control team on the same road link, on the same day and at the same times as the measuring crew;

- The unannounced visit of a passive observer who will join the measuring crew for one day to make sure its work methods are correct.

In both cases, at the end of the day, after analyzing the documents, an appraisal was to be made together with the crew, resulting as appropriate, in:

a) simple advice to correct and improve the crews' working practices. This advice was to be outlined in a document countersigned by the crew leader. He will immediately be take a copy with another copy will be forwarded to the co-ordinator.

b) in the event of non-compliance with the method or serious implementing errors affecting that day's work, it was obligatory to repeat all or any of the day's measurements (if they prove unusable).

2.3.1 In field campaign

The infield campaign of the two team started simultaneously. The work undertaken was basically the filling of the Data Input sheet. (Shown as Fig. 3)

The Data Input sheets can be divided into the following three sections:

- General information (Section 1 )

- Measurement during actual experiment (Section 2)

- Factors that affect the experiment (Section 3)

The general information section, provided an insight into:

- the road section or subsection to be undertaken,

- classification of the road;

- characteristic of road ;

- data and time of operation;

- start and end sections of subsection;

- vehicle used for the assignment.

This information was filled before the start of the assignment, with the exception of the information available only at end of section. This was the time for completion of the section and the land make used for the end of section.

Measurements and factors affecting the experiment were recorded during the campaign. The measurements obtained, as started earlier, where representative distances and time in order to obtained speed. Factors such as weather, built up areas, congestions etc. were noted in order to neutralize sections that may have an effect on the overall measurements.

All the end of each days activity (where possible) the team leaders contacted the Team Co-ordinator and discussed the progress of work. All proposed modifications to the work plan were discussed and the information was faxed to ISTED.

Completed field data was obtained by the Team Co-ordinator by:

- Courier services at specified times;

- On unscheduled visits by the Co-ordinator

2.4  Processing of Data

The processing of data was made up of the following:

- extraction of times, distances and traffic volumes from data input sheets for

- subsection/sections;

- computation of distance correction coefficient from calibration sheets;

- Filling of summary sheets for each road link.

2.4.1 Extraction of times and distances from data input sheet

for subsections/section

The major focus of the experiment, as stated in chapter 1.1 is to obtain speed.

Thus the essence of processing the data is obtain representative travel distances and time in order to compute speed.

A sequence starts at an "At stop" and ends at a "Measurement suspension or interruption". During measurement of a subsection there may be one or more of such sequences.

The sum of the distances and time of each sequence provides information for the computation of the average speed of travel for a subsection/section.

However, the SOURCE experiment, is not to obtain average speeds but representative speed. Thus the results of each sequence is analyzed and all non representative information deleted, e.g. suspect subsection caused by built-up areas, congestions, convoy etc.

After the analysis of the data the information is subdivided into workable, unworkable and impossible sections.

Workable sections refer to representative portions, unworkable section refer to areas affected by perturbations whilst impossible sections refer to areas that measurements have been interrupted (e.g. Distance measurements, but with no corresponding time measurement).

Traffic volumes obtained are treated likewise.

The net of each subsection is obtained by the summation of the representative

data for each sequence. Fig. 4 illustrates how the information is presented.

This data extraction was undertaken initially by the project co-ordinator and two in-house assistant engineer, who had been adequately trained by the co-ordinator. On the completion of the field work, the field team leaders also got involved in the extraction of data;

To ensure the required quality control, all work done by one team was checked by another team. The project co-ordinator however undertook the final check.

2.4.2  Distance correction coefficient from Calibration Sheet

The distance indicated by a vehicle odometer may not be accurate. Thus to base distance measurements entirely on them could lead to inherent errors in the results. To prevent this, a counter calibration was undertaking at the start of the project and at any point that the vehicle was modified or likely to be affected e.g. changing of tyres, specified two week period, etc.

This was done by filling form attached (fig. 5). The result obtained is the ratio of the vehicle distance to an actually measured distance. This is ensuring repeatability of the results.

The counter calibration factors obtained were in the neighbour of 0.989. This implies that for a distance 10km recorded by the vehicles, the actual distance recorded is actually 9.89km. Thus if the vehicles odometer had formed the basis for the measurements speed obtained would have been lower by 1.1 %

2.4.3  Filling of Summary Sheet

A summary sheet is filled for each road link. Information on each subsection of a road link is keyed in and the results for each subsection are obtained. Additionally information on the overall road link is provided.

This is an automated process based on a software (Excel spread sheet based) provided by ISTED.

The results of the study are as shown in Annex 2. As stated earlier, these would form the basis for the future evaluation of the network.

2.5.0  Suggestions proposed to improve experiment

During the implementation of the experiment a number of measures in order to improve the experiment where noted. They are classified in the following phases

- Experimentation Phase

- Data Input Phase

- Data Analysis Phase

- They were communicated to ISTED.

2.5.1  Experimentation Phase

2.5.1.1 Personal

The experiment is basically undertaken by a driver and an Engineer/Technician. The two had to work effectively as a team for the experiment to be successful. It was however observed on heavily traffic roads, where traffic roads, where traffic conditions varying rapidly, it was difficult for the Engineer, even though he was being assisted by the driver to effectively monitor the flow of on-coming vehicles. It is therefore suggested that before the experiment is carried out, roads links should be classified into groups of high, medium and low traffic, based on historic data if available.

On roads which fall into the high category, it would then be advisable to introduce a third parson whose responsibility would be dedicated to the counting of on-coming traffic.

2.5.1.2  Equipment

For safety reasons, it is advisable to ensure maximum concentration of the driver on busy roads. It is therefore suggested that vehicles to be used on such experiments should be equipped with topometer (odometers). This would also ensure more accurate readings as the Engineer/Technician would be able to read the distance more readily, especially if the topometre is mounted directly in front of him or her.

2.5.1.3  Measurements

The 'stop' and 'free stop' durations have been specified as 15 minutes.

This sometimes is a major inconvenience on low volume traffic roads. It is therefore suggested that on low traffic roads, the duration of the 'at stop' period may be reduced with a corresponding increase to the 'free speed' duration, provided it does not complicated the experimental procedure.

Alternatively, another means of measurement should be adopted to roads in the extremely low traffic.

2.5.2  Data Input Phase

The following are suggested to improve the data input phase.

i. Introduction of a separate row to for neutral vehicles instead of circling the line dividing the rows of hares and tortoise.

ii. A clear cut convention adopted for suspension of measurement e.g. Timer is fixed as counter is allowed to move (creating an impossible speed measurement).

2.5.3  Data Analysis Phase

It would be advisable to give the analysis team an insight to the compilation aspects of the projects, such that if any formula needs to be modified it can be rapidly undertaken. Also, it is suggested that the data input sheet should be modified to include effective net intervals as modified on the analysis sheet attached, which was used in the analysis.

ISTED responded to these suggestions and have accordingly modified their operational manual to enhance the performance of the experiments.

3.0  COST OF EXPERIMENT

The cost of undertaken the assignment for the 6,500 km of roads was US$28, 780.77. A breakdown is herewith attached.

The assignment was completed in 50 days, with the actual experiment and analysis of data in 38 days. The maximum productivity rate achieved was 350km per day for paved roads and 240km per day for unpaved roads.

For comparison purposes only, a roughness measurement assignment, using the same team, is likely to take a period of at least 100 days (as against the 38 days). The results to be obtained would only provide information on the pavement characteristic, without any information on traffic levels.

The equipment required for such a measurements is specialized and more expensive. Its utilization would also be very limited due to its specialist nature.

A surface degradation measurement would even require more time for collection of field data and analysis. It also requires much more resources.

| ## TABLE 1 : COST OF EXECUTION OF WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N° | Description | Unit | Quantity | Unit Price | Total Amount |
| 1 | 4-wheel drive vehicle All-inclusive expect petrol With driver, per-diem including Insurance, maintenance, repairs And unlimited kilometerage | Day | 64 | 110 | 7,040.00 |
| 2 | Petrol | Day | 64 | 16.67 | 1,066.67 |
| 3 | Equipment | Lump sum | 1 | 350 | 350 |
| 4 | Personnel, expertise cost |  |  |  |  |
| 4a | Project coordinator |  |  |  |  |
|  | Desk studies | Day | 4 | 159.09 | 636.36 |
|  | Planning and preparatory phases | Day | 4 | 159.09 | 636.36 |
|  | Experimentation phases | Day | 6 | 159.09 | 954.55 |
|  | Data processing phases | Day | 12 | 159.09 | 1,909.09 |
|  | Report writing phases | Day | 2 | 159.09 | 318.18 |
| 4b | Engineer |  |  |  |  |
|  | Desk studies | Day | 4 | 113.64 | 454.56 |
|  | Planning and preparatory phases | Day | 10 | 113.64 | 1,136.36 |
|  | Experimentation phases | Day | 52 | 113.64 | 5,909.09 |
|  | Data processing phases | Day | 14 | 113.64 | 1,590.91 |
|  | Report writing phases | Day | 1 | 113.64 | 113.64 |
| 4c | Assistant Engineer |  |  |  |  |
|  | Data processing phases | Day | 14 | 97.5 | 1,365.00 |
| 5 | Daily personnel charge (per diem) | Day | 64 | 75 | 4,800.00 |
| 6 | Supply of reports (cf. Terms of Reference) | Lump sum | 1 | 300 | 300.00 |
| 7 | Secretarial Services | Lump sum | 1 | 200 | 200 |
|  | ### Grand total |  |  |  | 28,780.77 |