SOURCE (Standard Overall Ultralite Road Care Estimate)

Outcontracting by tender for a SOURCE Campaign : Procurement Models ### Terms of reference

### The following model is given for guidance only. It conforms in shape with the relevant procurement models from the World Bank. Anyway the compliance with the following document will bind neither the World Bank nor another partner for non-objection if applicable.

### TERMS

### OF

### REFERENCE

A. BACKGROUND

[TO BE COMPLETED according to the local context]

B. OBJECTIVES

The typical objective of a SOURCE measurement campaign is to provide standardized key data for the overall monitoring of the situation of the core road network, to be described in terms of functional performance, i.e. the transport-service to the user.

The standard method is based on:

- the definition of the “reference network” to be measured (done beforehand under the responsibility of the Client)

- the standardized measurements of current traffic-flow speeds and

- the simultaneous measurements of traffic levels.

The latter two combined parameters will be aggregated into a simple set of macroscopic indicator so to ascertain the level of service provided by the main roads. Various other derivative results are made possible by this method.

Such a countrywide measurement campaign is liable to be repeated later in order to follow up progress, the envisaged frequency being each [INSERT: “three” OR “one”] years.

C. SCOPE OF THE SERVICES

# C1. Description of the measurement campaign

# Place and date

The operation will take place in [INSERT: Country’s name] in [INSERT: Month(s) AND Year] (performing measurements during the dry season or a comparable period only is mandatory).

# Scope of the operation

The campaign will cover a network of roads with an overall approximate length of [INSERT: Length of network] km.

[THIS INDENT IS OPTIONAL:]

This reference network has first been pre-determined, according to the extraction specifications under the SOURCE method, which apply uniformly for all the Sub-Saharan countries. This special sample network, which includes virtually all the main roads in [INSERT: Country’s name] outside urban areas, has been completed by additional roadlinks so to meet exactly the Client’s monitoring needs. The resulting network to be measured is known as the Reference Network.

This reference network breaks down into some [INSERT: Paved length] km of paved roads and [INSERT: Unpaved length] km of unpaved roads. It includes roads throughout the national territory, with the highest concentration in the most populated areas.

Throughout this document, the terms “road link” or “link” will mean either a portion of road (paved or unpaved) in the reference network included between two successive intersections with other roads in this network or a portion of road between and end of this network and its first intersection with another road in the network. Based on the network map provided by the Client, the service provider will finalize the division into links and assign the appropriate link codes.

All the field measurements will be performed for each road link duly divided into homogeneous sections to be determined by the service provider (according to the traffic flows, the alignment, the intersections, the environment, the pavement type, etc.). Details on the method for division into sections are given in the SOURCE Handbook.

# Types of measurements to be made

###### Summarized protocol

The measuring method to be used – the “floating” method – consists in integrating and ordinary vehicle into the traffic flow (the measuring vehicle, known as the floating vehicle). This vehicle will alternately follow a fast vehicle (by which it has been overtaken) and a slow vehicle (with which it has caught up), subject to a few adjustments. The actual speed of the floating vehicle over the travel path is then used as a result, after various corrections.

A single run, in a single way of travel, will usually be enough.

Simultaneously the floating vehicle keeps a count of traffic travelling in the opposite way (“apparent” reverse traffic). With various corrections, this gives both a series of speeds and a series of traffic counts.

For both speed measuring and traffic counting, only light vehicles (LVs) are considered. This "LV" category is precisely defined in the SOURCE Handbook.

This general “floating” measuring technique is applied the same way to unpaved and paved roads, even multi-lane roads, and whatever their traffic level (except in special cases).

###### Precautions

The speed to be measured is the “free” speed of LVs over a significant length – their spontaneous speed when not impended by any particular disturbance. At the same time, the traffic level encountered must remain representative. This requires certain precautions such as the following:

- All measurements must be made on business days and in business hours,

- Any interfering factors liable to adversely affect and impede speeds or flows during measuring operations are to be avoided or circumvented according to strict rules.

Lasting reasons for reducing speeds, due to the condition of the road itself (any defects in an ageing, inadequately maintained roadway) are not in themselves considered as disturbances. Through their influence on speeds, this poor condition of the road is precisely what the SOURCE operation aims to determine.

# Detailed behavior of the floating vehicle

The method mobilizes a single measuring vehicle (the floating vehicle – FV -) with a driver and an operator on board, known as a measurement crew.

As a general rule, the FV operates its “measurement run” at one go over a predetermined section. Speed measuring is performed in the travelling direction of the FV (two distinct measurements can be made on the same section, one in each direction of travel; but as a rule, only one of these measurements is required and the direction does not matter).

The FV must always follow another LV and constantly adjust its own speed accordingly.

It will frequently change followed LVs, choosing another LV during an overtaking maneuver between the followed LV and another LV travelling the same way (either because the followed LV is overtaking another slower LV – a “tortoise” - or because the followed LV and the FV are being overtaken by another fast LV – a “hare” -.

But not all overtaking gives rise to a change in the followed LV. A specific rule stipulates the case in which a change must occur (roughly to alternate tortoises and hares). Another exception is that the same LV must not be followed for too long at a time (roughly 15 mn), so not to bias the measurement. Additional rules exist for non-traffic situations (where the FV is allowed to run “solo” for limited times).

However, various situations may form exceptions:

- If the followed LV stops or turns off onto another road, the FV stops and suspends the ongoing measurement until another LV arrives,

- Measuring should be suspended for a “skip” while going on running whenever a "disturbing event" occurs in the traffic, that is liable to affect a very short length (e.g. a cattle market along the roadway), or to be continuous (e.g. driving through a built-up area),

- At the contrary the FV stops and waits at a “standstill” whenever a "disturbing event" occurs in the traffic, that is liable to be very momentary (e.g. a heavy shower, etc.),

- In a few cases of peri-urban sections, peak hours must be avoided,

- Some unexpected events, which interfere considerably with measuring, may affect its quality and oblige to delay, to postpone or to repeat the measurement.

During the measuring period, the operator uses manual data pages (model to be provided) on which he records various information concerning the followed LVs, the elapsed time, the kilometric recordings or various events. Each current line on the data page will correspond to a followed LV. At the same time as the speed measurements are made in one direction of traffic flow, the LVs passing in the opposite direction will merely be counted on the same data page. Hence the relevant traffic will easily be reconstructed.

After each measured section on the field, the operator close the set of input pages by adding the section data sheet (model to be provided), where he recaps and aggregates data at section level.

# Data processing

The information recorded section by section on the section data sheets enables the consultant, at his office, to transfer key data to the special SOURCE calculation workbook, which will perform some checks then directly calculate values at link level and extract the desired indicators (a blank version of this special workbook, including Help facilities and examples, to be provided).

C2. Requirements for performance of work and resources

Field campaign organization

The combined speed/traffic measurements must be made over the entire reference network. To shorten the operation time, the field campaign is liable to be shared between [INSERT: suggested number of crews] crews each with a vehicle, up to the provider to propose another solution.

The travel path for each crew, on its part of the network, is carefully programmed in advance. This program is determined so that inter-section connecting trips made necessary by the geography of the road network are preferably made on road links or sections of the reference network (and not on other roads) while prioritizing the more heavily trafficked ones. Such situations will be used to make two distinct measurements on the relevant sections, one on the way out, the other on the way back.

But on the whole, the number of inter-section connecting trips must be kept to a minimum when planning the operation.

No measurement can be made when traffic flow is significantly disturbed (heavy rain, traffic congestion… See the SOURCE Handbook for full criteria). Thus daily timetables must provide for sufficient flexibility margins. More, the program must include extra days for any measurement runs that could have to be postponed or repeated for those reasons. The recommended number of extra days is 15% of the total number of days estimated for the measuring campaign (note that business days only are authorized for standard measurements, due to non-typical traffic situations during holidays).

# Productivity

Previous SOURCE campaigns performed in comparable countries have proved the following figures: the net productivity of a measuring crew was over 150 km of network measured per working day for 200 km covered (due to some unproductive connecting trips, etc.).

These orders of magnitude are given as a guide.

# Material resources

The measurements will be made using a standard 4-wheel drive vehicle in good, safe running condition, capable of travelling on all types of roads. This vehicle does not require any special equipment. However, it must be ensured that the standard kilometric recorder is duly fitted with a kilometric recorder with reset button and a display in hundreds of meters.

Regularly during the campaign of measurements, the accuracy of the kilometric recorder must be checked using a method specified in the SOURCE Handbook. This will require calibration test to be repeated each fortnight over a single 2,000-m standard section, the curvilinear length of which (with respect to the road axis) has been measured by a professional surveyor once and for all (under the responsibility of the provider). This standard section is known as the permanent calibrator. A manual calibration sheet (model to be provided) will receive the records of each successive calibration test.

It is not mandatory for the vehicle to be fitted with a topometer (although this would make easier the operators’ work and improve their comfort while measuring) or to have an on-board microcomputer, a radio transmitter or any other special equipment.

On electronic request, the SOURCE Handbook together with models of calibration sheet, section data page and sheet, the blank SOURCE calculation workbook and the plan of the reference network will be provided in the computerized form (as Word and Excel files), free-on-charge, to potential tenderers. The electronic address figures in the Data Sheet (attached to the Request for Proposals).

# Human resources

The full measurement campaign requires coordination tasks. The campaign coordinator is supposed: (i) to conduct the preparatory phase, (ii) to supervise the field campaign and (iii) ensure internal quality controls, then to (iv) perform data processing and finalization. The coordinator should be the current representative of the provider towards the Client (and towards the Controller if any) regarding the execution of the contract. The role of campaign coordinator requires a civil engineer, transport economist or equivalent and must not be held by a crew leader.

The field operation requires the presence of two people on board each measuring vehicle:

- 1 experienced driver (with experience on all types of roads) who will assist the operator by providing him with information in the course of the measurement run (kilometric recordings, etc.) and must be able to assimilate and accurately apply, under the operator's control, the vehicle driving rules given by the method.

- 1 operator in position of “crew leader”, who will be a senior technician or an engineer

- with a good knowledge of the network,

- able to assess and analyze all types of situations including those requiring certain choices given in the method (concerning when to stop measurements, driving through villages, vehicle convoys, fast LVs, etc.), and able to apply the relevant rules given in the SOURCE Handbook.

- displaying considerable rigor, organizational qualities, a spirit of initiative and decision-making ability.

Each crew must remain unchanged throughout the campaign, including the preparatory phase (see next paragraph).

# Preparatory phase

[THIS INDENT IS OPTIONAL:]

As soon as the contract is awarded, an expert will be made available by the Client during [INSERT: “half a day” OR “a full day” OR “a few days”] for meeting with the campaign coordinator and the crew leaders, explaining and commenting on the practical measuring method, discussing the drawing up of the final program and giving additional explanations on quality requirements and the control procedure.

Each crew leader must receive a SOURCE Handbook, which they must fully assimilate before beginning the field work. The Consultant will be responsible for organizing the crews’ prior training and needed practice, preferably jointly for all crews (in order to confront and harmonize their practical routines).

Before the start of the campaign of measurements, it is compulsory for each crew to accomplish a full day of final training on any diversified network sample so to ensure it has mastered the method (including recording the input data sheets). The outputs will be submitted to the Client as mentioned below.

The campaign can only begin after formal agreement has been given by the Client on the basis of documents previously handed over by the service provider, which will consist of the following:

- The detailed program of the campaign of measurements for each of the crews, including all information on the crews (identity and task of crew members, description and registration of the vehicle, etc.)

- The original calibration sheets, all the original section data pages and sheets and the filled-out calculation workbook relevant to the crews' day of final training.

- [THIS INDENT OPTIONAL] A special questionnaire related to the implementation of the SOURCE method (blank version to be provided), duly filled out by each crew leader.

- These last items will ensure that the methods have been properly assimilated, in order to accredit the crews for the performance of this measurement campaign, as mentioned further on at Chapter E. “Quality Control Procedure”. Please note that this accrediting limits to the relevant campaign: a crew previously accredited for a prior campaign would not spare this mandatory day of final training and could be refused the new accreditation if necessary.

- The preparatory phase fully submits to the quality control procedure (for the relevant quality control specifications, see the section. E, "Quality Control Procedure” below).

- It is the responsibility of the Client to obtain and supply the service provider with the attestations and permits to enable the crews to operate on the roads in [INSERT: Country’s name], if needed.

# D. REPORTS

- The items to be submitted after the full campaign and data processing comprise:

- All the originals of the manual section data pages and sheets (according to the model that will be provided), completely and legibly filled out; no copies are permissible.

- All the original of the manual calibration sheets corresponding to the successive calibration tests (according to the model that will be provided), completely and legibly filled out; no copies are permissible.

- The special SOURCE calculation workbook, duly filled out (using the blank version of this Excel spreadsheet, as provided)

- The actual measurement campaign program, as actually performed (with details per day) with a copy of eventual correspondences relevant to any changes from the original program (see the section E, "Quality Control Procedure" below).

- A brief report on the performance of the campaign, which will report any significant problems encountered and will highlight any specific causes of tainted measurements and any causes of delays or interference in its progress.

#### E. QUALITY CONTROL PROCEDURE

- The provider will be bound to an obligation of result for the end quality of its measurements, except in cases of force majeure duly acknowledged by the Client.

In addition to his own arrangements for internal quality control, the provider will have to submit the special SOURCE procedure of external quality control, to be implemented by [INSERT: “a third party –firm or individual consultant- to be separately appointed by the Client” OR “an agent of the Client itself to be chosen”] in due time and known as The Controller.

This special quality control procedure will apply though three different phases as follows (also see the SOURCE Handbook: especially for specific prescriptions pp. 21, 23-25 and 109; anyway this procedure intends to verify the full and professional compliance with all methods prescriptions from the SOURCE Handbook).

- Preparatory phase

As already mentioned, at the end of the preparatory phase, the consultant will submit different documents to the Client’s prior agreement. This agreement will express both the approval of the detailed campaign program and the accreditation of the crews by the Client. If justified, the Client denies his agreement in the following cases:

- Case 1: If the submitted detailed campaign program violates SOURCE specifications, which would demand for due correction before its agreement,

- Case 2: If the submitted data pages and sheets [INSERT IF APPLICABLE: and the questionnaires] prove insufficient mastering of the SOURCE field-measurement method by one crew at least, which would demand for the provider to perfect the practice of the concerned crew(s) and to re-organize a final training day for them then to submit the corresponding new materials,

- Case 3: If the submitted calculation workbook proves insufficient mastering of the SOURCE data processing method by the campaign’s coordinator, which would demand for him to correct and submit the corrected version instead.

##### Anyway the campaign is not authorized to start until this formal prior agreement by the Client. The rejection cases listed hereabove do not give rise to specific financial penalties, but cost overruns due to repeated tasks are to be borne by the provider. Any delay over five days before the Client’s response(s) (whether positive or negative) would prolong the contractual deadline accordingly.

- During the field-measurement campaign

Compliance with the campaign program

Each measurement crew will be obliged to comply strictly with the approved measurement campaign program. However, in the event of force majeure or serious constraint, changes are possible providing the following procedure is met:

- Any unforeseeable change forced by a field contingency must be notified to the Client within 48 hours to be regularized a posteriori. The Client will decide to regularize or not according to the unpredictability and the reason itself.

- Any suggestion of change in the campaign program, made necessary or indispensable, must be submitted a priori to the Client as a proposal, together with the relevant reasons, at least 48 hours in advance. The Client has the right to reject the proposal before 2 working days if it does not consider such change necessary or expedient.

The detailed campaign program, as approved by the client before the start of the field campaign and duly updated according to the eventual regularizations and the non-rejected change proposals, is known as “the approved campaign program”.

Any case of non-compliance with the approved program would prevent proper exercise of in-field quality control and might cause the logistical costs of this control to be wasted Except of justified cases to be pleaded in due time and acknowledged by the Client, this would constitute serious negligence on the part of the provider, subject to a specific financial penalty, known as “penalty for non-compliance with the campaign program”.

In-field quality controls

- Without notice, the Client’s Controller is liable to perform in-field quality controls, namely:

- Unannounced cross-check measurements by a control crew, on the same road sections and times as a measurement crew,

- Unannounced visit(s) of a passive observer who will join the measuring crew for one day so to observe its measurement process and consult the available documents.

Any formal outputs of such controls will be expressed to the provider by the Client himself. This could result in the followings:

- Case 1: An absence of remark,

- Case 2: The Controller’s simple advice in order to improve the crews' working practices,

- If non-compliance with the method or serious implementing errors are detected:

- Case 3: Either an express warning that the concerned measurement runs (and eventually all previous measurement runs) are doubtful and liable to be rejected at the time of final control (see para c below). Thus the provider should consider the opportunity of repeating the concerned measurement runs without further ado,

- Case 4: Or a notice of immediate rejection of the affected runs, which would oblige the provider to repeat them (at his own cost); together with an express warning against the same defaults liable to affect previous runs.

These two cases do not give rise to specific financial penalties, but cost overruns due to eventual repeated runs are to be borne by the provider. However changes in the campaign program due to such unexpected measurement repeating require the usual prior notice to the Client.

- Case 5: a statement of non-compliance with the program if the control crew or the Controller’s observer is unable to meet the measuring crew at the location and time mentioned in the approved campaign program. Except of justified cases to be pleaded in due time and acknowledged by the Client, thus the provider will incur the corresponding penalty.

Even if the result of such a control is favorable, this would not entail any definite validation of the corresponding measurements by the Client.

- Final control

After the provider submits the final report and accompanying deliverables to the Client, the Controller will perform a series of desk control against these pieces (sampling tests at first, then full tests if needed). Any formal outputs of such controls will be expressed to the provider by the Client himself. This could result in the followings:

- Case 1: The full acceptance of the services

- Case 2: The rejection of pieces that do not meet the basic requirements (e.g. non-original data pages or sheets, etc.), which obliges the provider to substitute for the appropriate pieces,

- Case 3: The rejection of the calculation workbook if it proves incompletely or incorrectly filled-in (data transfer or data input errors, etc.), which obliges the provider to correct and substitute for the corrected version,

- Case 4: The justifiable rejection of measurement runs to be designated. Note that refusing a single measurement run entails joint rejection of the calculation workbook. Such a rejection is essential in the following cases that inevitably affect the quality of results:

- Gaps in calibration coverage, if some measurement runs are not covered by a calibration test according to the relevant validity rules: thus all the measurement runs performed by the concerned crew during this gap must be rejected, provided this gap is considered “non-reparable” (see further on); anyway this will entail a statement of non-compliance with the approved campaign program.

- Time overlapping, if the confrontation of the records shows that several operations (runs or tests) by the same crew apparently overlap in time: all of them will thus be rejected due to manifest impossibility; anyway this will entail a statement of non-compliance with the approved campaign program.

- Other cases of obvious inconsistencies, impossible data, incomplete data, illegible data, evidence of rigged data: e.g. absurd speeds, etc, provided the corresponding data are considered “unusable” (see further on).

- Other cases of obvious field-method violations: specially forbidden speeds (over 90 km/h), negligence ratios exceeding 10%, Sunday or holiday traffic measurements, etc, but also errors in type of LV considered, maximum time following the same LV, runs operated despite of obvious traffic disturbances perfectly proved  (e.g. roads through built-up areas), undeniable errors of judgment in neutralizing or not doubtful subsections, etc, provided the corresponding data are considered “unusable” (see further on).

- Case 5: By exception to the previous case 4, the direct correction by the Controller himself in such cases of calibration faults considered “desk-reparable” or cases of faulty runs considered “usable” runs (see further on). Thus the Controller will directly correct the affected data throughout the input pages or sheets as well as correct the calculation workbook accordingly.

- Case 6: By another exception to the case 4, the correction by the Controller himself in some cases of calibration faults considered “field-test-reparable” (see further on). Here the Controller will enter the appropriate corrections too, but only after the Consultant has provided him with the results of a specific counter-calibration program. This program will be prescribed by a special order of service for counter-calibration.

So to distinguish between rejection and xxx corrections (under cases 4, 5 or 6 above), the following rules apply:

- Rejecting a calibration test, as well as detecting gaps in calibration coverage, will usually result in the automatic rejection of all the measurement runs that were operated during the corresponding period. But the Controller may consider the problem “reparable” and the runs “usable” as a consequence, provided the missing or faulty calibration correction coefficient can easily be rebuilt without risk of error, by instance if:

- this is just a paper error without field physical error (e.g. data transfer error, calculation error, etc.), provided some reliable data permit to substitute for the correct data easily,

- or the reliable values of the calibration correction coefficient for the relevant vehicle happen to be strictly constant,

- or the variation of this coefficient, as shown through these reliable data, keeps minor and even.

In  the above three cases, the problem is considered “desk-reparable”, as correcting does not require to return to the field. These “desk-reparable” calibration faults fall under the case 5 above.

- The Controller may also declare some other calibration faults reparable, if he considers, according to the available pieces, that a restricted program of counter-calibration tests will permit to rebuild for certain the faulty calibration correction coefficient (just by re-measuring, after proper calibration, the sole lengths for a light sample of the affected sections). This low-cost procedure will avoid rejecting and having to repeat the corresponding measurement runs.

These “field-test-reparable” calibration faults fall under the case 6 above.

- It is the same with the measurement runs that are affected by direct errors: the Controller may consider them “usable”, if ever:

- this is just a paper error without field physical error (e.g. data transfer error, calculation error, etc.), provided some reliable data permit to substitute for the correct data easily,

- or the impact of the error on the different values transferred to the calculation workbook can easily be proved nil or negligible (towards the various SOURCE thresholds and other quality criteria).

These runs affected by direct errors but considered “usable” will not be rejected: they fall under the case 5 above equally.

The different cases defined above entails the additional following consequences:

- The above two cases 2 and 3 do not give rise to specific financial penalties. The final control will just be adjourned at the provider’s fault until due delivery.

- Nor do the faulty calibration errors considered non-reparable give rise to specific financial penalty, but the corresponding affected runs are automatically considered unusable. Each of them gives rise to a specific financial penalty known as “measurement non-quality penalty”. By the same time it is subject to rejection.

- Each faulty measurement runs affected by direct errors, whether considered “usable” under the above case 5 or not, gives rise to the above-mentioned financial penalty for “measurement non-quality”. Nevertheless if they prove usable, they are not rejected nor subject to repeating.

- All the runs that are affected by direct or indirect errors and considered unusable oblige the provider to repeat the corresponding measurements (at his own cost). Such measurement remakes submit to the same specifications and requirements, including the full quality control procedure. Repeating the unusable runs is a contractual obligation and does not exempt the service provider from the corresponding penalty.

- In case of reparable calibration fault (whether desk-reparable or field-test-reparable), the corresponding affected runs (unless they are affected by their own direct errors) are considered usable. As a result they are not rejected nor subject to repeating or to the penalty for “measurement non-quality”. But they give rise to a specific substitutive penalty, known as “penalty for reparable calibration fault”.

- In case of field-test-reparable calibration fault, the Controller will establish the counter-calibration program and attached specifications, to be prescribed to the provider by service order. These counter-calibrations tests, prescribed so to repair previous calibration errors, will submit the same quality requirements as the initial tests. Performing such counter-tests on request, at his own costs, is a contractual obligation and does not exempt the service provider from the above-mentioned penalty.

##### The regular time for final quality control figures in the Data Sheet (attached to the Request for Proposals). At the first delivery of the final report, any delay in the Client’s response (whether it is full acceptance or not) over this regular time must be duly announced by advance and will prolong the contractual time for completion of services accordingly (this prolongation will not exceed 25% of the initial time and will not be reiterated).  Later at the eventual partial delivery due to rejections, the relevant time for partial control will be calculated from this regular time in proportion to the gross lengths of the runs to be controlled, subject to a minimum of five days, not liable to prolongation.

At the end of this stage of final control, the provider (after repeating some measurements until satisfaction if necessary) will have fully met the quality requirements, except case of termination of the contract (see further on section G).

#### F. FINANCIAL PENALTIES UNDER QUALITY CONTROL

The penalties, applicable in the event of non-compliance with the general and specific quality requirements, are as follows:

- Penalty for measurement non-quality

This penalty applies at the time of the final control, as stated above. Its calculation formula is as follows, for each relevant measurement run (in case of direct error, whether usable or not; or in case of non-reparable calibration fault that affects this run):

A being the full contract amount,  L the cumulated gross length of all the measurement runs of the entire campaign as appearing through the calculation workbook at its first presentation, and  the gross length of the considered faulty run, the corresponding penalty P will amount to:

P = 0.5 x (  / L ) x A

- Penalty for reparable calibration fault

This penalty applies at the time of the final control, as stated above. Its calculation formula is as follows, for each case of faulty calibration test or gap in calibration coverage (provided the case is considered reparable).

A being the full contract amount and N the total number of calibration tests for all crews during the full campaign as appearing in the approved program, the corresponding penalty P will amount to:

P = 0.2 x ( 1 / N ) x A

Again the non-reparable cases will not give rise to specific financial penalties.

- Penalty for non-compliance with the campaign program

The penalty for non-compliance with the campaign program will amount to 10% (ten percent) of the total contract value, to be applied to each successive case. Penalties of this type are not liable to cumulate over 30% (thirty percent) of the total contract.

- Penalty for delayed completion of service

The contractual completion time (as finally stated in the contract) will run from the signature of the contract to the completion of the services. Here the completion of the services means the full acceptance by the Client after the final quality control has been performed and after any eventual repeats of measurements have been made as required and, if necessary, after the corresponding documents have successfully submitted the same quality control procedure.

This completion time will be automatically prolonged in some cases of delay in Client’s responses as specified above. The calculation formula for this penalty is given herebelow.

A being the full contract amount,  T the entire time for completion in calendar days (after eventual prolongation as said above), the penalty for a calendar day of delay in completing the services will amount to:

P = 0.2 x (1/T ) x A

In case of amicable termination of the contract in pursuance of section G herebelow, the eventual delay in completion subject to this penalty will be calculated by substituting the date of the termination request for the date of full acceptance of the services, all other things being equal.

#### G. AMICABLE TERMINATION OF THE CONTRACT UNDER QUALITY CONTROL

The following three particular situations will allow either the Client or the provider to choose to state the provider’s failure and demand the amicable termination of the contract, while the other part cannot oppose:

- at any time, if the Client has asserted 3 (three) successive cases of non-compliance with the approved campaign program, that lead to the corresponding penalties,

- if at the end of the quality control following the first presentation of the final report, more than 25% (twenty five percent() in cumulated gross length of all the presented runs must be repeated,

- or if at the end of the quality control following the first presentation of the final report, immediate performance of all the required repeated measurement proves inconceivable with regard to both the date and the SOURCE season requirements.

Before 15 (fifteen) days from the request of amicable termination, the non-requesting party must respond to the requesting party by stating whether it disputes or not that the current situation falls under one of the three cases above. In case of non-dispute or non-response, the amicable termination is established.

Then the terminated contract will be settled in due proportion of its initial amount based on the performed services that have been delivered to the Client and duly met the quality control criteria until the date of the termination request. This amicable termination cannot exempt the provider from the applicable penalties (the eventual penalty for delayed completion of services will be computed as stipulated at the last indent of the previous section F).

However it is expected that both the Client and the provider will do their best to attain, within the agreed time, the required output quality, without having to apply any penalties, and to the satisfaction of both parties. This entire quality control procedure intends at first to prevent factors of non-quality or to eliminate them before they entail significant difficulties for either party.

H.   PRECEDENCE OF THE SOURCE HANDBOOK

Apart of particular exceptions to be agreed by both parts and expressly mentioned in due time on the contract, the SOURCE Handbook will override, as regard the generic method only, any other contractual document. This includes the present Terms of Reference, where method indications are given for information only.

The SOURCE Handbook will be considered as forming part of the Contract.

I .  RECAP OF TIME SCHEDULING

| - Sending of the request for proposals : | [INSERT: Date] |
| --- | --- |
| - Receipt of proposals: | [INSERT: Date] |
| - Opening and evaluation of proposals : | [INSERT: Dates (From… To…)] |
| - Selection of consultant and negotiation : | [INSERT: Dates (From… To…)] |
| - Signature of contract and beginning of assignment (expected date): | [INSERT: Date] |
| - Completion time (mandatory deadline): | [INSERT: Date] |

By signing these terms of reference, the Consultant is acknowledging his sufficient acquaintance of the following annexes, that he has obtained in due time in the computerized form: (i) the plan of the reference network, (ii) the SOURCE Handbook, (iii) the data input models and (iv) the special SOURCE calculation workbook.

FOR CLIENT					FOR THE CONSULTANT

Signed by					Signed by

Title:						Title:

More, please note that the Client will reserve the right to demand that any agent is promptly replaced if his work shows definitive shortcomings.

These eventualities refer to the main rules of the in-field SOURCE method.

…after due correction by the Controller in case of omissions or errors affecting the lengths.

…after due correction by the Controller in case of errors.

Find at Chapter F para d above the exact definition of the completion time.