SOURCE (Standard Overall Ultralite Road Care Estimate)

# 7-step

# Methodology for extraction

of reference networks

## Summarized from Benoît Ninnin, Consultant

|  | CAUTION: As set out elsewhere, the Reference Networks as presented here constitute simple statistic samples for the calculation of standard macro-indicators. |  |  | The rules as presented herebelow are simplified and summarized. Please consult the relevant Tech-nical Note for full detail and comments (available in French only). |
| --- | --- | --- | --- | --- |
| ## Step #1 Classifying cities |  |  |  |  |

## Size A:  500 < Pop                           (or seaports: 250<Pop)

Size B:  100 < Pop < 500                   (or seaports: 75<Pop<250)

Size C:  50 < Pop < 100

Size D:  25 < Pop < 50

### Pop being the population in thousands of inhabitants.

The cities sized «E» (under 25,000 inhab.) will not be considered.

Before moving on to step #2…

The basic tool :

The matrix of ranks

| Rank of the route from… to … At the following steps, each route will be assigned a rank. First the opposite matrix will apply: depending on whether this route connects an size-X city to a size-Y city or a size-X city to a rank-N route. We will process rank per rank. |  |  |  | to |  | (Route Rank or City Size) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 |  | 3 | 4 |
|  |  |  |  | ### A | ## B |  | ### C | ###### D |
|  |  | # from (City Size) | # A | 1 | 1 |  | 2 | 3 |
|  |  |  | #### B | 1 | 2 |  | 2 | 3 |
|  |  |  | ##### C | 2 | 2 |  | 3 | 3 |
|  |  |  | ###### D | 3 | 3 |  | 3 | 4? |

## N.B. The connections D-D and D-4 either receive the rank or are not ranked. See Step #5..

## Step #2  Listing rank-1 routes

### Rank 1 =    Connections between a city A or B and a city A

+   Connections between a city A or B and a route ranked 1

The connections between a city A and a route ranked 2 will be added at the next step.

# Before moving on to Step #3…

…Some basic rules for ranking, to be applied at each step

| Please respect the steps: first treat the major connections |  |  |  |  |
| --- | --- | --- | --- | --- |
| Suppose 3 cities of various sizes, connected as illustrated. | This diagram comports in fact 3 different routes: A-B, B-C and A-C. | Applying the matrix to each of these routes separately would give rank 1 for A-B, and rank 2 for both B-C and A-C. | Actually, we proceed by steps, beginning with ranked 1 routes: thus let’s assign the rank 1 to A-B. | Now let’s treat the remaining connection between a city sized C and a route ranked 1: according to the matrix, its rank is 2. |
|  |  |  | Feel free to step over smaller cities |  |
|  |  |  | Since we proceed by steps, the first connection to be treated is A-B, that receives the rank 1 according to the matrix (here do not consider the city sized C which interposes). At the next step (about rank 2)), there is no need to treat the A-C or C-B connections, as they are already ranked. |  |

|  |  |  | In case of competing routes, rank the most user-convenient If, between 2 given cities, there are 2 possible routes, the only one to be ranked is the most user-convenient, with regard to : - the kilometerage - the type of pavement - the alignment and other geometric characteristics, without considering the effective condition or the effective travel time. Just compare as if both routes were brand new. |
| --- | --- | --- | --- |
| Please consider these 2 cities sized A. Their connection, according to the matrix, is given the rank 1. | Between these 2 cities, there are 2 possible routes («competing routes»). | Assign the rank 1 to the sole most user-convenient route. The other route will receive no rank at the present step (here, it will be assigned later the rank 4). |  |

Find further on some additional rules, illustrated at the following steps.

This decision rule to be applied at each step.

### Caution: you must decide

Whenever a city sized X is

connected to several cities

of the same higher size Y*,

the rank N given by the

matrix is assigned to the

most user-convenient

of these routes only. The

other routes will downgrade

from rank N to rank (N-1).

### * or to a route the rank of which corresponds to the size Y (with 1 for A, 2 for B, etc).

## Step #3  Listing rank-2 routes

### Rank 2 =   Connections between a city B or C and a city B

### +  Connections between a city C and a city A

### +  Connections between a city B and a route ranked 2

+  Connections between a city C and a route ranked 1 or 2

The connections between a city A or B and a route ranked 3 will be added at the next step.

Now don’t forget to assign the rank 1 to connections between a city A and a route ranked 2

## Step #4  Listing rank-3 routes

## Step #5 Listing rank-4 routes

### Rank 4 =  Other routes listed as “non rudimentary» by one at least of the three major editors of road cartography that cover the entire Africa.

### Rank 3 = Connections between a city C or D and a city C

### + Connections between a city D and a city A or B

+ Connections between a city C and a route ranked 3

+ Connections between a city D and a route ranked 1, 2 or 3

Now don’t forget to assign the rank 2 to connec-tions between a city A or B and a route ranked 3.

### Caution: sometimes you must “associate”

Several cities, if

very close, must

be considered,

from downstream,

as a single city

of upper size.

See the opposite example:

2 cities sized C, are finally

considered as a single

city sized B with regard

to the downstream city

sized A. The route

connecting to A must be

upgraded accordingly,

from rank 2 to rank 1.

How to settle?  Just suppose that the « upstream » city C has 60 000 inhabitants and is 120 km far from A, and the « downstream » city C has 80 000 inhabitants and is 100 Km far from A. The upstream city C will be assigned, just for ranking its connection to A, an equivalent population equal to its own population (80 000) plus the downstream population multiplied by the ratio of their squared distances to A : (60 000 x (1002 / 1202), which gives 121 667; hence you get an equivalent size B.

Its connection to A must thus be considered as connecting a city sized B to a city A, and be upgraded accordingly from 2 to 1.

But the connection between the two « associated » cities C remains at its initial rank. Anyway such an upgrading must not entail any « upstream » change at all.

Why this ratio of squared distances? In fact it is estimated that the traffic demand between two centers is proportion al to each population and in inverse ratio to their squared distance. Hence this formula.

It is the same for more than two cities. This association model applies to both the cases of multi-center conurbation and the cases of a row of successive cities.

This association rule to be applied at each step.

## Step #6  Up/down-grading corridors

### Within enclosed countries:

Maximum rank =   the major opening-up axis, that connects the demographic capital (sized X) to the border on the way to the Ocean, will be assigned the regular rank for a route connecting a city sized X and a city sized A (this one fictious).

Maximum rank - 1 =   the other effective options for leading to the Ocean, in pursuance of the above decision rule, will be assigned the immediately inferior rank.

### In the “crossed” countries:

- All along these opening-up corridors to the end seaport, the rank of the route cannot decrease (but it may increase in pursuance of the basic ranking rules).

### Other crossborder corridors:

- The major crossborder corridors of the giant countries (Nigeria, South Africa), according to the degree of economic connection with their neighbours, are assimilated to opening-up corridors (as for applying the above rule 1).

### Railway corridors:

- As for main railway corridors (that combine road and rail, with effective rail service –at least for freight-), the road is downgraded by one rank point (unless it is ranked 1).

Then the manifest aberrations, with regard to geography only, will be corrected case by case.

même

### The 3-rank rule («1+2+3 otherwise 2+3+4») :

### If the country comprises route(s) ranked 1, the nominal reference network is:  Rank1+Rank2+Rank3

## Step #7  Extracting the Ref.Network

No more distinction will be made for field measurements.

The same measurement method applies whatever the rank.

- Otherwise, it is extended to Rank 4

Roadlinks: The final division of routes into «roadlinks»  consists in cutting each route of the network at each crossroad within the network and at each city sized A, B or C.

Codes: Codes are assigned with respect to the rank groups (from L1 for the rank 1, etc).

même

The nominal reference networks In order to apply to each country a relevant measurement pitch, according to its size, while preserving the overall consistency of the series collections to be based on theses sample networks, the following rule applies for SOURCE measurements : - For countries that contain link(s) of rank 1, the roadlinks of ranks 1, 2 and 3 constitute together the nominal SOURCE reference network (rank 4 excluded). - For small countries without link of rank 1, this reference network is extended to the roadlinks of rank 4. These provisions allow to organize the measurement campaigns then to express the macro-indicators on more relevant sample sets, from the local point of view and from the regional point of view as well (subject to due precautions for cross-category comparisons). Please find further on as an appendix the composition of the SOURCE Reference Networks of the 48 Sub-Saharan countries, in pursuance of this rule. These 48 networks (as rough drafts) can be viewed using the Map Editor provided in the SOURCE CD-ROM (it contains a full library of these networks, digitized and classified).

SOURCE (Standard Overall Ultralite Road Care Estimate)

Recap of African SOURCE reference networks (in Km)

| Country | Rank 1 | Rank 2 | Rank 3 | Rank 4 | TOTAL 4Ranks | Tot Refer. Networks |  | Country | Rank 1 | Rank 2 | Rank 3 | Rank 4 | TOTAL 4Ranks | Tot Refer. Networks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Angola | 752 | 976 | 2361 | 8010 | 12099 | 4089 |  | Malawi | 360 | 336 | 928 | 892 | 2516 | 1624 |
| Benin | 806 | 137 | 755 | 539 | 2237 | 1698 |  | Mali | 1066 | 2066 | 658 | 2743 | 6533 | 3790 |
| Botswana | 277 | 418 | 1321 | 2591 | 4607 | 2016 |  | Mauritania | 0 | 585 | 1556 | 771 | 2912 | 2912 |
| Burkina Faso | 805 | 468 | 1392 | 1772 | 4437 | 2665 |  | Mauritius | 0 | 12 | 31 | 171 | 214 | 214 |
| Burundi | 69 | 143 | 138 | 542 | 892 | 350 |  | Mozambique | 2381 | 105 | 904 | 5167 | 8557 | 3390 |
| Cameroon | 1727 | 1348 | 1119 | 3355 | 7549 | 4194 |  | Namibia | 355 | 0 | 1380 | 7140 | 8875 | 1735 |
| Cape Verde | 0 | 0 | 0 | 119 | 119 | 119 |  | Niger | 1473 | 942 | 424 | 2821 | 5660 | 2839 |
| Central African Rep. | 348 | 772 | 1296 | 3492 | 5908 | 2416 |  | Nigeria | 9963 | 4024 | 4600 | 4879 | 23466 | 18587 |
| Chad | 739 | 1692 | 540 | 3901 | 6872 | 2971 |  | Rwanda | 178 | 280 | 69 | 508 | 1035 | 527 |
| Comoros | 0 | 0 | 0 | 188 | 188 | 188 |  | Sao Tome & Princ. | 0 | 0 | 0 | 98 | 98 | 98 |
| Congo | 433 | 0 | 770 | 2179 | 3382 | 1203 |  | Senegal | 674 | 646 | 529 | 2359 | 4208 | 1849 |
| Dem.Rep.of Congo | 9158 | 2067 | 2239 | 10407 | 23871 | 13464 |  | Seychelles | 0 | 0 | 0 | 24 | 24 | 24 |
| Djibouti | 54 | 294 | 54 | 130 | 532 | 402 |  | Sierra Leone | 221 | 158 | 171 | 1620 | 2170 | 550 |
| Equat.Guinea | 0 | 0 | 0 | 388 | 388 | 388 |  | Somalia | 2011 | 0 | 361 | 2414 | 4786 | 2372 |
| Eritrea | 203 | 184 | 254 | 400 | 1041 | 641 |  | South Africa | 5017 | 1697 | 5649 | 25273 | 37636 | 12363 |
| Ethiopia | 2632 | 1264 | 1714 | 4608 | 10218 | 5610 |  | Sudan | 4314 | 1961 | 1818 | 5728 | 13821 | 8093 |
| Gabon | 626 | 0 | 672 | 1540 | 2838 | 1298 |  | Swaziland | 0 | 11 | 43 | 523 | 577 | 577 |
| Gambia | 80 | 135 | 147 | 58 | 420 | 362 |  | Tanzania | 3586 | 512 | 2122 | 3452 | 9672 | 6220 |
| Ghana | 1217 | 1583 | 1036 | 2515 | 6351 | 3836 |  | Togo | 391 | 332 | 642 | 512 | 1877 | 1365 |
| Guinea | 520 | 606 | 1212 | 1459 | 3797 | 2338 |  | Uganda | 366 | 1005 | 714 | 1716 | 3801 | 2085 |
| Guinea Bissau | 0 | 0 | 275 | 477 | 752 | 752 |  | Zambia | 966 | 1969 | 1042 | 5545 | 9522 | 3977 |
| Ivory Coast | 1781 | 887 | 2057 | 3194 | 7919 | 4725 |  | Zimbabwe | 1031 | 881 | 847 | 4600 | 7359 | 2759 |
| Kenya | 813 | 223 | 556 | 6930 | 8522 | 1592 |  |  |  |  |  |  |  |  |
| Lesotho | 0 | 7 | 23 | 394 | 424 | 424 |  | Country | Rank 1 | Rank 2 | Rank 3 | Rank 4 | TOTAL 4Ranks | Tot Refer. Networks |
| Liberia | 0 | 343 | 418 | 858 | 1619 | 1619 |  |  |  |  |  |  |  |  |
| Madagascar | 1545 | 767 | 1762 | 3690 | 7764 | 4074 |  | TOTAL 48 ctries | 58938 | 31836 | 46599 | 142692 | 280065 | 141384 |

N.B. Kilometerages as shown here are simple graphic estimates, duly rectified. The error margin is around 5%.