SOURCE (Standard Overall Ultralite Road Care Estimate)

Traffic rectification for a national measurement campaign

## Louis Fernique, SSATP / Road Management Initiative (RMI)

This Note explains and justifies the method for traffic rectification that is implemented by the SOURCE calculation workbook. Abridged versions figure in the SOURCE Handbook and in the incorporated Helpsheet of the workbook as well. Both the workbook and the Handbook come, just as this Note, in the SOURCE CD-ROM.

- Algorithm

  - H-rectification

  - D-rectification

- Traffic behaviour model

- Conditions of use

- Rectification impact

- Insufficient coverage

- Forbidden rectification?

The SOURCE workbook, once fully filled in for a given measurement campaign, contain, throughout its two input sheets, the entire set of needed data from field measurement. Although the rectifying algorithm instantaneously resfreshes at each entry, for explaination convenience, its is described hebelow as a sequential process.

The workbook directly makes a simple statistical analysis of all the valid traffic observations during the campaign, from which it derives a dual rectification key: H-rectification (hour-based rectification) then D-rectification (day-based rectification).

Each rectified traffic level thus reflect a standard business hour of a standard business day, instead of the actual time of the corresponding observation.

But of course this method does not allow to extract a variation law throughout the year. The SOURCE rectified traffic profile is considered typical of the dry season only (which supposedly corresponds to the optimal driveability throughout the year).

The two rectification profiles finally obtained are illustrated by graphics in the country summary sheets of the workbook.

| Country LV Traffic Profile |  |
| --- | --- |
| The black line shows the average traffic for standard business hours (8 AM to 5 PM). The coefficients for each hourly slot figure under the graphic.. | The black line shows the average traffic for standard business days 9Monday to Saturday). The coefficients for each weekday figure under the graphic. |

Traffic profile graphics : example of Ghana (231 observation slots for a total of 143 hours, distributed over 3 weeks of dry season and covering most of the interurban network).

1. Algorithm

The rectification formula is always calculated from all the available observations, without taking weightings into account.

1.1. H-rectification:

- All the available observations are broken down into 12 mn segments (1/5th hour), with a dual cumulation: counts on the one hand and observation quantities on the other hand.

- We thus calculate the mean traffic level observed during the campaign over the entire network, for each 12 mn segment.

- This traffic level curve is smoothed using a 15-segment (3 hours) sliding windoid. Special precautions must be taken to avoid undesirable effects at the edge of the observation window or in the event of segments without any observation. The curve is in fact consolidated before smoothing according to precise rules that regulate its smoothing behaviour.

- We calculate the mean traffic level for a standard 8AM–5PM window, and then for each 12 mn segment, a rectifying coefficient enabling the corresponding smoothed traffic to be related to the standard mean traffic. The rectification formula according to the hours in a day (H-rectification) is thus determined.

- The real counts of each observation are distorted by applying these rectifying coefficients in 12 mn fractions of the corresponding observation segment. From this, the H-rectified traffic levels are derived for each observation.

1.2. D-rectification

We now work on the basis of the already H-rectified counts. We apply a very similar process to them

At this time it is not based on 90 12-mn segments among which 45 formed the calibrator (the segments from 8AM to 5PM) but on the 6 days of the business week, which themselves form their own calibrator (6 days from Monday to Saturday, traffic observations not being allowed on Sundays). The logical steps remain the same and from these steps we derive the series of 6 coefficients of the D-rectification process, which is the rectification based on the days of the week.

From this, we derive the counts, then the (H and D)-rectified traffic levels for each observation.

The calculation tables with all the successive H-rectification and D-rectification stages can be consulted at any time in the "Distributions" sheet.

But in practice, this is of no practical concern to the calculation workbook user as all the calculations are made directly without any action on his part.

2. The traffic behaviour model

The model on which this algorithm is based is as follows:

We will call StandardTrafi the standard traffic level of a given section with homogeneous traffic. By standard traffic level, we mean its mean traffic level in the standard segment of business hours (conventionally, 8AM-5PM) and in the standard segment of business days (conventionally Monday to Saturday, special public holidays excluded), in the dry season.

We consider that the instantaneous LV traffic level measured on this section at any given hour H and on a given day D of the standard business week, in free-flowing traffic conditions, is as follows:

Trafi (H , D) = StandardTrafi  x HRectifCoef (H) x DRectifCoef (D)

In which the HRectifCoef (H)  and  DRectifCoef (D) functions are "country-specific functions" independent from the choice of section in the relevant network.

The successive H-rectification et de D-rectification processes consist in approximating the two inverse functions of these coefficient-functions, by statistical analysis of all the campaign measurements: in 12 mn increments for the H-rectification, for day 1 to 6 for the D-rectification which is a discrete function.

3. Conditions of use

This modelling assumption is only applied to SOURCE observation sets, determined by taking the special precautions of the SOURCE method set out in the manual, both as regards measurement precautions and general campaign organization: network coverage; limits of the daily hourly window; exclusion of public holidays; exclusion of non free-flowing traffic; etc.

These conditions guarantee inter alia a sufficient favourable volume and distributions of observations for the model to give good results with experience. For instance, in an ordinary country, more than 200 traffic observation segments over a three-week period exceed 100 cumulative net hours evenly distributed over the network, and uniformly cover a  6:30AM to 7:00PM time segment from Monday to Saturday.

4. Rectification impact

Only the primary and tertiary macro indicators are influenced by the quality of the rectification, as the secondary indicator (travel speed) does not take into account the traffic observations.

Experience has shown that the impact of rectification on the primary and tertiary indices is insignificant: around 5 per thousand to 1 per cent. Anyone can check this at any time for any special case, since the workbook comports a simple checkbox that enables you to deactivate the rectification algorithm and calculate the indicators on the basis of "as is" traffic.

This very slight impact on the overall indices is to be expected because the basic distributions of the corpus are roughly homogeneous and in fact only the internal redistributions of the traffic volumes are applied to it, which only affect the overall volume because of the uniformity defects of the edges (particularly spill-overs of the of the measurement time segments with reference to the standard time segment). The algorithm is necessarily a "quasi-nil sum" process.

However, the algorithm is essential and its impact significant as soon as we consider traffic-sensitive indicators at link level, which are measured in a specific time segment on a specific day. The traffic distribution map over the network would be worthless without a sound traffic rectification model.

- Insufficient coverage

As long as you are dealing with data entry from the first links, the coverage of standard hour and weekday ranges keeps undoubtly presenting gaps.

For example: there is no Thursday measurement, or no measurement including the 1PM-2PM slot as yet.

At this time the already available traffic observations keep certainly insufficient for the traffic level rectification algorithm to give relevant results.

Then the workbook continuously displays a special warning mentioning this problem. It will automatically disappear when this coverage becomes complete. Which should not be considered not a definite clearance.

Rectifying prohibited? When performin a standard SOURCE campaign on a very small country –for instance a small island-country of the Indian Ocean - , 3 or 4 days could be sufficient to measure the entire Reference Network. The ordinary traffic level rectification mechanism, based on campaign observations (and only these observations) will not be valid. It will therefore be necessary: - either to extend the observation of the traffic levels alone until an adequate standard of coverage is achieved, - or to make the combined speed/traffic measurements while complying with narrow hour/day time segments that have been well chosen at the start. These are special cases that require the method to be carefully adapted and possibly handled slightly differently. The "as is" workbook, designed for the most common cases, will simply issue a warning. At this time, there is no suggestion of a pertinency threshold that is statistically justified. Anyway, under 50 observation hours, the corpus is certainly not sufficient to found the rectification process, and you need additional investigations.

As for the definitions of the 3 SOURCE macro-indicators (common speed, travel speed and shape ratio), you are referred to the SOURCE Handbook or to the Technical Note « The SOURCE macro-indicators, construction and interpretation ».