| PARTICIPANT’S HANDBOOK | T&T |
| --- | --- |
| Basics (25-word list) | 1.5 |

Contentsx

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________

##### Road types .

##### Paved (or sealed) roads (“asphalted” or “tarred”)

- Unpaved (or unsealed) roads and tracks

###### Operation types .

- Routine maintenance

- Periodic maintenance

- Rehabilitation or reconstruction

- Curative maintenance / preventive maintenance

###### Works .

- Road menders / Road mending  / Roadmen work / Lengthmen work /

« Cantonnage »

- Potholes

- Patching

- Reshaping / Regravelling

###### Traffic impact .

- Traffic / Heavy (or truck) traffic / Light traffic

- Axleload / Aggressiveness / Aggression

- Wet season barriers  / “rain” barriers

###### Execution line .

- Owner / Road authority / Construction financing authority

- Owner's agent / Owner’s surrogate / Contracting agency

- Construction authority

- Force account

###### Resources .

- Tax / Taxation / Levy

- Fees / User fees

- Road tariff

###### Financing principles .

- Budget arbitration / Finance Act / Earmarking

- Road "commercialization"

###### Economic impact .

- Vehicle operating costs (VOC)

- Cost-effectiveness / Internal rate of return (IRR)

###### Responsibilities .

### Decentralization

__________________________________________________________________________________________

##### Paved (or sealed) roads (“asphalted” or “tarred”):

- A road is "paved" if its surfacing comprises a highly cohesive, sealing upper course, which, among other functions, protects the complete pavement structure.

- Unpaved (or unsealed) roads and tracks:

- "Unpaved" roads (or tracks) are fragile, due to the absence of such an upper course.  Heavy traffic scatters the loose material which is not cohesive.  In-depth damage is also caused when the pavement structure is waterlogged.  Surface water can also very easily cause damage by erosion.

- You can improve at the construction the performance of the upper materials ( by careful selection, incorporation of cement, etc.).

- Construction of a good unpaved road costs roughly 5 times less than for a paved road.  However, proper maintenance of a circulated unpaved road can be more costly, and even with efficient maintenance, its lifetime will be shorter.

- Everything depends on the traffic which the road has to carry, mainly on truck traffic.  If expected traffic levels are too low, paving would represent an enormous, unjustified investment (by comparison, the economic benefits derived from the road are much too limited).  However, paving is essential above a certain traffic level; but traffic expectations depend on the paving/unpaving option, due to driveability that differs.

- This means that a simple economic calculation is required in each case, in order to establish  which option is the most "cost-effective" for the community.

- Routine maintenance:

- This corresponds to "light" maintenance work, executed throughout the year in order to extend the lifetime of a road as much as possible and at lowest cost.  Routine maintenance includes cleansing ditches, grass mowing, undergrowth clearing (all tasks eventually performed by road menders in a “cantonnage” process), and patching on paved roads, or frequent reshaping of unpaved roads.

- Periodic maintenance:

- "Semi-heavy" maintenance work, to be periodically performed in order to preserve the pavement structure by renewing its topping, also called resurfacing: adding a new wearing coat for paved roads (undertaken every 8 to 12 years, according to traffic)  or regravelling unpaved roads (undertaken every 4 to 6 years, according to traffic).

- Periodic maintenance rejuvenates the road topping, without renovating the pavement structure itself.

- Rehabilitation or reconstruction:

- "Heavy work" designed to reconstitute the structure of a road in very degraded condition.  Rehabilitation and reconstruction programs are not included under the heading of maintenance, as they are full re-investments required to re-establish an infrastructure which, in principle, has already been fully depreciated and comes to the end of its lifetime.

- Curative / preventive maintenance:

- Curative maintenance consists of repairing damage after it has occurred.  Preventive maintenance consists of taking action at the time when damage becomes probable but before it has actually occurred, in order to avoid or reduce such damage.

- Example: Removal of slop slide material is curative.  Planting the slope with vegetation to avoid erosion due to water circulation is preventive.

- An appropriate preventive maintenance policy can achieve savings and a better general level of service, against a policy limited to curative maintenance (even if executed rapidly and efficiently).

- Both routine maintenance and periodic maintenance include a majority of preventive tasks, plus a number of curative actions.

- Road menders / Road mending  /  Roadmen work  /  Lengthmen work / « Cantonnage »:

It refers to a way of organizing all the smaller tasks of routine maintenance (the tasks a single man, on foot, can perform with just basic manual tools ; these tasks involve grass mowing, undergrowth clearing, cleansing ditches, pipes and culverts, etc.). Each road mender (or roadman, or lengthman) is entrusted with a 2- or 3-km road section and take care of it all year long, under some supervision.

- Equally, « cantonnage » may either be performed by force account either be outcontracted.

- Potholes:

- Isolated destruction of the sealing of a paved road, generally induced in a worn surfacing by heavy traffic.

- If a pothole is not dealt with promptly, it spreads and deepens ; there is locally no longer sealing nor any protection for lower courses, which can now be seriously damaged by water infiltration or  direct axle impacts. A pothole, if not promptly repaired, worsens quickly ; it creates a spot of accelerated deterioration.

- Patching:

- This routine maintenance operation involves dealing with a pothole "in time", namely as soon as it appears, before any structural damage to the road can occur. Its French name (“point-à-temps”, which means exactly “patching-in-time”), is even more self-explanatory.

- You just cleanse,  fill in,  compact and  re-seal. Patching can be hand-crafted or performed more mechanically, but it is always rather technical and requires some experience.

- When the surfacing comes to a certain wear level, both the density and the outbreak frequency of potholes grow highly; at this step the patching method does not fit any longer (too slow, too expensive and technically sub-optimal). Then the time has come for resurfacing, which is periodic maintenance.

- Reshaping / Regravelling:

Maintenance operations concerning unpaved roads only.  A distinction must be made between:

- Reshaping, with no systematic addition of external material, involving leveling and rectifying the pavement profile with a grader (or more rustic equipment).  This procedure eliminates rutting, mud-holes, etc.  This work constitutes  routine maintenance, required one or more times per year  according to traffic (up to 6 times per year in some cases).

- Regravelling, which is reshaping with systematic addition of external selected material to re-gravel the pavement, reconstituting satisfactory topping characteristics. The pavement profile is inevitably rectified too. This corresponds to periodic maintenance, required every 4 to 6 years according to traffic.

- Traffic / Heavy (or truck) traffic / Light traffic:

Circulation of vehicles on roads.  A distinction is made between light traffic (cars, etc.), which cause little damage to the road, and  heavy  traffic (trucks, etc.) which cause considerable damage.

If we rate at 1 the damage caused by a car passing, the damage caused by a truck passing must be rated at 50,000 to  100,000 (in other words, a truck causes as much damage as 50,000 to  100,000 private cars).  The damage caused by a heavily overloaded truck must then be rated at 500,000 to 1,000,000.

- Axleload / Aggressiveness / Aggression:

The axleload determines both the pressure and the horizontal strains which the tire exerts on the pavement, and consequently the damage (the aggression) caused to the road surface by the vehicle.  For equal loads, the aggressiveness of 2 trucks against the road can differ considerably according to the number, type and arrangement of their axles (the global load being unequally shared between the axles).

The pavement structure is initially designed for a specified lifetime (the design time), on the basis of assumptions concerning the future volume of heavy traffic, and according to the maximum statutory axleload limit.  If the actual heavy traffic is more intense than anticipated, or the axleload limit is not observed, the pavement is subjected to greater fatigue and ages prematurely.

The road could be irreparably damaged long before it has returned the whole economic benefit which was expected (although the community have to continue reimbursing the initial investment). Economically, it would be a fiasco.

The damage induced by an axle is proportional to the 4th power of the load on the axle. Comparison between trucks and private cars: - Taking a private car, with each of its 2 axles subject to the same load of 0.6 T, the damage induced is: - DamCar = 2 x (Coeff x 0.64) [Here the value of Coeff does not matter.] - Taking an overloaded truck, with 5 axles each subjected to the same load of 16 T, the damage is: DamTruck = 5 x (Coeff x 124) = 5 x (Coeff x (0.6 x 2 x 10)4) = 5 x 104 x 23 x 2 x (Coeff x 0.64) = (5 x 104 x 23) x DamCar = 400,000 x DamCar This means that one single truck can cause as much damage to the road as 400,000 private cars (for a busy African road this corresponds to about 1 year of light traffic). Axle or vehicle aggressiveness Assuming that the maximum authorized axleload is 11 T, we will take the case of an axle loaded 20% above this limit (its load is 1.2 times greater than the maximum authorized “standard” axleload, which gives 13.2 T). The damage caused by this overloaded axle is then: DamOverloadedAxle = 1.24 x DamMaxAuthorizedAxle  2.07 x DamMaxAuthorizedAxle We say that the aggressiveness factor of this overloaded axle is 2.07 (because it causes 2.07 times more damage than the “standard” truck axle, i.e. the most loaded as authorized). Going back to our private car example, remember that each axle was subject to a load of 0.6 T, or only 5.5% of the 11 T standard truck axle. The aggressiveness of the private car axle is consequently: 0.0554  10-5 (or 0.00001, or 1/100,000). This figure is clearly negligible, and we can say, in broad terms, that the aggressiveness of a private car axle (compared with a standard truck axle) is zero. For simplification purposes in the “Tariff & Traffic” game, we use the total aggressiveness of a vehicle (and not the aggressiveness of each single axle), calculated on the basis of the total aggressiveness of a standard truck.

- Wet season barriers (or “rain” barriers):

By definition, unpaved roads are not protected by a watertight surfacing.  Consequently heavy rain infiltrates deep into the material of the road structure, reducing its cohesion and therefore its bearing capacity (far from unknown, your foot sinks easily in a muddy ground, which has therefore a much lower bearing capacity than a perfectly dry beaten earth).  During several hours after heavy rains, the road is temporarily much more fragile than in dry weather.

If a heavy truck uses the road under these conditions, it causes major damage both to the topping and internal structure of the road.

This is why rain barriers are installed in certain cases, used to close the road to heavy traffic (or even to all traffic) following heavy rain, until the pavement dries out and returns to its normal bearing capacity.

- Owner / Road authority / Construction financing authority:

- This is the "owner" of the road, in other words the authority which is supposed to manage the network and schedule the road works, which finances the work (or raises the funds), and issues work orders.  The Owner is responsible to road users for network management quality.

- Owner's agent / Owner’s surrogate / Contracting agency:

- This is a specialist body, acting on behalf of the Owner, which in certain cases orders and supervises all work on the road network, if the Owner does not wish to assume these functions itself.  The Owner's agent is sometimes para-governmental, often non-profit, that acts as a contracting agency for design, work or control contracts. This proxy, to be entrusted on a multi-annual basis, is rarely subject to formal competition.

- Construction authority:

- This is a specialist entity responsible, under contract from the Owner (or the Owner's agent acting on behalf of the Owner), generally for both defining work projects and controlling the contractor charged with executing the work.

- A construction authority is needed in the majority of cases, to be in daily contact with the contractor, to issue detailed instructions, to check the reality, quantity and quality of the work executed and then to give the first clearance for payments.

- The construction authority is generally a private engineering firm, and is awarded a control/supervision contract specific to a given operation, generally as a result of a call for tenders.

- Force account:

- The administration is said to be working on a "force account" basis, where it undertakes execution of road work using its own staff and its own plant and equipment.

- In this case, the work is also naturally supervised on a force account basis, generally by the department responsible for actual execution of the work.  There is then no clear distinction between the site management function and the "control/supervision" function.  Indeed, where the administration is operating on a national road on a force account basis, it represents the Owner (the State) while acting both as construction authority and as contractor.

- The real cost price of force account work is uneasy to determine from public accounts (numerous items, such as the depreciation of equipment, salaries of administration officials, head-office expenses, etc. are hard to isolate and share out).  Frequently only marginal costs (for instance fuel costs) are taken into consideration, all the non-marginal costs being forgotten; hence the illusion that force account is highly competitive!

- Nor has the force account system any strong economic logic providing a natural incentive for improving performance (productivity gains and cost control).

- The traditional weakness of the force account system lies in the area of non-financed operations to which both machines and manpower are often extensively rerouted (but they are only apparently "free").

- Tax / Taxation / Levy:

- This concerns additional charges paid by the tax-payer in connection with particular operations (purchase of fuel, registration of a vehicle, etc.), which represent the tax-payer's contribution to the community needs.  These levies or indirect taxes form part of the general taxation system.  They are paid into the general State budget, even though some are earmarked, namely destined for a particular type of expenditure.

- Fees / User fees:

- These are additional charges paid by the user in connection with certain operations (purchase of fuel, registration of a vehicle, etc.), which represent the user’s return for using a public utility, namely a payment for service (examples: airport fees, water and electricity bills, etc. or road maintenance fees in some cases). These fees are not to be paid into the general State budget, but are paid directly to the entity responsible for managing this specific resource and  financing the operation of the service. These fees are not taxes nor levies.  They are sometimes referred to as "parafiscal" charges, to indicate that they are in addition to taxation.

- Road tariff:

Where the road user is subject to a set of user fees, assigned to finance road maintenance, these fees as a whole are referred to as road tariff.

- Budget arbitration / Finance Act / Earmarking:

The general State budget is a forecast document, indicating both forecast revenue (including tax revenue in particular), and forecast expenditure for the coming year.

Each year, the budget prepared by the Ministry of Finance is approved by the Government following arbitration between the requirements expressed by the various spender ministries.

The budget is then submitted to the Parliament, which debates, amends, corrects and finally approves the budget by vote, under the form of a special law, referred to as the "Finance Act".  This law authorizes the Government to implement the yearly budget, namely to collect revenue and commit expenses.  Until the Finance Act has been promulgated, no expenditure can be committed by the administration for the new fiscal year (apart from exceptional cases).

This means that even "earmarked" tax revenue, namely revenue theoretically intended for total allocation to a clearly defined application, is subject to the budget arbitration and Finance Act process.  So there is no total guarantee that the budgets allocated will ultimately correspond to the full amount of such revenues.

[ It is indeed a traditional principle of public management that the State should retain ample budget management margins by prohibiting "earmarked" budget revenue.  This is referred as the "single treasury" principle.]

Earmarking fiscal revenues for financing public utilities operation is a poor and hazardous method. Commercialization is the clear and proper solution.

- Road "commercialization":

Such an institutional reform process consists of transition from a previous traditional situation where road maintenance (as a State responsibility) was financed from the general State budget, and managed exclusively by the administration, to a new situation where road maintenance (as a special market service) will be financed by user fees constituting the road tariff, managed jointly by the administration, road authorities and road users.

- Vehicle Operating Costs (VOC)

#### Where a transport company provides a commercial service (passenger or good transport), a considerable number of items are included for calculation of the cost price of this service.

#### These include notably:

#### items which constitute the real technical vehicle operating cost (“VOC”), including fuel and lubricants, consumables and wear parts (tires, V-belts, filters, brake discs and shoes, shock absorbers, clutch disks, etc.),

#### and also  some other direct charges, fixed or variable, as:

#### the components of the road tariff (not to count twice the fuel fee, as this is already counted in the price of the fuel; including annual road fee and/or registration fee if any, official tolls, etc.),

#### as well as

#### informal tolls, etc.,

#### vehicle insurance,  etc.

#### …not to mention items that are usual for any industry (as wages, commercial costs, head-office costs, asset depreciation, etc.).

#### The VOCs, calculated per tons x kilometers (TxKm) for goods, or kilometers (km) for passengers, naturally increase if the road tariff increases, but drop (much more quickly) when the state of the road is improved.

In actual fact, every dollar properly spent in road maintenance is considered to save all the road users around three dollars on vehicle operating costs (VOCs).

- The primary objective of a good road maintenance policy is to reduce VOCs, therefore to reduce the cost price to the transport company and so to reduce the transport price billed by the transport company to its customers…and then to reduce the transport part within the cost of any product.  This is indeed the way the transport industry must support the general economy.

- Cost-effectiveness of public expenditures / Internal rate of return (IRR)

The choices to be made when setting up a road maintenance program are not always particularly clear-cut.

For example: For a given road, should periodic maintenance be initiated straight away, or is it better to wait a year, commencing with simple routine maintenance which costs ten times less?

Or: Of two periodic maintenance operations on different roads but which cost the same amount, which should be given priority this year?

For such public expenditure, since the concept of profitability is not relevant, the concept of cost-effectiveness is being used.

Generally the relative interest of each solution (or choice) is estimated by comparison with a reference solution, conventionally called the status quo.  The interest of the solution examined (its interest for the community) is frequently assessed according to its internal rate of return.

The procedure is as follows:

- For each year of the lifetime of the operation, the benefits for the community on the one hand, and the cost for the community on the other, are estimated in terms of monetary value.  We therefore have annual balances (benefits versus costs), which can be positive or negative according to the year.

In fact it is not absolute costs or absolute benefits which are taken into account, but only the excess costs and additional benefits by comparison with the reference solution (status quo).

Values are estimated in constant monetary units (start year value); so the calculation keeps valid regardless of the inflation that will occur during the period.

What aspects are examined in terms of benefits and costs for the community? Example of an investment operation: The costs are essentially investment costs and the annual maintenance differential cost, although other less evident costs (or negative benefits, which are the same thing) can be involved. As regards benefits (but also costs in certain cases), the consequences of the project for the following are principally taken into account: - Transport (impact for users) - Local economy (surplus for producers in micro-economic terms) - The State as an economic unit - National revenue (in macro-economic terms) - The environment, etc.

- It is not sufficient merely to add these balances.  The time scale element must be taken into account: the further an expected benefit, the lesser it appears interesting…and vice versa for costs: the further a scheduled expenditure, the lesser it appears penalizing.

All the annual benefit-cost balances are therefore added, each being depreciated according to its distance in time.  A discount rate is used for this purpose.

BalanceGlobal =   Balanceyear1 / (1+d)

+   Balanceyear2 / (1+d)2

+   Balanceyear3 / (1+d)3    +    etc.

The discount rate ("d") is a percentage expressing the degree of  "preference for the present" of the community concerned.  This is determined according to its budgetary leeway (cash position and capacity for raising external funding).  Thus $ 100 of cost or benefit this year is regarded as the equivalent of $ [100/(1+d) ] of cost or benefit in one years' time.

Yearly balances can vary considerably from year to year. In a typical case, for a simple road maintenance operation, the main costs are concentrated in the first year, while the benefits are spread over the following years.

Thus, in a case of this type, the greater the value of “d”, the greater the extent to which the benefits are depreciated, and vice versa.  If “d” is very high, the global balance will inevitably be negative (for in this case immediate costs are greater than highly discounted subsequent benefits).  If “d” is very low, the global balance is generally positive (for the substantially enhanced future benefits are greater in this case).  Somewhere between the two extremes therefore, there is a value of “d” for which the global balance would be zero.  This special value of “d” is referred to as the internal rate of return (IRR) for the operation.

If the IRR for an operation is 17%, what does this signify? - If the discount rate for this community concerned is exactly 17%, this operation would be regarded as neutral, with costs and benefits equivalent, corresponding to an economic balance of zero. - If the discount rate is greater than 17% (money is harder to find for the community), the balance for this operation is negative, and consequently not profitable for this community. - If the discount rate is less than 17% (money is easier to find), the balance is positive, and this operation can be regarded as profitable. Naturally, if two technical solutions have been considered for this operation, where the IRR is 17% in one case and 34% in the other, the latter is twice as profitable as the first (each dollar invested in the first solution produces twice as many discounted collective benefits than if the second solution had been adopted).

The economic substance behind this calculation is very simple: a country which can raise funds easily can engage in investments for which the return does not occur for a long time.  In a country where money is short, investments for which the return is rapid must be privileged.

For developing countries with low budgetary leeway and limited borrowing power, the discount rate to be taken into account is between 12 and 14%.

This means that only operations where the IRR exceeds these values are of interest.

In the present context in Africa, where road traffic levels are relatively low, effectively funded new road investment operations rarely generate IRR values exceeding 15%.  In some cases they are on the break-even borderline.

In contrast, rehabilitation operations frequently show IRR values of 20%.  Above all, maintenance operations (and periodic maintenance in particular) systematically generate "exceptional" rates of return exceeding 40%, in cases going as high as 100% (as the status quo, in other words non-maintenance, causes very substantial loss of benefits, as well as substantial subsequent excess).

This is a general rule, extending beyond the road domain, and in no way restricted to developing countries: the rate of return from maintenance is substantially greater than that from investment.

The economic lesson is very simple: any dollar spent in a maintenance operation would roughly be twice more profitable as any dollar spent in an investment operation.  So if you really only have one dollar while needing 10 dollars for maintenance and 1 dollar for investment, then maintenance is a mandatory choice.

TO UNDERSTAND MORE CLEARLY: “Discount rate”, "preference for the present” - what does all this mean? Taking a simple example: My neighbor owes me $ 100. He offers this choice: either he will pay me back the $ 100 now, or $ 101 in one week. Naturally I prefer to get my $ 100 now. Then he offers me another choice: $ 100 now, or $ 102 in one week. I still prefer $ 100 now. He then raises his offer to $ 103, and so on. Sooner or later I will have to agree, for example at $ 150. I would indeed prefer $ 150 in one week, rather than $ 100 today. My discount rate (or my preference for the present rate) is therefore 49.5% (in this case we are talking about a weekly discount rate). In point of fact I had already refused at 149, which means that $ 149 tomorrow is worth less to me than $ 100 today. But I finally accepted 150, which means that $ 150 tomorrow is worth more than $ 100 today. My reactions demonstrated that I have a preference threshold situated at about 149.5. However, my discount rate depends on my situation at the moment of choice, and particular on my cash position. I reacted in the way described because I already had $ 1,000 in hand for my weekly expenses, and so no cash problem. On the contrary, if I had had no cash in hand and a genuine need for the $ 100 right away, I would certainly have reacted differently. I would have more firmly demanded my $ 100 immediately, with no intention to agree any delaying arrangement. Although perhaps if my neighbor had gone up to 190, I would have finally agreed. Never mind $ 100 straight away, I would have found a way round the problem. $ 190 in a weeks' time – that would be worth waiting for. It shows that by lack of ready cash, my discount rate should become 89.5% (per week). But my discount rate also depends on the cost of money, or the interest rate which I have to pay if I borrow. If I agree to wait for my $ 100 (although I have immediate need of this sum), I will have to borrow from a loan shark who charges me 60% interest per week. So if I accept my neighbor's offer of $ 155 in one week, I will then receive my 155 but will also have to repay 160. The balance for me is then not a gain of $ 50 in a week, but a loss of $ 5 in a week. So the offer is not acceptable. I might end up by accepting an offer from my neighbor at 210 knowing that my actual gain one week later would not be 210 - 100 = $ 110, but only 210 - 100 - 60 = $ 50. Due to the high money cost, my discount rate would then be 109.5%. If my loan shark had been even greedier, I would have demanded more from my neighbor. My discount rate consequently depends on both my cash position and my ability to procure funds from an outside source. Coming back to cost/benefit analysis: If the discount rate for the community is 13% per year (taking a more realistic value), what does it mean, in view of the budget situation of the country and the borrowing rates to which it has access? - a benefit (budget income) of 100 millions this year is considered exactly equivalent to a benefit of 113 millions next year (it is preferable to defer a possible 100 million income this year, provided it will become at least a 113 million income next year); - likewise a cost (budget expense) of 100 millions this year is the equivalent of a cost of 113 millions next year (it is preferable to put back a 100 million expense this year, provided the same operation next year will cost no more than 113 millions).

For the real enthusiast: So what is the internal rate of return for the status quo solution (adopted as a reference for analyzing a particular operation)? Each year, the costs and benefits to be taken into account (or rather the cost differences and benefit differences compared with the reference solution) are zero by definition. Consequently the cost-benefit balance is always strictly zero regardless of the discount rate applied. This means that the IRR for the status quo solution (namely the discount rate for which the cost-benefit balance cancels out) is non-zero: this IRR is indeterminate, also by definition. In fact the question about the IRR of “status quo” makes no sense.

### Decentralization

This corresponds to a general reform program for the institutional organization at the country level,  whereby the central government (the State) transfers certain of its responsibilities to new local authorities (at various levels: province, region, district, county, municipality, or whatever their names in a particular country), provided they are and governed by locally elected representatives and fully empowered (not under central supervision).

Such local communities are no longer merely administrative divisions under State-appointed officials.

The basic idea of decentralization is that some responsibilities, depending the matter, could be better managed from the local level than under an overcentralized scheme.

#### But for roads as well as other matters, decentralization will not remedy an extensive lack of resources.

#### In principle, if the State transfers certain of its responsibilities (namely charges to be assumed, such as for example, the maintenance of a local road network), it must also transfer the resources necessary for exerting these responsibilities. This is achieved either by budget allowances to be renewed annually, or by setting up a local taxation system which will provide the local authorities with their own, adequate direct resources, and, in addition in the case of road maintenance… and by allowing them equitable access to the resources managed by the Road Maintenance Fund.

Usually bituminous.  Asphalt or tar are in fact specific materials.  The investment cost for concrete paving is very high, and is consequently only justified for very high traffic levels or particular operation constraints.

The discount rate has nothing to do with the inflation rate (which describes monetary erosion).

In this case we calculate in terms of constant monetary unit value, referenced to year zero. Even in a country with a non-inflationary economy, commitment of an expense this year is more restrictive, in economic terms, than commitment of the same expense next year. The discount rate expresses this phenomenon and nothing else.

If calculations are not made in constant monetary unit value, the annual balances must be corrected by applying both the expected inflation rate and the discount rate.

For a well-designed, correctly timed operation.  If the same operation were put back 2 or 3 years, the IRR would certainly be less favorable, due to reduced benefits and excess costs.  IRR calculation makes it possible to optimize the start year for a road maintenance operation.