| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) ) Contentsx | 5.6 |

##### Road types .

##### Paved (or sealed) roads (“asphalted” or “tarred”)

- Unpaved (or unsealed) roads and tracks

###### Operation types .

- Routine maintenance

- Periodic maintenance

- Rehabilitation or reconstruction

- Curative maintenance / preventive maintenance

###### Works .

- Road menders / Road mending  / Roadmen work / Lengthmen

work / « Cantonnage »

- Potholes

- Patching

- Reshaping / Regravelling

###### Traffic impact .

- Traffic / Heavy (or truck) traffic / Light traffic

- Axleload / Aggressiveness / Aggression

- Wet season barriers  / “rain” barriers

###### Execution line .

- Owner / Road authority / Construction financing authority

- Owner's agent / Owner’s surrogate / Contracting agency

- Construction authority

- Force account

###### Resources .

- Tax / Taxation / Levy

- Fees / User fees

- Road tariff

###### Financing principles .

- Budget arbitration / Finance Act / Earmarking

- Road "commercialization"

###### Economic impact .

- Vehicle operating costs (VOC)

- Cost-effectiveness / Internal rate of return (IRR)

###### Responsibilities .

### Decentralization

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Road types . | 5.6 |

##### Paved (or sealed) roads (“asphalted” or “tarred”) - Unpaved (or unsealed) roads and tracks

##### Paved (or sealed) roads (“asphalted” or “tarred”):

- A road is "paved" if its surfacing comprises a highly cohesive, sealing upper course, which, among other functions, protects the complete pavement structure.

- Unpaved (or unsealed) roads and tracks:

- "Unpaved" roads (or tracks) are fragile, due to the absence of such an upper course.  Heavy traffic scatters the loose material which is not cohesive.  In-depth damage is also caused when the pavement structure is waterlogged.  Surface water can also very easily cause damage by erosion.

- You can improve at the construction the performance of the upper materials ( by careful selection, incorporation of cement, etc.).    

-  Continuation: Unpaved etc.

- Construction of a good unpaved road costs roughly 5 times less than for a paved road.  However, proper maintenance of a circulated unpaved road can be more costly, and even with efficient maintenance, its lifetime will be shorter.

- Everything depends on the traffic which the road has to carry, mainly on truck traffic.  If expected traffic levels are too low, paving would represent an enormous, unjustified investment (by comparison, the economic benefits derived from the road are much too limited).  However, paving is essential above a certain traffic level; but traffic expectations depend on the paving/unpaving option, due to driveability that differs.

- This means that a simple economic calculation is required in each case, in order to establish  which option is the most "cost-effective" for the community.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Operation types . | 5.6 |

- Routine maintenance - Periodic maintenance - Rehabilitation or reconstruction - Curative maintenance / preventive maintenance

- Routine maintenance:

- This corresponds to "light" maintenance work, executed throughout the year in order to extend the lifetime of a road as much as possible and at lowest cost.  Routine maintenance includes cleansing ditches, grass mowing, undergrowth clearing (all tasks eventually performed by road menders in a “cantonnage” process), and patching on paved roads, or frequent reshaping of unpaved roads.

- Periodic maintenance:

- "Semi-heavy" maintenance work, to be periodically performed in order to preserve the pavement structure by renewing its topping, also called resurfacing: adding a new wearing coat for paved roads (undertaken every 8 to 12 years, according to traffic)  or regravelling unpaved roads (undertaken every 4 to 6 years, according to traffic).

- Periodic maintenance rejuvenates the road topping, without renovating the pavement structure itself.

- 

- Rehabilitation or reconstruction:

- "Heavy work" designed to reconstitute the structure of a road in very degraded condition. Rehabilitation and reconstruction programs are not included under the heading of maintenance, as they are full re-investments required to re-establish an infrastructure which, in principle, has already been fully depreciated and comes to the end of its lifetime.

- Curative maintenance / preventive maintenance:

- Curative maintenance consists of repairing damage after it has occurred.  Preventive maintenance consists of taking action at the time when damage becomes probable but before it has actually occurred, in order to avoid or reduce such damage.

- Example: Removal of slop slide material is curative.  Planting the slope with vegetation to avoid erosion due to water circulation is preventive.

- An appropriate preventive maintenance policy can achieve savings and a better general level of service, against a policy limited to curative maintenance (even if executed rapidly and efficiently).

- Both routine maintenance and periodic maintenance include a majority of preventive tasks, plus a number of curative actions.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Works . | 5.6 |

- Road menders / Road mending / Roadmen work / - Lengthmen work / « Cantonnage » - Potholes - Patching - Reshaping / Regravelling

- Road menders / Road mending  /  Roadmen work  /  Lengthmen work / « Cantonnage »:

It refers to a way of organizing all the smaller tasks of routine maintenance (the tasks a single man, on foot, can perform with just basic manual tools ; these tasks involve grass mowing, undergrowth clearing, cleansing ditches, pipes and culverts, etc.). Each road mender (or roadman, or lengthman) is entrusted with a 2- or 3-km road section and take care of it all year long, under some supervision.

- Equally, « cantonnage » may either be performed by force account either be outcontracted.

- Potholes:

- Isolated destruction of the sealing of a paved road, generally induced in a worn surfacing by heavy traffic.                               

-   Continuation: Potholes

- If a pothole is not dealt with promptly, it spreads and deepens ; there is locally no longer sealing nor any protection for lower courses, which can now be seriously damaged by water infiltration or  direct axle impacts. A pothole, if not promptly repaired, worsens quickly ; it creates a spot of accelerated deterioration.

- Patching:

- This routine maintenance operation involves dealing with a pothole "in time", namely as soon as it appears, before any structural damage to the road can occur. Its French name (“point-à-temps”, which means exactly “patching-in-time”), is even more self-explanatory.

- You just cleanse,  fill in,  compact and  re-seal. Patching can be hand-crafted or performed more mechanically, but it is always rather technical and requires some experience.

- When the surfacing comes to a certain wear level, both the density and the outbreak frequency of potholes grow highly; at this step the patching method does not fit any longer (too slow, too expensive and technically sub-optimal). Then the time has come for resurfacing, which is periodic maintenance.

- 

- 

- Reshaping / Regravelling:

Maintenance operations concerning unpaved roads only. A distinction must be made between:

- Reshaping, with no systematic addition of external material, involving leveling and rectifying the pavement profile with a grader (or more rustic equipment).  This procedure eliminates rutting, mud-holes, etc.  This work constitutes  routine maintenance, required one or more times per year  according to traffic (up to 6 times per year in some cases).

- Regravelling, which is reshaping with systematic addition of external selected material to re-gravel the pavement, reconstituting satisfactory topping characteristics. The pavement profile is inevitably rectified too. This corresponds to periodic maintenance, required every 4 to 6 years according to traffic.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Traffic Impact . | 5.6 |

- Traffic / Heavy (or truck) traffic / Light traffic - Axleload / Aggressiveness / Aggression - Wet season barriers / “rain” barriers

- Traffic / Heavy (or truck) traffic / Light traffic:

Circulation of vehicles on roads.  A distinction is made between light traffic (cars, etc.), which cause little damage to the road, and  heavy  traffic (trucks, etc.) which cause considerable damage.

If we rate at 1 the damage caused by a car passing, the damage caused by a truck passing must be rated at 50,000 to  100,000 (in other words, a truck causes as much damage as 50,000 to  100,000 private cars).  The damage caused by a heavily overloaded truck must then be rated at 500,000 to 1,000,000.

- Axleload / Aggressiveness / Aggression:

The axleload determines both the pressure and the horizontal strains which the tire exerts on the pavement, and consequently the damage (the aggression) caused to the road surface by the vehicle.  For equal loads, the aggressiveness of 2 trucks against the road can differ considerably according to the number, type and arrangement of their axles (the global load being unequally shared between the axles).                                                                       

 Continuation: Axleload etc.

The pavement structure is initially designed for a specified lifetime (the design time), on the basis of assumptions concerning the future volume of heavy traffic, and according to the maximum statutory axleload limit.  If the actual heavy traffic is more intense than anticipated, or the axleload limit is not observed, the pavement is subjected to greater fatigue and ages prematurely.

The road could be irreparably damaged long before it has returned the whole economic benefit which was expected (although the community have to continue reimbursing the initial investment). Economically, it would be a fiasco.

- Wet season barriers (or “rain” barriers):

By definition, unpaved roads are not protected by a watertight surfacing.  Consequently heavy rain infiltrates deep into the material of the road structure, reducing its cohesion and therefore its bearing capacity (far from unknown, your foot sinks in a muddy ground, which has therefore a much lower bearing capacity than a perfectly dry beaten earth).  During several hours after heavy rains, the road is temporarily much more fragile than in dry weather.

If a heavy truck uses the road under these conditions, it causes major damage both to the topping and internal structure of the road.

This is why rain barriers are installed in certain cases, used to close the road to heavy traffic (or even to all traffic) following heavy rain, until the pavement dries out and returns to its normal bearing capacity.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Execution line . | 5.6 |

- Owner / Road authority / Construction financing authority - Owner’s agent / Owner’s surrogate / Contracting agency - Construction authority - Force account

- Owner / Road authority / Construction financing authority:

- This is the "owner" of the road, in other words the authority which is supposed to manage the network and schedule the roadworks, which finances the work (or raises the funds), and issues work orders.  The Owner is responsible to road users for network management quality.

- Owner's agent / Owner’s surrogate / Contracting agency:

- This is a specialist body, acting on behalf of the Owner, which in certain cases orders and supervises all work on the road network, if the Owner does not wish to assume these functions itself.           

-  Continuation: Owner’s agent etc.

- The Owner's agent is sometimes para-governmental, often non-profit, that acts as a contracting agency for design, work or control contracts. This proxy, to be entrusted on a multi-annual basis, is rarely subject to formal competition.

- Construction authority:

- This is a specialist entity responsible, under contract from the Owner (or the Owner's agent acting on behalf of the Owner), generally for both defining work projects and controlling the contractor charged with executing the work.

- A construction authority is needed in the majority of cases, to be in daily contact with the contractor, to issue detailed instructions, to check the reality, quantity and quality of the work executed and then to give the first clearance for payments.

- The construction authority is generally a private engineering firm, and is awarded a control/supervision contract specific to a given operation, generally as a result of a call for tenders.

- Force account:

- The administration is said to be working on a "force account" basis, where it undertakes execution of road work using its own staff and its own plant and equipment.                                           

-  Continuation: Force account

- In this case, the work is also naturally supervised on a force account basis, generally by the department responsible for actual execution of the work.  There is then no clear distinction between the site management function and the "control/supervision" function.  Indeed, where the administration is operating on a national road on a force account basis, it represents the Owner (the State) while acting both as construction authority and as contractor.

- The real cost price of force account work is uneasy to determine from public accounts (numerous items, such as the depreciation of equipment, salaries of administration officials, head-office expenses, etc. are hard to isolate and share out).  Frequently only marginal costs (for instance fuel costs) are taken into consideration, while forgetting all the non-marginal costs; hence the illusion that force account is highly competitive!

- Nor has the force account system any strong economic logic providing a natural incentive for improving performance (productivity gains and cost control).

- The traditional weakness of the force account system lies in the area of non-financed operations to which both machines and manpower are often extensively rerouted (but they are only apparently "free").

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Resources . | 5.6 |

- Tax / Taxation / Levy - Fees / User fees - Road tariff

- Tax / Taxation / Levy:

- This concerns additional charges paid by the tax-payer in connection with particular operations (purchase of fuel, registration of a vehicle, etc.), which represent the tax-payer's contribution to the community needs.  These levies or indirect taxes form part of the general taxation system.  They are paid into the general State budget, even though some are earmarked, namely destined for a particular type of expenditure.

- Fees / User fees:

- These are additional charges paid by the user in connection with certain operations (purchase of fuel, registration of a vehicle, etc.), which represent the user’s return for using a public utility, namely a payment for service (examples: airport fees, water and electricity bills, etc. or road maintenance fees in some cases).

- 

-  Continuation: Fees / User fees

- These fees are not to be paid into the general State budget, but are paid directly to the entity responsible for managing this specific resource and  financing the operation of the service. These fees are not taxes nor levies.  They are sometimes referred to as "parafiscal" charges, to indicate that they are in addition to taxation.

- Road tariff:

Where the road user is subject to a set of user fees, assigned to finance road maintenance, these fees as a whole are referred to as road tariff.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Financing principles . | 5.6 |

- Budget arbitration / Finance Act / Earmarking - Road “commercialization”

- Budget arbitration / Finance Act / Earmarking:

The general State budget is a forecast document, indicating both forecast revenue (including tax revenue in particular), and forecast expenditure for the coming year.

Each year, the budget prepared by the Ministry of Finance is approved by the Government following arbitration between the requirements expressed by the various spender ministries.

The budget is then submitted to the Parliament, which debates, amends, corrects and finally approves the budget by vote, under the form of a special law, referred to as the "Finance Act".  This law authorizes the Government to implement the yearly budget, namely to collect revenue and commit expenses.  Until the Finance Act has been promulgated, no expenditure can be committed by the administration for the new fiscal year (apart from exceptional cases).                                                                                           

 Continuation: Budget arbitration etc.

This means that even "earmarked" tax revenue, namely revenue theoretically intended for total allocation to a clearly defined application, is subject to the budget arbitration and Finance Act process.  So there is no total guarantee that the budgets allocated will ultimately correspond to the full amount of such revenues.

[ It is indeed a traditional principle of public management that the State should retain ample budget management margins by prohibiting "earmarked" budget revenue.  This is referred as the "single treasury" principle.]

Earmarking fiscal revenues for financing public utilities operation is a poor and hazardous method. Commercialization is the clear and proper solution.

- Road "commercialization":

Such an institutional reform process consists of transition from a previous traditional situation where road maintenance (as a State responsibility) was financed from the general State budget, and managed exclusively by the administration, to a new situation where road maintenance (as a special market service) will be financed by user fees constituting the road tariff, managed jointly by the administration, road authorities and road users.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Economic impact . | 5.6 |

- Vehicle operating costs (VOC) - Cost-effectiveness / Internal rate of return (IRR)

- Vehicle Operating Costs (VOC)

#### Where a transport company provides a commercial service (passenger or good transport), a considerable number of items are included for calculation of the cost price of this service.

#### These include notably:

#### items which constitute the real technical vehicle operating cost (“VOC”), including fuel and lubricants, consumables and wear parts (tires, V-belts, filters, brake discs and shoes, shock absorbers, clutch disks, etc.),

#### and also  some other direct charges, fixed or variable, as:

#### the components of the road tariff (not to count twice the fuel fee, as this is already counted in the price of the fuel; including annual road fee and/or registration fee if any, official tolls, etc.),

#### as well as

#### informal tolls, etc.,

#### vehicle insurance,  etc.

#### …not to mention items that are usual for any industry (as wages, commercial costs, head-office costs, asset depreciation, etc.).     

####  Continuation: VOCs

#### The VOCs, calculated per tons x kilometers (TxKm) for goods, or kilometers (km) for passengers, naturally increase if the road tariff increases, but drop (much more quickly) when the state of the road is improved.

In actual fact, every dollar properly spent in road maintenance is considered to save all the road users around three dollars on vehicle operating costs (VOCs).

- The primary objective of a good road maintenance policy is to reduce VOCs, therefore to reduce the cost price to the transport company and so to reduce the transport price billed by the transport company to its customers…and then to reduce the transport part within the cost of any product.  This is indeed the way the transport industry must support the general economy.

- Cost-effectiveness of public expenditures / Internal rate of return (IRR)

The choices to be made when setting up a road maintenance program are not always particularly clear-cut.

For example: For a given road, should periodic maintenance be initiated straight away, or is it better to wait a year, commencing with simple routine maintenance which costs ten times less?

Or: Of two periodic maintenance operations on different roads but which cost the same amount, which should be given priority this year?                                                                                    

 Continuation #1: IRR

For such public expenditure, since the concept of profitability is not relevant, the concept of cost-effectiveness is being used.

Generally the relative interest of each solution (or choice) is estimated by comparison with a reference solution, conventionally called the status quo.  The interest of the solution examined (its interest for the community) is frequently assessed according to its internal rate of return.

The procedure is as follows:

- For each year of the lifetime of the operation, the benefits for the community on the one hand, and the cost for the community on the other, are estimated in terms of monetary value.  We therefore have annual balances (benefits versus costs), which can be positive or negative according to the year.

In fact it is not absolute costs or absolute benefits which are taken into account, but only the excess costs and additional benefits by comparison with the reference solution (status quo).

- It is not sufficient merely to add these balances.  The time scale element must be taken into account: the further an expected benefit, the lesser it appears interesting…and vice versa for costs: the further a scheduled expenditure, the lesser it appears penalizing.

All the annual benefit-cost balances are therefore added, each being depreciated according to its distance in time.  A discount rate is used for this purpose.                                                      

 Continuation #2: IRR

BalanceGlobal =   Balanceyear1 / (1+d)

+   Balanceyear2 / (1+d)2

+   Balanceyear3 / (1+d)3    +    etc.

The discount rate ("d") is a percentage expressing the degree of  "preference for the present" of the community concerned.  This is determined according to its budgetary leeway (cash position and capacity for raising external funding).  Thus $ 100 of cost or benefit this year is regarded as the equivalent of $ [100/(1+d) ] of cost or benefit in one years' time.

Yearly balances can vary considerably from year to year. In a typical case, for a simple road maintenance operation, the main costs are concentrated in the first year, while the benefits are spread over the following years.                                                   

 Continuation #3: IRR

Thus, in a case of this type, the greater the value of d, the greater the extent to which the benefits are depreciated, and vice versa.  If d is very high, the global balance will inevitably be negative (for in this case immediate costs are greater than highly discounted subsequent benefits).  If d is very low, the global balance is generally positive (for the substantially enhanced future benefits are greater in this case).  Somewhere between the two extremes therefore, there is a value of d for which the global balance would be zero.  This special value of d is referred to as the internal rate of return (IRR) for the operation.

If the IRR for an operation is 17%, what does this signify? - If the discount rate for this community concerned is exactly 17%, this operation would be regarded as neutral, with costs and benefits equivalent, corresponding to an economic balance of zero. - If the discount rate is greater than 17% (money is harder to find for the community), the balance for this operation is negative, and consequently not profitable for this community. - If the discount rate is less than 17% (money is easier to find), the balance is positive, and this operation can be regarded as profitable.



 Continuation #4: IRR

The economic substance behind this calculation is very simple: a country which can raise funds easily can engage in investments for which the return does not occur for a long time.  In a country where money is short, investments for which the return is rapid must be privileged.

For developing countries with low budgetary leeway and limited borrowing power, the discount rate to be taken into account is between 12 and 14%.

This means that only operations where the IRR exceeds these values are of interest.

In the present context in Africa, where road traffic levels are relatively low, effectively funded new road investment operations rarely generate IRR values exceeding 15%.  In some cases they are on the break-even borderline.

In contrast, rehabilitation operations frequently show IRR values of 20%.  Above all, maintenance operations (and periodic maintenance in particular) systematically generate "exceptional" rates of return exceeding 40%, in cases going as high as 100% (as the status quo, in other words non-maintenance, causes very substantial loss of benefits as well as substantial subsequent excess costs).                                         

 Continuation #5: IRR

This is a general rule, extending beyond the road domain, and in no way restricted to developing countries: the rate of return from maintenance is substantially greater than that from investment.

The economic lesson is very simple: any dollar spent in a maintenance operation would roughly be twice more profitable as any dollar spent in an investment operation. So if you really only have one dollar while needing 10 dollars for maintenance and 1 dollar for investment, then maintenance is a mandatory choice.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| Basics (25-word list) Responsibilities . | 5.6 |

### Decentralization

### Decentralization

This corresponds to a general reform program for the institutional organization at the country level,  whereby the central government (the State) transfers certain of its responsibilities to new local authorities (at various levels: province, region, district, county, municipality, or whatever their names in a particular country), provided they are and governed by locally elected representatives and fully empowered (not under central supervision).

Such local communities are no longer merely administrative divisions under State-appointed officials.

#### The basic idea of decentralization is that some responsibilities, depending the matter, could be better managed from the local level than under an overcentralized scheme.

#### 

####  Continuation: Decentralization

But for roads as well as other matters, decentralization will not remedy an extensive lack of resources.

#### In principle, if the State transfers certain of its responsibilities (namely charges to be assumed, such as for example, the maintenance of a local road network), it must also transfer the resources necessary for exerting these responsibilities. This is achieved either by budget allowances to be renewed annually, or by setting up a local taxation system which will provide the local authorities with their own, adequate direct resources, and, in addition in the case of road maintenance, by allowing them equitable access to the resources managed by the Road Maintenance Fund.

Usually bituminous.  Asphalt or tar are in fact specific materials.  The investment cost for concrete paving is very high, and is consequently only justified for very high traffic levels or particular operation constraints.

The discount rate has nothing to do with the inflation rate (which describes monetary erosion).

In this case we calculate in terms of constant monetary unit value, referenced to year zero.  Even in a country with a non-inflationary economy, commitment of an expense this year is more restrictive, in economic terms, than commitment of the same expense next year. The discount rate expresses this phenomenon and nothing else.

If calculations are not made in constant monetary unit value, the annual balances must be corrected by applying both the expected inflation rate and the discount rate.