| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| ###### Road Life Cycle : Deterioration curve ###### for a paved road, 4 condition levels 1/9 7/9 | 5.11 |

## Regular ## Bad Very bad ## Good  Road Condition 0 5 10 15 20 25 #### Years since construction  Remark  This curve corresponds to a paved road. For another road type, the deterioration curve would be different, but the principle would remain the same.

Courtesy of UN-ECLAC. See Table of Contents for reference.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| ###### Road Life Cycle : Deterioration curve ###### for a paved road, 4 phases 2/9 | 5.11 |

## Regular ## Bad ## Very bad ## Good A After construction B Slow deterioration C Critical phase D Destruction  Road Condition #### Years since construction 

Courtesy of UN-ECLAC. See Table of Contents for reference.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| ###### Road Life Cycle : ###### the 4 phases (paved road) 3/9 1/9 | 5.11 |

|  | Courtesy of UN-ECLAC. See Table of Contents for reference. Phase A. Just after construction  light routine maintenance Phase B. Slow and mostly unperceived deterioration  growing routine maintenance Phase C. Critical phase : damage to the state of the pavement  periodic maintenance Phase D. Destruction : disappearance of the pavement and damage to the road structure if it comes to phase D, it is too late for periodic maintenance. It requires now :  rehabilitation / reconstruction of the road |  |  |
| --- | --- | --- | --- |
| PEDAGOGICAL BASICS |  | T&T |  |
| ###### Road Life Cycle : Phase A ###### (Just after construction) 4/9 |  | 5.11 |  |

|  | Courtesy of UN-ECLAC. See Table of Contents for reference. - Road condition: very good -  the road is put into service as soon as completed and fully satisfies user expectations. - Work involved: light routine maintenance -  conservation: maintenance of drainage (clearing of ditches and vegetation control along edges) - Remarks -  The needs of routine maintenance depend on the quality of the initial construction -  Routine maintenance is not costly and is easy to carry out. |  |  |
| --- | --- | --- | --- |
| - PEDAGOGICAL BASICS |  | - T&T |  |
| ###### Road Life Cycle : Phase B ###### (Slow deterioration) 5/9 |  | - 5.11 |  |

|  | - Courtesy of UN-ECLAC. See Table of Contents for reference. - Road condition: regular -  minor damage to the pavement and weakening of the basic structure - This damage is caused by normal road use by vehicles and by climatic factors. - Work involved: growing routine maintenance -  conservation: filling in potholes (patching) and mainte-nance of ancillaries (clearing of ditches and vegetation control along edges) - Remarks -  The needs of routine maintenance depend on the quality of the initial construction, but also on current truck traffic. -  Routine maintenance is not costly and is easy to carry out (but patching requires both equipment and experience). |  |  |
| --- | --- | --- | --- |
| - PEDAGOGICAL BASICS |  | - T&T |  |
| ###### Road Life Cycle : Phase C ###### (Beginning of the critical phase) 6/9 |  | - 5.11 |  |

- Road condition: bad -  becomes "tired" on the surface and in its basic structure; loses its ability to withstand traffic - Work involved: surface strengthening (as periodic maintenance to be launched as soon as possible) -  Adding a layer of asphalt concrete several centimeters thick. This maintenance must be carried out periodically, every 8-10 years depending on the traffic. - Remarks -  Surface treatment costs approximately 15% of the original construction cost of the road. -  It involves stopping the rapid deterioration process and fully preserving the structure so that it is possible to use the road for many more years to come.



- Courtesy of UN-ECLAC. See Table of Contents for reference.

| - PEDAGOGICAL BASICS | - T&T |
| --- | --- |
| ###### Road Life Cycle : Phase C ###### (End of the critical phase) 7/9 | - 5.11 |

- 

|  | - Courtesy of UN-ECLAC. See Table of Contents for reference. - Road condition: bad - Very obvious damage to the pavement. - Damage to the basic road structure - Work involved: repairing (i.e.rehabilitation) -  Repositioning the road base. Strengthening the pavement (replacing full sections of road in some cases, repaving the entire road) - Remarks -  The longer this work is delayed, the more severe and costly will be the rehabilitation work |  |  |
| --- | --- | --- | --- |
| - PEDAGOGICAL BASICS |  | - T&T |  |
| ###### Road Life Cycle : Phase D ###### (Destruction) 8/9 |  | - 5.11 |  |

- Road condition : very bad -  Disappearance of the pavement. The initially-paved road ends up being a gravel road and, in the long run, a dirt track. - Work involved: Reconstruction -  Reconstruction of the road - Remarks - Only trucks and four-wheel-drive vehicles can use the road, but at a very reduced average speed. Vehicle maintenance costs rise sharply. - The cost of completely reconstructing the road can be as high as 50% to 80% of the cost of initial building of the road, depending on many factors.

Courtesy of UN-ECLAC. See Table of Contents for reference.

| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| ###### Road Life Cycle : ###### Unpaved road vs Paved road 9/9 | 5.11 |

 ## Regular ## Bad ## Very bad ## Good Paved Unpaved Courtesy of UN-ECLAC. See Table of Contents for reference. Road Condition #### Years since construction  Remarks  A non-paved road has typically this type of behavior. By lack of protection, deep degradation starts very quickly, without grace period. The advantage is that a non-paved road costs 3 to 5 times less than a paved road, but it is much more fragile.  Note that here, the traffic level for the paved road above could be 10 to 50 times more than for the unpaved one. You do not use both the same way.