| GAME AIDS | T&T |
| --- | --- |
| ###### Setting the road Tariff: ###### Breakdown and scales (10 pages) ###### 1/4 | 2.15 |

Equitable and acceptable allocation of costs to the user:

Basic arguments

A.

Determination of a cost allocation structure

For each $100 required by the Road Fund, Who will pay and how?

A1.

Distinction between fixed and variable costs

These $100 are used to cover network maintenance needs (routine and periodic maintenance), and operating costs in certain cases.  These costs can be divided into two categories, fixed costs and variable costs:

- Fixed costs correspond to work which is necessary whatever the traffic level (and in the extreme case, even if there is no traffic).

Example: Cleaning of ditches.  Rainwater run-off carries material which blocks ditches, ducts, etc. This happens even if no traffic is using the road.  No particular type of vehicle is "guilty", but the cleaning of ditches is essential for network maintenance.  This is consequently a fixed cost.

- Variable costs correspond to work linked to the volume of traffic, or to certain categories of traffic (in the extreme case, variable costs would be superfluous if there were no traffic).

Example: The appearance of potholes is directly linked to the intensity of truck traffic, which wears and breaks up the road surfacing.  If there were no heavy traffic, no potholes would appear.  Each additional truck aggravates this problem.  The cost of patching (pothole repair), consequently depends on traffic volume and type.  This is consequently a  variable cost.

All the costs to be covered (routine and periodic maintenance, and operation where appropriate) can be classified as fixed or variable costs.

Coming back to our $100, we could have $40 of fixed costs and $60 of variable costs, for example.

A2.

Allocation of variable costs by vehicle category

The aim is to make each user category pay for the road damage which it causes.

To achieve this, we apply an aggressivity factor for each type of vehicle, corresponding to the degree of gravity of the damage which it causes.  We know that a truck is the equivalent of 50,000 private cars in terms of the damage which it causes to the pavement structure.  The difference is considerably less for other types of damage.  Analysis of the cost categories involved makes it possible to determine a reasonable allocation scale, for example with factors ranging from 1 for a private car up to 15 for an articulated vehicle (1 to 15, and not 1 to 50,000!).

If we combine this scale with traffic volumes, we can allocate variable costs to the different types of user equitably.  This requires good statistics.

For example: Articulated vehicles represent 10% of annual traffic (expressed in veh./km), but  55% of total aggression to which the roads are subjected.  This means that all articulated vehicles must pay 55% of our $60, namely  $33.

Private cars comprise 50% of annual traffic (expressed in veh./km), but account for only 8% of the global aggression to which the roads are subjected.  This means that all private cars must pay  8 % of our $60, namely $4.80.

The remaining $22.20 are then shared between the other types of vehicle (taxis, buses, etc.) in the same way.

A3.

Allocation of fixed costs by vehicle category

In this case, no-one is "guilty", but all users must contribute to the essential effort required.  Fixed costs allocated to users can be regarded as a sort of "entrance fee” to the road network, and not payment for road damage caused.

Fixed costs are generally shared between the different types of user more evenly.

For example: Fixed costs are allocated in proportion to traffic volumes.  In this case, if private cars represent 50% of annual traffic volume (expressed in veh./km), all private cars must pay 50% of the $40, namely $20.

The remaining $20 are then shared between the other types of vehicle (taxis, cars, small vans, ordinary trucks, articulated vehicles, etc.) in the same way.

The fairness of cost allocation can be improved further by taking account of the load carried by each type of vehicle, reflecting the economic benefit which each type of user obtains per vehicle/km.

A4.

Establishing the cost allocation balance sheet for each user category

We now know, in principle, how much of the initial $100 should be contributed by each vehicle category.

Example: For each $100, private cars should pay $4.80 for variable costs and $20 for fixed costs, or a total of $24.80.

B.

Determination of the standard year budget level to be achieved

Calculations can be based on:

- maintenance requirements, according to RMF rules:

- eligible networks (caution: eventual rehabilitation programs in the coming years will increase the size of the networks eligible for maintenance, etc.)

- types of work financeable for each network

- methods of financing adopted for each network (integral financing, aid funds or subsidies, etc.)

- and indeed any contributions already programmed by the donors

- the global financial pressure consequently borne by the user, and the acceptability of this pressure, and

- the capacity for processing and implementing the work programs (by owners and contractors).  This is frequently the maximum extent of ambitions in this context.

In many cases a number of scenarios are prepared.  Each is examined in full, and the following phases provide the opportunity to make a more fully considered choice.

We now know the standard year budget required for a given scenario. This is not the $100 which we have used so far for reference purposes, but a figure of the order of $20 million for example.

The fixed and variable parts to be allocated to each user category are then calculated directly, applying the calculation procedure defined in step A4.

Example: For a budget of $20 million ( 200,000 x $100), all private cars must pay 200,000 x 4.8 = $960,000 for variable costs, and 200,000 x 20 = $4,000,000 for fixed costs, namely a total of $4.96 million.

C.

Road tariff construction

C1.

Selection of types of fee

We must now select the types of fee to be applied to collect road user charges.  Should we select one type only, if so which, or several types, and again if so which?

Each type of fee presents both advantages and disadvantages:

- The majority of possible fees (road fee disc, registration fee, tolls, etc.) can be modulated by category.  This means we can “stick” to our category-related allocation plan.

This makes it possible to create a complete scale directly, which applies differently to each vehicle category (according to vehicle "silhouette", cubic capacity, power, load, etc.).

- However, road fee disc and vehicle registration fee apply indiscriminately to all users in the same category, irrespective of the distance traveled by each user.

Example: We only introduce an annual road fee disc.  If there are 1,000 private cars on the road, each car user will pay 1/1000 part of $24.80 per year, to recover the $24.80 due for all private cars.  The same price will then be paid for a car doing 1,000 km in the year, as for another doing 40,000 km in the year.

- In contrast, a fuel fee is, by nature, "proportional" to the service provided for each user.

Example: But if a fuel fee is chosen, a part of the $24.80 is paid for each private car, naturally according to its annual fuel consumption.  For two private cars with the same engine rating (which have consequently about the same mean consumption, expressed in liters/100 km) one doing 1,000 km per year and the other 40,000, the cost for the second is 40 times more than for the first.

- Certain types of fee (tolls in particular) are expensive to collect.  These collection costs must also be passed on.

Example: Generalized road toll systems, with toll collection sub-contracted to a private operator.  The operator must depreciate its equipment, pay its staff and make a profit, which means collection costs of the order of 20% of gross revenue.  To pay $24.80 towards road maintenance, $31 must be collected from private cars, of which $6.20 (20%) will be used to remunerate the toll operator.  This means that for every $1 paid by the user, only $0.80 goes to the Road Fund.

- Only collection of a fuel fee is "free"

For a fuel fee, it is frequently possible to set up a simple collection system, the cost of which is nothing or practically nothing.  The fee is pre-paid by the fuel importers on withdrawal from customs stocks, and this cost is passed on to the user dollar for dollar, when the fuel is sold at the pump.  $24.80 is collected from the importer-distributor, who recovers the same amount by selling fuel to the user.  For $1 paid by the user, $1 goes to the Road Fund.

- However, a fuel fee does not allow for finally adjusted allocation.  In fact, it introduces distortion for private cars running on diesel.

A fuel fee only provides for a rough scale, with one rate for gas and another for diesel.  Unfortunately, this gas/diesel distinction does not correspond exactly to the basic distinction between private cars and trucks (in certain countries, up to 40% of diesel can be consumed by private cars).  While the aim is to penalize trucks, it is inevitable that private cars running on diesel are also, and unfairly, penalized.

- Registrations fees are accepted more easily than an annual road fee disc by the average user, but can have a braking effect on modernization of the vehicle fleet.

With registration fees paid on purchase of the vehicle (whether new or second-hand) in the first years after setting up, many users do not happen to pay such a fee.

- Any fee induces a certain degree of fraud (with consequent loss of revenue for the Road Maintenance Fund), and some are more prone to this problem than others.  The predictable fraud rate makes it necessary to increase the scale for those who do pay.

Example: Road toll fraud.  When the toll stations cannot be bypassed physically, it is found cheaper sometimes to bribe toll station personnel than to pay the official toll.

- However, the most serious risk of fraud concerns fuel fees.  In some cases, fuel smuggling creates a loss of theoretical revenue exceeding 40%.  This risk depends partly on the relative cost of fuel in terms of pump prices.  This risk is minimum in some countries, but very high in others.

- Fuel fee fraud takes two traditional forms: either quotas of tax-and-fee-free fuels intended for non-road purposes are diverted, or fuel is smuggled in from neighboring countries where pump prices are lower.

- The fuel fee "competes" with conventional tax revenue (fuel taxes, destined for the general budget), and this can create a substantial constraint.

General fuel taxes represent a convenient way of boosting the State budget.  By introducing a fee which is also incorporated in the pump price, the margin for maneuver in the tax domain is reduced (as the end price at the pump is an extremely sensitive factor in social terms, and the price differential with neighboring countries must be watched closely).

- And so on.

The distortions particular to each type of fee generally make it necessary to combine a number of different types, the individual shortcomings of which offset each other, in order to stay in line with the allocation matrix while limiting excess costs.

The application of more than two or three types of fee at a time must be avoided, otherwise the user will be completely lost.

C2.

Construction of the general road tariff

A typical combination is as follows:

1. Road fee disc (or possibly registration fee), making it broadly possible to allocate a specific share of variable costs to each vehicle category, on the basis of an aggressivity scale.

To go back to our first example, this road fee disc must generate  60% of revenue, or $12 million, representing the total amount of variable costs.  In line with the allocation system indicated above, private cars must provide $0.96 million.  If there are 100,000 private cars on the road, each will then pay an annual road fee disc of about $10 (or in the case of a registration fee, if 15,000 cars are registered per year, the registration fee will be $64).

Articulated vehicles should cover  55% of variable costs, or $6.6 million.  If there are 4,000 vehicles in circulation, the annual road fee disc will cost $1650 (or in the case of a registration fee, if 800 articulated vehicles are registered per year, the registration fee should earn $8,250).

2. AND a fuel fee (gas/diesel) providing the remainder of the revenue, corresponding broadly to recovery of fixed costs roughly in proportion to the service provided for each user.

Staying with the same example, this fee must produce 40% of revenue, namely $8 million, representing the amount of fixed costs.  Private cars must produce $4 million.  If there are 100,000 private cars on the road, each doing an average of 20,000 km per year, and with a mean fuel consumption of 8 liters/100 km, annual fuel consumption is then 16 x 107 liters (ignoring private cars running on diesel).  The gas fuel fee which must then be charged is 2.5 cents per liter.

Once the road tariff has been determined, based on appropriate statistics (annual registration rate per category, annual fuel consumption figures, etc.) it is then possible to calculate the scales required for the best possible, equitable allocation of charges by category (see step A4).

C3.

Special case of foreign trucks

In some countries, foreign trucks can account for a large part or the quasi-totality of truck traffic.

This is the case in a number of small land-locked countries, or small countries providing the route to the coast for their large land-locked neighbors.

In the above road tariff example, foreign trucks:

- Pay neither road fee disc nor registration fee, as they pay these charges in their country of origin.

- If the transit route is short, and if the price of diesel is cheaper across the border, they naturally come in with full tanks.  Consequently they pay very little or no fuel fee.

- They nevertheless benefit from the existence of the road network, and cause as much damage as national trucks.  It would therefore be fairer for them to contribute to the cost of maintaining the network.

In certain cases an international transit fee is charged to foreign trucks, to offset the loss of earnings due to the fact that they avoid payment of all or part of the general road tariff.

CAUTION: This can present a delicate problem if, conversely, a material number of national trucks transit via the country in question, thus involving problems of reciprocity.

C4.

Determination of credible and justifiable assumptions concerning the conversion of existing "allocated" taxes

This usually concerns special existing fuel taxes, and more generally, all earlier taxes or fees "allocated", in principle, to road maintenance (and which were frequently no longer applied, at least not in full, for this purpose).

- By debudgetizing road maintenance (by creating an autonomous Road Maintenance Fund), the State is relieved of expenses which constituted a budget charge, fully quantifiable in terms of both theoretical needs and  actual charges to the most recent budgets.  It is not to be imagined that the State would not also transfer a block of existing resources, as to fail to do so would be to cheat the user.

- It is therefore reasonable that all or part of specific taxes previously "allocated" in principle, should be converted into the form of a user fee (of a non-fiscal nature), in favor of the new autonomous Road Maintenance Fund.  Technically, this corresponds to cancellation of all or part of the tax, which is immediately replaced by an equivalent fee.

- Caution: We are concerned here with general taxation of a transaction (such as the purchase of fuel). The tax could well have been expressed in % of the price ex tax, and its amount was supposed to vary with the ex tax price (in the case of fuel, according to the import price for hydrocarbon products).  As a user fee is required to cover known maintenance costs, it is expressed as a fixed amount (so many cents/liter), and not as a percentage.

- This makes it possible to found the new Fund on a correct initial basis, with an assured minimum revenue structure involving constant pressure for the user, thus representing an initial break-even operation.

- break-even for the State, relieved both of expenses and resources

- break-even for the RMF, which takes over the expenses but also receives transferred resources, and

- break-even for the user, who pays the same amount in fees to the RMF as previously paid in taxes to the State).

- Obviously there is no chance of this simple transfer being sufficient to set up a correct road maintenance budget.  It is no more than an initial basis which will necessarily be increased by a few extra centimes, to achieve the necessary road tariff scales as set in step C2.

- The additional pressure on the user only amounts to these few additional centimes, as the "transfer" part does not count.

Example: An initial special petroleum product tax of 5% of the pump price for diesel, or 12 centimes per liter, was, in principle, allocated to road maintenance.  However, in reality, only the equivalent of 
8 centimes was paid out for road maintenance, in the form of a budget appropriation, over the last few years.

Following discussions between the RMF and the Ministry of Finance, with guidelines set by an order from the Prime Minister, the special tax was reduced to 2% (dropping from 12 to 4.8 centimes per liter), and a diesel road fee of 14 centimes per liter was created (7.2 centimes transferred – break-even operation – plus an additional 6.8 centimes).  The pump price for diesel has only risen by  
6.8 centimes, although the RMF received 14 centimes.

Without this transfer, the price of diesel would have had to be in creased by 14 centimes instead of 6.8 (and the State would have unjustifiably abandoned 8 centimes of expenditure which it had previously borne, without the transfer of any revenue).

C5.

Assessment of the level of global resultant pressure on the user, in terms of economic and social acceptability

- As fuel prices represent a sensitive economic index, this is frequently the only factor to which any attention is paid.  CAUTION: to assess the level of global pressure on the user (reflected in an immediate apparent excess charge on the T/km or passenger/km rate), all components of the road tariff (road fee disc, registration fees, toll, fuel price increase, etc.) must be integrated.

- In reality, this excess charge affecting road transport costs is only a short-term phenomenon.

- Efficient road maintenance financed in this way will be progressive reflected in a genuine reduction in vehicle operating costs (in global terms, we reckon that for every $1 devoted to road network maintenance, operating costs for the complete vehicle fleet are reduced by $2 to 3).

Efficient road maintenance is entirely in the economic interest of the road user.

However, the economic return in terms of operating costs is not immediate, although the increase in the price of diesel (for example) commences straight away.

$

Excess charges induced by the road tariff

## Years

### Vehicle operating cost savings

- We therefore need to examine the immediate impact of setting up a road tariff very closely, in the light of the various budget scenarios (established for the current year) (see step B above).

- This will probably lead to phasing the increase in road fees, until the target budget level is achieved in the current year.

Caution: we must maintain a realistic view:

any delay in build-up of the road tariff induces a corresponding delay in build-up of the road maintenance budget, and a consequent delay in achieving economic return.deldeldd

$

Excess charges induced by the road tariff

Years

#### Vehicle operating cost savings

D.

Definition of a multi-annual road tariff policy on these bases

CAUTION: Even if the partners agree on setting up consistent road maintenance budgets rapidly, the most severe constraint in many cases is:

- the capacity for processing and executing work programs (on the part of the owners and contractors).  This is frequently the maximum extent of ambitions in this domain.

There is no magic path from a series of irregular and very low budgets to much more consistent budgets, with no major problems of implementation.  A progressive approach must be accepted, and specific efforts must be devoted, over a period of time, to improving owner performance, control and execution.

Execution and control of the work:

Periodic maintenance of paved roads may not induce any major problems of capacity, insofar as international or regional contractors and engineering companies take an interest in the project.

On the other hand, routine maintenance in general (with particular reference to patching, this being a highly technical "small" trade) and periodic maintenance of unpaved roads (largely similar to earthworks) may involve major problems of capacity with regard to small local contractors..

The typical working process described above, which should make it possible to define a road tariff policy which is equitable, acceptable… …and reasonable generally involves recourse to a specialist consultant for: collating the necessary statistics, defining relevant scenarios, finalizing different charge scale options, analyzing their probable impact, participating in discussions between the parties involved, and formalizing the final choice. The expertise provided by this consultant enables the various parties involved in management of the RMF to take decisions on a fully informed basis, …without the need for each party to become a specialist in taxation, road technology, or the management of a road transport company.